3GPP TSG-RAN WG3 Meeting #127	             R3-250686

Greece, GR, 17th – 21th Feb 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Inventory Procedure and Signaling

Document for:	Discussion

1	Introduction

In RAN#106 meeting, a WID on Solutions for Ambient IoT (Internet of Things) in NR has been approved. During the WI phase, only Topology 1 will be standardized and the following RAN3-led objectives have been identified within the general scope described in the WID:

- RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will provide our preliminary views on signaling and procedure between A-IoT RAN and A-IoT CN to support inventory operation.

2	Discussion

2.1	Inventory Procedures

In TR 38.769, companies completed the following message flow and procedural text for A-IoT inventory operation in Topology 1.

Figure 1 Message flow for A-IoT Inventory in Topology 1 1a. The A-IoT CN sends an Inventory request message to the A-IoT RAN node, taking into account, among others, the A-IoT transaction scope. 1b. The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources. 2. The A-IoT RAN node sends an Inventory response message to the A-IoT CN. NOTE 1: In step 2, the A-IoT RAN node may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s). 3. The A-IoT RAN node performs the inventory procedure towards the A-IoT device(s) over the A-IoT radio interface. 4a/4b. After receiving the inventory result from the A-IoT device(s), the A-IoT RAN node may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result. NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

As the outcome of study item, we can take this message flow as baseline for A-IoT Inventory case. During the inventory operation, A-IoT CN should initiate a class-1 Inventory Request Procedure to the A-IoT RAN node, including INVENTORY REQUEST message and INVENTORY RESPONSE message. The purpose of Inventory Request procedure is to request A-IoT RAN node to perform inventory operation towards A-IoT devices.

Proposal 1: It is proposed to introduce a class-1 A-IoT Inventory procedure between A-IoT CN and A-IoT RAN node, including INVENTORY REQUEST message and INVENTORY RESPONSE message.

In addition, if the A-IoT RAN node is not able to perform inventory operation towards A-IoT device(s), the A-IoT RAN node can reply with the INVENTORY FAILURE message to reject the inventory request from A-IoT CN.

Proposal 2: If the A-IoT RAN node is not able to perform inventory operation towards A-IoT device(s), the A-IoT RAN node can reply with the INVENTORY FAILURE message to reject the inventory request from A-IoT CN.

After receiving the inventory result from the A-IoT device(s), the A-IoT RAN node may initiate a class-2 Inventory Report procedure by sending Inventory Report message to the A-IoT CN including multiple A-IoT device IDs, including INVENTORY REPORT message. The purpose of Inventory Report procedure is to provide the device ID to the A-IoT CN.

Proposal 3: It is proposed to introduce a class-2 Inventory Report procedure to provide the device ID from A-IoT RAN node to the A-IoT CN, including INVENTORY REPORT message.

2.2	Information exchanged between A-IoT CN and A-IoT RAN for Inventory

In TR 38.769, we studied the fundamental information should be exchanged between A-IoT CN and A-IoT RAN for inventory operation.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) NOTE 1: The definition of this identification is out of RAN3 scope. For topology 1, A-IoT RAN needs to store the A-IoT Device Identification received from the inventory request. NOTE 2: Whether to interpret and process AIoT Device Identification received from the inventory request needs further discussion. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report. NOTE 3: For topology 1, whether device ID received from the device is sent transparently to the A-IoT CN by the A-IoT RAN depends on RAN2 and SA2. NOTE 4: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2.

From the TR, it is found that the inventory request message may include the A-IoT Device Identification information, which is used to A-IoT Devices compare the A-IoT Device Identification information with their own A-IoT Device Identifier (part of or full A-IoT Device Identifier) to determine whether respond to the paging message. In additional, the inventory category indicator should be included in inventory request message which is used to indicate the inventory operation is for single device, a group of devices, or all devices. Obviously, this information should be visible to the A-IoT RAN node, which may help A-IoT RAN node allocate radio resources and select RA type properly. Assumed that AIOTF wants to find single device, the A-IoT RAN node can allocate the dedicated resource for the device to perform CFRA. Regarding single device is to be found, the inventory request message only includes this specific A-IoT device identifier. Assumed that AIOTF wants to find more than one device, the A-IoT RAN node should perform CBRA to all devices. When a group of devices is to be found, the A-IoT inventory request message may include a group ID or all A-IoT device identifiers within the group. If the A-IoT Device Identification information includes all A-IoT device identifiers within the group, the “start point” and “end point” or the length of each A-IoT device identifier should also be provided by A-IoT CN. Then the A-IoT RAN node can identify each device ID within the group and perform A-IoT paging subsequently. When all devices are to be found, the inventory request message may include no A-IoT identifier with NULL codepoint representing the inventory targets all A-IoT devices.

Proposal 4: The inventory category indicator should be included in inventory request message which is used to indicate the inventory operation is for single device, a group of devices, or all devices.

Proposal 5: If single device is to be found, the inventory request message only include this specific A-IoT device identifier.

Proposal 6: If a group of devices is to be found, the inventory request message may include a group ID or all A-IoT device identifiers within the group.

Proposal 7: If the A-IoT Device Identification information includes all A-IoT device identifiers within the group, the “start point” and “end point” or the length of each A-IoT device identifier should also be provided by A-IoT CN.

Proposal 8: If all devices are to be found, the inventory request message may include no A-IoT identifier with NULL codepoint representing the inventory targets all A-IoT devices.

In the study phase, RAN3 also reached a common understanding that the scope of inventory request may be included in inventory request message. As we discussed in the R3-250686, if the A-IOT RAN node dominates the reader selection, it needs to be informed the scope of inventory request which consists of the reader’s service area. Therefore, the A-IoT CN may include the reader’s service area to the corresponding A-IoT RAN node for reader selection. If the AIOTF dominates the reader selection, the A-IoT CN may include the selected reader ID to the corresponding A-IoT RAN node for reader selection in inventory request message.

Proposal 9: The A-IoT CN may include the selected reader ID or the reader’s service area to the A-IoT RAN node in inventory request message.

In addition to the information mentioned above, RAN3 can take some principles and conclusions on the inventory procedure made by SA2 TR 23.700-13 into account.

### 8.1.1 General ### The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: 1. A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIOTF performs the following functionality. … e. The AIOTF may provide the following assistance information to RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request. NOTE 2: If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation. NOTE 3: The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3. f. When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF. … ### 8.3.4 Principles on the procedures to support AIoT services To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported: - Inventory Procedure. - Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages. There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1). All the procedures follow have the following steps: … 3. For the requested operation the AIOTF: … 4). Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection. 5). The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

From the yellow highlight part, it is observed that the AIOTF constructs a request including a correlation identifier for the AIOTF to correlate the inventory responses to the request, which uniquely identifies the inventory transaction. The correlation identifier can be used by the A-IoT RAN node to identify the different inventory transactions from the AIOTF. Assumed that AIOTF may initiate multiple inventory transactions to an A-IoT RAN node, AIOTF should assign the different correlation identifier in the request message. As response, the A-IoT RAN node should reply with the correlation identifier in response message or failure message to enable the AIOTF to determine whether the inventory operation has been successfully initiated. After the A-IoT RAN node successfully initiates the inventory operation, it will report the device ID with the correlation identifier including in inventory report message.

Proposal 9: The corelation identifier should be included in inventory request/response/failure/report message.

For the indirect path deployment, as the intermediate node between the AIOTF and the A-IoT RAN node, the AMF may connect with multiple AIOTFs, and the A-IoT RAN node will receive the inventory operation requests from different AIOTFs sent by the same AMF. It is not enough to only add the corelation identifier in the A-IoT related messages because the same corelation identifier may assigned by different AIOTFs. To ensure that multiple inventory operations from different AIOTFs can proceed simultaneously, besides the corelation identifier, the AMF should additionally assign an AIOTF identifier to correlate the request from the AIOTF to enable the A-IoT RAN node differentiate the inventory operation from different AIOTFs. With the help of AIOTF identifier, the AMF can be routed back the requesting AIOTF which is returned with the response message related to the request.

Proposal 10: The AIOTF identifier should be included in inventory request/response/failure/report message.

From the green highlight part, SA2 propose that the approximate number of A-IoT devices may be transferred from the AIOTF to the A-IoT RAN node. The approximate number of A-IoT devices from AF is beneficial for inventory operation. If the number is available and sent from AIOTF, the A-IoT RAN node can effectively allocate resources in A-IoT paging procedure. For example, if ten devices are expected to be paged, allocating the equal amount of access occasions can avoid collisions as much as possible. the approximate number of A-IoT devices is optionally provided in the inventory request message from the AIOTF to the A-IoT RAN node.

Proposal 11: The approximate number of A-IoT devices is optionally provided in inventory request message.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the inventory procedure and signaling:

Proposal 1: It is proposed to introduce a class-1 A-IoT Inventory procedure between A-IoT CN and A-IoT RAN node, including INVENTORY REQUEST message and INVENTORY RESPONSE message.

Proposal 2: If the A-IoT RAN node is not able to perform inventory operation towards A-IoT device(s), the A-IoT RAN node can reply with the INVENTORY FAILURE message to reject the inventory request from A-IoT CN.

Proposal 3: It is proposed to introduce a class-2 Inventory Report procedure to provide the device ID from A-IoT RAN node to the A-IoT CN, including INVENTORY REPORT message.

Proposal 4: The inventory category indicator should be included in inventory request message which is used to indicate the inventory operation is for single device, a group of devices, or all devices.

Proposal 5: If single device is to be found, the inventory request message only include this specific A-IoT device identifier.

Proposal 6: If a group of devices is to be found, the inventory request message may include a group ID or all A-IoT device identifiers within the group.

Proposal 7: If the A-IoT Device Identification information includes all A-IoT device identifiers within the group, the “start point” and “end point” or the length of each A-IoT device identifier should also be provided by A-IoT CN.

Proposal 8: If all devices are to be found, the inventory request message may include no A-IoT identifier with NULL codepoint representing the inventory targets all A-IoT devices.

Proposal 9: The corelation identifier should be included in inventory request/response/failure/report message.

Proposal 10: The AIOTF identifier should be included in inventory request/response/failure/report message.

Proposal 11: The approximate number of A-IoT devices is optionally provided in inventory request message.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;