3GPP TSG-RAN WG3 Meeting #127	             R3-250687

Greece, GR, 17th – 21th Feb 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Command Procedure and Signaling

Document for:	Discussion

1	Introduction

In RAN#106 meeting, a WID on Solutions for Ambient IoT (Internet of Things) in NR has been approved. During the WI phase, only Topology 1 will be standardized and the following RAN3-led objectives have been identified within the general scope described in the WID:

- RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will provide our preliminary views on command procedure and signaling between A-IoT RAN and A-IoT CN to support command operation.

2	Discussion

2.1	Command Procedure and Signalling

In TR 38.769, companies completed the following message flow and procedural text for A-IoT command operation in Topology 1.

Figure 1 Message flow for A-IoT Command in Topology 1 1. The Inventory procedures are performed in step 1, which are the same as in Figure 6.5.2.1-1 in step 1-4. NOTE 1: Step 1 is performed for the "A-IoT Inventory and Command" case. Whether and under which conditions step 1 may be skipped in case of "Command-only" and which consequences this would have for the overall message flow depends on discussions led by SA2, SA3 and RAN2. NOTE 2: Whether and which way steps 2a and 2b may be part of step 1 is dependent on discussions led by SA2, SA3 and RAN2. 2a. The A-IoT CN node sends a Command Request message to the A-IoT RAN. 2b. The A-IoT RAN node coordinates the usage of A-IoT radio resources and allocates A-IoT radio resources for the A-IoT command session. 3. The A-IoT RAN node performs A-IoT command procedures towards the A-IoT device over the A-IoT radio interface. 4. The A-IoT RAN node sends a Command Response message to the A-IoT CN, if any command result is received from A-IoT device, the A-IoT RAN node may include the command result in the Command response message. NOTE 3: In step 4, the A-IoT RAN node may instead send a Command Failure message to the A-IoT CN indicating that the command procedure has failed.

As the outcome of study item, we can take this message flow as baseline for A-IoT Command case. It is noted that the A-IoT Command case depicted in the message flow is actually the "A-IoT Inventory and Command" case where the command operation is performed after the inventory operation. The issue for the "Command-only" case requires to be answered is whether and under which conditions the inventory operation may be skipped, and currently the answer still discussed in SA2, SA3 and RAN2 progress. From RAN3 point of view, we need to focus on the "A-IoT Inventory and Command" case first.

Proposal 1: RAN3 needs to focus on the "A-IoT Inventory and Command" case first.

During the "A-IoT Inventory and Command" case, after inventory operation is finished, the A-IoT CN initiates a class-1 Command Request procedure to the A-IoT RAN node, including COMMAND REQUEST message and COMMAND RESPONSE message. The purpose of Command Request procedure is to request A-IoT RAN node to perform command operation towards single A-IoT device. After receiving the command result from the A-IoT device, the A-IoT RAN node may reply with the command result to the A-IoT CN by the COMMAND RESPONSE message. In addition, if an A-IoT RAN node is not able to perform command operation towards A-IoT device, the A-IoT RAN node can reply with the COMMAND FAILURE message to reject the command request from A-IoT CN.

Proposal 2: It is proposed to introduce a class-1 Command Request procedure between the A-IoT CN and the A-IoT RAN node, including COMMAND REQUEST message and COMMAND RESPONSE message.

Proposal 3: If an A-IoT RAN node is not able to perform command operation towards A-IoT device, the A-IoT RAN node can reply with the COMMAND FAILURE message to reject the command request from A-IoT CN.

2.2	Information exchanged between A-IoT CN and A-IoT RAN for Command

The following contents captured in TR 38.769 are the principles related to the command case both made by RAN2 and RAN3.

#### 6.3.1 General aspects and overall procedure The following information are considered useful to be visible to the reader from CN: NOTE 2: It needs to be further discussed on whether following information is mandatory or optional. - The A-IoT service type (e.g., inventory, command) NOTE 3: It needs to be further discussed if more information on command type (e.g. read/write/disable) is useful. The reader needs to know whether a D2R response is expected in D2R direction and, if so, the expected D2R message size. It needs to be further discussed on the details on how the reader gets that information (related to the "expected D2R message size" discussion in clause 6.3.5). It needs to be further discussed on whether the command type, if needed, is explicit or inferred from the "expected D2R message size" if available. #### 6.5.1.2 Command Command can be sent by the A-IoT CN for a single device. For topology 1, A-IoT RAN node should be able to differentiate between command and inventory. NOTE 1: Whether command can be performed for a group of devices depends on RAN2, SA2 and SA3.

It can be seen that command can be sent by the A-IoT CN for a single device, and whether command can be performed for a group of devices depends on RAN2, SA2 and SA3. As the reference, we capture the interim conclusions in SA2 TR 23.700-13 which are reflected as follows:

### 8.3.2 AIoT services supported by the 5GC The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2: - Inventory: Request to perform an inventory operation. - Read: Request to read information from an AIoT Device. - Write: Request to write information to an AIoT Device. - Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled. NOTE 1: The security aspect of Enable and Disable is to be concluded by SA WG3. NOTE 2: The temporarily disabling and enabling of AIoT devices did not reach conclusion and will therefore not be supported in Release 19. ### 8.3.5 Information provided by the AF The AF provides one or some of the following information: - Information about AIoT services: - Service type indicating e.g. inventory or command (e.g. read or write). - Information to be used for AIoT reader selection: - At least one of the UE reader ID or the target area information. - Optionally, Information about the target AIoT device(s): - At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices. - Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8]: - Approximate number of AIoT devices. - Approximate D2R message size.

The service type for command operations has been expanded, including read/write/disable, and all of the command operations only apply to single A-IoT device. Both RAN3 and SA2 reach a consensus that command case is only towards single A-IoT device. In WI phase, RAN3 also should follow the conclusion this consensus.

Observation 1: From SA2 TR, the service type command operations include read/write/disable, and all of the command operations only apply to single A-IoT device.

Proposal 4: The command case is only towards single A-IoT device.

### 6.3.1 General aspects and overall procedure The study aims that the design on the A-IoT radio interface between reader and A-IoT device is common for Topology 1 and Topology 2. … The following information are considered useful to be visible to the reader from CN: NOTE 2: It needs to be further discussed on whether following information is mandatory or optional. - The A-IoT service type (e.g., inventory, command) NOTE 3: It needs to be further discussed if more information on command type (e.g. read/write/disable) is useful. The reader needs to know whether a D2R response is expected in D2R direction and, if so, the expected D2R message size. It needs to be further discussed on the details on how the reader gets that information (related to the "expected D2R message size" discussion in clause 6.3.5). It needs to be further discussed on whether the command type, if needed, is explicit or inferred from the "expected D2R message size" if available. - Whether the service is targeted for one or more than one A-IoT devices - The approximate number of target A-IoT devices of this service ### 6.3.5 A-IoT data transmission ### The use of the following assistance information is studied (the need of each is still to be decided): … - It is beneficial for the reader to know an estimate of expected D2R message size. This D2R message size information would be beneficial but it is not essential.

From the TR 38.769 shown above, it is noted that the service type needs to be visible to the reader from CN and to be further discussed if more information on command type (e.g. read/write/disable) is useful. From our point of view, the service type of A-IoT can help A-IoT RAN node to allocate resources efficiently for different types of services. For example, if the A-IoT service is read, A-IoT RAN node will reserve more bits to retrieve the content from A-IoT device. If the A-IoT service is write, A-IoT RAN node will only allocate limited resource to only receive the feedback on whether the result of write is successful or not. Also, SA2 proposes that AF provides service type indicating e.g. inventory or command (e.g. read or write). If AF provides such kind of information, the AIOTF correspondingly understand the indicated service type. If the AIOTF already knew the service type, there's no reason not to let the A-IoT RAN node know. Therefore, it is proposed to introduce the command service type which includes the read, write and disable in COMMAND REQUEST message.

Proposal 5: Introduce the command service type (Read/Write/Disable) in COMMAND REQUEST message.

Besides the service type, the expected D2R message size is beneficial for reader to know for resource allocation and scheduling. SA2 thinks that the approximate D2R message size is optionally provided by the AF, and AIOTF provides the reader with the approximate D2R message size based on AF request. With such information, the A-IoT RAN node could obtain the rough D2R message size. Hence, it can be seen that the approximate D2R message size can be optionally provided in the COMMAND REQUEST message.

Proposal 6: The approximate D2R message size is optionally provided in the COMMAND REQUEST message.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the procedure and procedure between A-IoT RAN and A-IoT CN to support command operation:

Observation 1: From SA2 TR, the service type command operations include read/write/disable, and all of the command operations only apply to single A-IoT device.

Proposal 1: RAN3 needs to focus on the "A-IoT Inventory and Command" case first.

Proposal 2: It is proposed to introduce a class-1 Command Request procedure between the A-IoT CN and the A-IoT RAN node, including COMMAND REQUEST message and COMMAND RESPONSE message.

Proposal 3: If an A-IoT RAN node is not able to perform command operation towards A-IoT device, the A-IoT RAN node can reply with the COMMAND FAILURE message to reject the command request from A-IoT CN.

Proposal 4: The command case is only towards single A-IoT device.

Proposal 5: Introduce the command service type (Read/Write/Disable) in COMMAND REQUEST message.

Proposal 6: The approximate D2R message size is optionally provided in the COMMAND REQUEST message.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;