3GPP TSG-RAN WG3 Meeting #R3-127       	                                                               R3-250697

Athens, Greece, Feb 17-21, 2025

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Discussion on OD-SSB based NES from neighbour gNB’s perspective

Document for:	Discussion and decision

# Introduction

RAN#102 has agreed the WI to specify the techniques to enhance the network energy savings as RP-234065.

There are three RAN3 related objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

RAN1 has the following relevant agreements:

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Notes:

- Scenario #2A refers to

  - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

- Scenario #3A refers to

  - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to

  - “When SCell activation is completed and SCell is activated” or

  - “After SCell activation is completed and SCell is activated”

In this paper, we will study the impact of activating the NES feature on the neighboring cell gNBs, and how a neighboring cell may activate OD-SSB transmission on the NES cell.

# NES Feature Discussion

## Supported NES mode

We define two modes of operation of a cell:

Default mode is when the cell under consideration transmits only period SSB.

NES mode is when the cell under consideration transmits OD-SSB (with/without always on SSB).

- Observation 1: A cell may operate in Default mode when it transmits only period SSB or NES mode is it transmits OD-SSB (with/without always on SSB).

## NES Feature Activation

When a cell transitions to a NES mode, it stops transmitting periodic SSB in Case #1 and perhaps reduces the instances of periodic SSB transmissions in Case #2. We propose that the decision to transition to NES mode be made by the cell gNB-DU, since it is responsible for transmission of SSB. Moreover, gNB-DU is the entity, which is responsible for Energy Saving Telemetry computation, and is aware of the need to conserve energy. The gNB-DU may also use Assistance Information from gNB-CU to make this decision. When the gNB-DU decides to transition to the NES mode, it informs the gNB-CU of the change in the SSB transmission condition over the F1 interface.

We propose that the NES mode activation and deactivation be controlled by the cell’s gNB-DU.  And the gNB-DU informs the gNB-CU of the change in the SSB transmission condition over the F1 interface.

## Impact on Neighbouring gNBs

There are certain features in the neighboring gNBs which may be impacted by the NES cell’s SSB transmission mode. Some of these features are:

- LTM/CHO/DAPS

- mTRP

- EN-DC/NR-DC

Some of the neighboring gNB’s functionalities like configuring the measurement objects in the UEs attached to it might also be impacted. If a UE is configured to report measurements on the NES cell’s SSB, and if the NES mode is activated, then the UE behavior is unknown. Furthermore, lack of this measurement can have detrimental effect on mobility scenarios. Other features like inter-frequency load balancing might also be affected by the lack of SSB transmission on cells which might have otherwise been considered as a target cell option for offloading the extra UEs.

Given the above, it is clear that SSB transmission in a cell has implications on the features and functionalities of the neighboring cells. Therefore, we propose that the NES Cell gNB-CU informs the neighboring cells gNB-CU of the current status of the SSB transmission of the NES Cell.

We propose that the NES Cell gNB-CU informs the neighbouring cells gNB-CU of the current status of the SSB transmission of the NES Cell.

Furthermore, any change in the NES status should also be notified by the NES Cell gNB-CU to the neighboring cells gNB-CU over the Xn interface. This may enable the neighboring cells to devise their strategy for the above-mentioned features and functionalities.

Any change in the NES status should also be notified by the NES Cell gNB-CU to the neighbouring cells gNB-CU over the Xn interface.

## Triggering OD-SSB from Neighbouring gNBs

The above-mentioned features act as a motivation for neighbouring gNBs to be able to trigger OD-SSB in the NES cell. When the trigger conditions are met (outside the scope of current discussion), the neighboring gNB-CU can send a OD-SSB Trigger message to the NES Cell gNB-CU over the Xn interface. This procedure may include a cause IE, which will provide additional information to the NES Cell. The NES Cell gNB-CU sends this trigger message to NES Cell gNB-DU, and the NES Cell gNB-DU may decide to continue to operate in NES mode, or deactivate it based on the trigger and cause IE received. It may also consider its own cell load, traffic conditions and energy consumption to make this decision.

We propose that the neighboring cell gNBs send a message containing cause IE to NES Cell gNB to trigger OD-SSB in the NES Cell over the Xn interface.

We propose that the NES Cell gNB-DU may decide on how to respond to the trigger for OD-SSB it receives from the neighbouring cell gNB.

In the case #1 of Not always on SSB, the neighboring gNB might not be aware of the NES cell, unless the NES cell gNB-CU has updated it about the NES mode activation of the NES cell. Optionally, the neighboring cell gNB maintains a NES NRT (Network Energy Saving Neighbor Relationship Table) which keeps a track of neighboring cells and their current NES status. The table can also contain information about the HO stats and feature support for each entry in the table. This enables the neighboring cell gNB to trigger OD-SSB for the most suitable NES Cell depending on the trigger condition.

The cell gNB may maintain a NES NRT (Network Energy Saving Neighbour Relationship Table) which keeps a track of neighbouring cells and their current NES status. The table can also contain information about the HO stats and feature support for each entry in the table.

In the case #2: Always on SSB, the above proposed table becomes non-essential, since the NES Cell is visible to the UEs attached to the neighboring cell gNBs.

In either case, on receiving the OD-SSB transmission trigger from a neighbor cell gNB-CU, the NES Cell gNB-CU sends this trigger message to NES Cell gNB-DU. The DU may take into consideration the following:

- Severity of the cause

- Traffic/Load condition

- Impact on its ES telemetry

- Duration for which it might be required to transmit OD-SSB

- SSB transmission mode required – OD-SSB or default SSB

The outcome of this decision could be to either continue in the NES mode, or begin OD-SSB transmission, or begin default SSB transmission.

The NES Cell gNB-DU informs the gNB-CU of its decision to change the current NES status over the F1 interface.

Once the NES Cell gNB-CU receives this information, either in response to the trigger from neighboring gNB, or independently as a decision from NES Cell gNB-DU to transition to the NES mode, it sends assistance information to the gNB-DU in terms of what parameters it may adopt for the chosen SSB transmission.

In response to the NES Cell gNB-DU informing NES Cell gNB-CU of the change in the NES mode, the gNB-CU sends the assistance information to the gNB-DU in terms of the SSB transmission parameters that gNB-DU may adopt for the chosen SSB transmission mode.

Like the NES NRT table proposed above to be maintained by the neighboring cells gNB, we propose that NES cells gNB also maintain their own version of NES NRT. In the NES cell gNB, this table will contain information about the PCell-SCell pairs, and also about the neighboring gNBs which have triggered request for OD-SSB transmission with the associated cause IE. This table provides information to gNB-CU which may be sent as Assistance Information to the NES Cell gNB-DU to enable it to decide on what SSB mode to support at any given time.

# Conclusion

We have the following observations and proposals:

- Observation 1: A cell may operate in Default mode when it transmits only period SSB or NES mode is it transmits OD-SSB (with/without always on SSB).

Proposal 1	We propose that the NES mode activation and deactivation be controlled by the cell’s gNB-DU.  And the gNB-DU informs the gNB-CU of the change in the SSB transmission condition over the F1 interface.

Proposal 2	We propose that the NES Cell gNB-CU informs the neighboring cells gNB-CU of the current status of the SSB transmission of the NES Cell.

Proposal 3	Any change in the NES status should also be notified by the NES Cell gNB-CU to the neighboring cells gNB-CU over the Xn interface.

Proposal 4	We propose that the neighboring cell gNBs send a message containing cause IE to NES Cell gNB to trigger OD-SSB in the NES Cell over the Xn interface.

Proposal 5	We propose that the NES Cell gNB-DU may decide on how to respond to the trigger for OD-SSB it receives from the neighboring cell gNB.

Proposal 6	The cell gNB may maintain a NES NRT (Network Energy Saving Neighbour Relationship Table) which keeps a track of neighbouring cells and their current NES status. The table can also contain information about the HO stats and feature support for each entry in the table.

Proposal 7	The NES Cell gNB-DU informs the gNB-CU of its decision to change the current NES status over the F1 interface.

Proposal 8	In response to the NES Cell gNB-DU informing NES Cell gNB-CU of the change in the NES mode, the gNB-CU sends the assistance information to the gNB-DU in terms of the SSB transmission parameters that gNB-DU may adopt for the chosen SSB transmission mode.

# Reference

Chairman’s Notes RAN3#125-bis

Chairman’s Notes RAN3#126