# 3GPP TSG-RAN WG3 Meeting #127	R3-250711

# Athens, GR, February 17th – 21st, 2025

# Agenda Item:	8.1

# Source:	ZTE Corporation

# Title:	Discussion on XR UL bit rate control

# Document for:	Discussions & Decision

- Introduction

RAN3 has received LS R2-2411218[1] from RAN2 on XR UL bit rate control with the following content:

===== content in RAN2 LS =====

RAN2 has made the following agreement regarding a solution on UL bit rate control

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

===== content in RAN2 LS =====

Additionally, RAN2 is also expecting RAN3 to provide the feedback for its agreement.

The intention of this contribution is to firstly clarify why RAN2 sent this LS and what RAN2 expect RAN3 to reply, then explain our view on the located RAN2 requirement based on the received LS and previous RAN2 discussion. The related CR and draft reply LS are also provided and can be found in our other contributions.

# 2 Discussion

In RAN2#128, RAN2 discussed the data rate control for XR function and confirmed that the rate control information shall be sent from gNB to UE by using MAC CE. But RAN2 can not make consensus on whether this information is per QoS flow level or per DRB level is FFS. More specifically, some companies in RAN2 believes that gNB looses visibility to QoS flow once the flow is mapped to DRB. Then QoS flow level rate control information is impossible. Meanwhile, other companies in RAN2 believes that there is already a feature in Rel-18 which relies on per QoS flow level estimation at gNB side and the feasibility for per QoS flow bit rate control was confirmed by RAN3.

At last, RAN2 finally preferred to send the above LS to RAN3 to further confirm whether gNB has the capability to perceive a QoS flow and/or configure information for the QoS flow once the QoS flow has been mapped to the DRB.

Proposal 1: RAN2 expects RAN3 to confirm whether gNB has the capability to perceive a QoS flow and/or configure information for the QoS flow once the QoS flow has been mapped to the DRB.

From RAN3 point of view, solve RAN2 concern is quite clear. RAN3 has already introduced the ECN marking at UPF function in Rel-18 which relies on the per QoS flow level estimation at gNB. gNB shall report the percentage of congestion level for DL and/or UL for a specific QoS flow if needed. Detail content for such function has been defined in TS 38.415 and shown below:

===== content in TS 38.415 =====

5.5.3.25	DL Congestion Information

Description: For the cases of ECN marking at UPF request, this field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked for a QoS flow.

For the case of congestion information request, this field should be interpreted as a percentage of congestion level in DL up to two decimal points for a QoS flow.

As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 2 octets.

5.5.3.26	UL Congestion Information

Description: For the cases of ECN marking at UPF request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a QoS flow.

For the case of congestion information request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a QoS Flow.

As an example, value 9574 corresponds to a percentage of 95.74%

Value range: {0..10000}.

Field length: 2 octets.

===== content in TS 38.415 =====

So lets ignore the discussion on the data rate control, from technical point of view, gNB has the capability to distinguish different QoS flows and configure/insight information to either of these QoS flows which has been mapped to a DRB based on the current mechanism.

Proposal 2: It is proposed for RAN3 to confirm that  gNB has the capability to distinguish different QoS flows and configure/insight information to either of these QoS flows which has been mapped to a DRB based on the current mechanism.

Hence, RAN3 shall send our understanding on the gNB capability to RAN2. A draft reply LS R3-250712 has been prepared from our side and can be considered as baseline.

Proposal 3: Send reply LS to RAN2 by considering the draft reply LS R3-250712as baseline.

In addition, we also prepare CR for the UL data rate control based on RAN2 discussion. Detail can also be checked in our other contributions.

# 3	Conclusion

Based on above analysis, we provide the following proposals.

Proposal 1: RAN2 expects RAN3 to confirm whether gNB has the capability to perceive a QoS flow and/or configure information for the QoS flow once the QoS flow has been mapped to the DRB.

Proposal 2: It is proposed for RAN3 to confirm that  gNB has the capability to distinguish different QoS flows and configure/insight information to either of these QoS flows which has been mapped to a DRB based on the current mechanism.

Proposal 3: Send reply LS to RAN2 by considering the draft reply LS R3-250712  as baseline.