3GPP TSG-RAN WG3 Meeting #127	R3-250719

Athens, Greece, 17 – 21 February 2025

Agenda Item:	16.3

Source: 				LG Electronics Inc.

Title:			Discussion on support of location report

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on Solutions for Ambient IoT (Internet of Things) in NR was agreed at RAN #106 [1]. One of the objectives in this WID is to specify necessary architectural aspects, and signaling and procedures including device location reporting at reader ID granularity as follows:

… - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2. …

In this contribution, we will discuss the issue on support of location report for the A-IoT Device and provide our view on it.

2.	Discussion

In TR 38.769, the RAN3 discussion for locating A-IoT devices is captured. There is the NOTE on whether and how to have awareness of Reader location in AIoTF needs further discussion.

TR 38.769: … ## 6.9 Locating ambient IoT devices ### 6.9.1 General A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported. NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future. ### 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion. NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion. NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion. …

SA2 also considers that the information related to the Reader supported by the A-IoT RAN is provided to the AIoTF. How to provide it to the AIoTF is pending to RAN3 decision.

TR 23.700-13: … The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: 1. A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. a. The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase. b. For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN. NOTE 1: Providing the UE Reader list to the RAN only applies to the RRC-based option. For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network. NOTE 2: Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications. NOTE 3: If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers. NOTE 4: The BS reader’s service area needs to coordinate with RAN WG(s). …

Regarding this issue, it is agreed that during the study of Ambient IoT, one or more Readers with its own service area can be co-located with the A-IoT RAN and locating the A-IoT device is supported at “Reader” granularity. When the A-IoT signaling from the A-IoT Device is received via a specific Reader, then the A-IoT RAN and the AIoTF can deduce/derive the location of the A-IoT Device considering the service area of the Reader associated to the A-IoT Device. Also, the A-IoT CN can use this information to select/determine an appropriate “Reader” close to the A-IoT Device. For example, when the AIoTF performs the Reader selection to forward the message towards the specific A-IoT Device, it may decide to select one or more Readers near to the last known Reader considering the maximum allowed end-to-end latency requirement from the AF. To do so, the A-IoT CN needs to know the Reader related information such as the Reader ID, its corresponding service area, and the ID of the serving A-IoT RAN. Considering the service area information for the Readers, the AIoTF can determine which Reader is close to the last known Reader. Also, when the AF provides the target area information for the requested service operation, the AIoTF can use the service area information to determine the Readers close to the A-IoT Device.

Observation 1: The service area of the Reader should be provided to the AIoTF.

According to the TR 23.700-13 and TR 38.769, there are two options to provide/configure the service area information for the Reader. First possible option to support this functionality is that the A-IoT RAN provides the Reader related information to the AIoTF during the NG setup procedure. This option enables the operator to dynamically manage/update the Reader related information in the AIoTF, whereas it causes the Stage 3 specification impact. We think that since this solution is very similar to the approach in TAC supported by the NG-RAN, foreseen specification impacts are already well known.

Observation 2: First option enables the operator to dynamically manage/update the Reader related information in the AIoTF.

Observation 3: Foreseen specification impacts on first option are already well known.

Another possible option is that the Reader related information is locally configured in the AIoTF by OAM. This can avoid the specification impact. However, it requires the operator’s effort to ensure the up to date information in AIoTF.

Observation 4: Second option can avoid the specification impact.

Observation 5: Second option requires the operator’s effort to ensure the up to date information in AIoTF.

Based on the above observations, the following proposal is suggested to RAN3:

Proposal 1: In Topology 1, the A-IoT RAN provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAN) to the A-IoT CN during the NG setup procedure.

3.	Conclusion

In this contribution, we focused on the issue on support of location report for the A-IoT Device and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: In Topology 1, the A-IoT RAN provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAN) to the A-IoT CN during the setup procedure for the XX interface.

4.	Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR,” RAN1 Vice-chair (Huawei), December, 2024.