3GPP TSG RAN WG3 Meeting #127	R3-250748

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	8.1

Source:	Ericsson

Title:	Response to R3-250553 and R3-250693

Document for:	Response paper

# 1	Introduction

This paper is a response to the discussion papers in [1] and [2], where the proponents expose their directions on RAN3’s action regarding the LS from RAN2 to SA2, CT1 (where RAN3 was only cc-ed) on usage of eDRX in RRC_INACTVE during an emergency PDU session sent by RAN2 in [3].

# 2	Discussion

Background:

RAN2 asked SA2 [3], cc-ing RAN3, if there may be cases where the UE has an emergency PDU session, but the gNB is not aware of this, i.e. the gNB may release the UE to RRC_INACTIVE with eDRX configured. This topic was discussed by SA2 at its January ad hoc meeting, but the conclusions and response LS were deferred to the February meeting. There are proposals for a CN-centric solution, or a solution left to NW implementation. It would be wise for RAN3 to await the conclusions of SA2 before approving a solution, if any. Especially as RAN3 has not been tasked to study the issue.

Nevertheless, looking at [1], the proponents propose the following NW- based solution:

- The AMF can currently flexibly enable/disable the INACTIVE eDRX mode usage in gNB by signaling or not the NR eDRX Information parameter in an NGAP Context modification; - The AMF is aware whether an emergency PDU session is ongoing. - Therefore, the reported issue can be avoided by specifying that the AMF should disable the usage of INACTIVE eDRX in the gNB as long as an emergency PDU session is ongoing. If an emergency PDU session is ongoing the gNB shall be configured by the AMF to disable the inactive eDRX.

This proposal is translated into AMF not sending the RRC Inactive eDRX information to RAN to disable the usage of eDRX. Note that other paging optimization features may be required to be disabled, such as PEI, and LP-WUS, etc.  by AMF not sending the relevant IE in the NGAP message. However, a change to RAN spec TS 38.300 is not required, as there is only a new requirement on the AMF. Also RAN3 does not specify the behaviour of a node based on the absence of the IE, it can be left to NW implementation

We note that however this solution has been discussed by SA2 but not concluded yet. Hence no RAN3 action is needed.

In [2], the proponents have additional proposal:

- RAN3 to initially discuss the two solutions below: - Option 1: when the AMF identifies the emergency PDU session, it updates the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE. - Option 2: when the NG-RAN itself identifies the emergency PDU session, it would not configure the RAN eDRX for the inactive UE

Option 1 is similar to the proposal from [1], however it is not agreed by SA2, and does not require to be captured in TS 38.300 as it is AMF behaviour.

Option 2 is incomplete/incorrect. The UE can have an emergency PDU session, without having an emergency call. For the emergency call a special ARP value is used. But when the emergency call is released, the UE can still have an emergency PDU session and such session will be kept for a while based on the inactivity timer in CN for the EMC PDU session. During that time the UE can receive a call back without this special ARP value, i.e. this is normal call, but the UE should not experience and eDRX delay.

We note that another solution being discussed by SA2 is to keep the UE in RRC_ connected. This way, eDRX, PEI, LP-WUS, and other paging optimization features will not be used because paging will not be triggered when UE has emergency PDU session. If this solution is agreed in SA2, there will be no need for a RAN3 impact.

Observation 1:  When UE has emergency PDU session, if the UE is kept in RRC_CONNECTED, then paging is not triggered and eDRX (together with paging optimization features that may cause delay to callback) are not used.

For the time being, RAN3 can wait for SA2 to liaise between RAN2 and RAN3 and be informed of their final solution.

Observation 2: The documents from [1] and [2] propose solutions that have been discussed by SA2 and not concluded on yet.

Observation 3: No need to capture RAN behaviour based on absence of an IE from AMF. If requirement is put on AMF side, then TS 38.300 does not need to be impacted.

Proposal 1: RAN3 to wait for SA2 to reach their conclusion on eDRX during EMU. No action needs to be taken for now.

# 3	Conclusion

Observation 1:  When UE has emergency PDU session, if the UE is kept in RRC_CONNECTED, then paging is not triggered and eDRX (together with paging optimization features that may cause delay to callback) are not used.

Observation 2: The documents from [1] and [2] propose solutions that have been discussed by SA2 and not concluded on yet.

Observation 3: No need to capture RAN behaviour based on absence of an IE from AMF. If requirement is put on AMF side, then TS 38.300 does not need to be impacted.

Proposal 1: RAN3 to wait for SA2 to reach their conclusion on eDRX during EMU. No action needs to be taken for now.

# 4	References

- R3-250553, Emergency Services and Inactive eDRX, Nokia

- R3-250693, Discussion on the emergency services and eDRX , Huawei

- R3-250006, LS on Emergency Services and eDRX, RAN2