3GPP TSG RAN WG3 Meeting #127	R3-250754

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	8.3

Source:	Huawei

Title:	Response to R3-250739

Document for:	Response paper

1	Introduction

This paper is a response to the discussion papers in [1], where the proponents, based on the received LS in [2], suggested to add new value for MBS communication service type and type indication in the QMC report. This paper tries to have further analysis on the necessity of such proposals.

2	Discussion

In [2], upon the request from RAN3, SA4 updated their specifications accordingly as follows:

| Element or Attribute Name |  | Use | Description |
| --- | --- | --- | --- |
|  | (Skip) |  |  |
|  | @communicationServiceType | OD default= ”all” | When present, this attribute indicates in which communication service type the QoE collection is requested: - The value mbsMulticast refers to the MBS Multicast communication service per clause 21.1 of TS 38.300 [71]. - The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300 [71]. - The value all refers to all communication service types. When absent, quality metrics collection is requested for all MBS modes. |

Thus, it was proposed in [1] that a new a new enumerated value “all” should also be added in 38.413, as follows:

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ..., all) |
| --- | --- | --- | --- |

According to SA4 spec, the intention of “all” is just to give a default value for this element, if “all” is to be introduced in RAN3, it will actually add confusions, since from RAN3 perspective, communication service type is either multicast or broadcast, no other meaning (unicast was already supported by other means). Note that in 38.413, we already have a mandatory IE to indicate which service for QoE measurement, see below:

| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. |
| --- | --- | --- | --- | --- |

The two IEs are enough for the RAN to generate configuration and sent to UE to perform QoE measurement, thus there is no need to introduce another enumerated value “all” in RAN3 side, needless to say that RAN node should not configure a UE to collect QoE measurements for all MBS communication service types at the same time, since RAN node also needs to decide the RRC mode for each QoE measurement.

Observation 1: RAN3 has already defined enough information to generate configuration and sent to UE to perform QoE measurement, while the introduction of another enumerated value “all” may cause confusions.

There is another proposal in [1] which proposed to add into the QoE reports an indication of whether the reported QoE measurements refer to Multicast, or Broadcast. It seems to be an optimization though, it is questionable if we go into details, since both CN and UE knows whether the ongoing mode is broadcast or multicast, and OAM also indicates the communication service type to measure, see below in TS 28.622:

| mBSCommunicationServiceType | This IE indicates for which type of MBS communication service the QoE measurement configuration pertains to. See the clause 4.5.1 of TS 28.405 [50] for additional details. allowedValue: BROADCAST, MULTICAST | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| --- | --- | --- |

Since OAM can, in its QoE configuration, indicate the communication service type to perform QoE measurement, and it is also true that the OAM could match the received QoE report with the corresponding QoE configuration.

Observation 2: OAM can, in its QoE configuration, indicate the communication service type to perform QoE measurement, OAM should also be able to match the received QoE report with the corresponding QoE configuration

With the two observations above, we think RAN3 needs to discuss the necessity of introducing another enumerated value “all” and, the necessity of introducing the communication service type indication in the QoE report.

Proposal: RAN3 to discuss the necessity of introducing another enumerated value “all” and, the necessity of introducing the communication service type indication in the QoE report.

In our understanding, it seems not necessary.

3	Conclusion

This response paper tries to discuss the necessity of the proposals in [1], and reach the following observations and proposal for RAN3 to discuss.

Observation 1: RAN3 has already defined enough information to generate configuration for the UE to perform QoE measurement, while the introduction of another enumerated value “all” may cause confusions.

Observation 2: OAM can, in its QoE configuration, indicate the communication service type to perform QoE measurement, OAM should also be able to match the received QoE report with the corresponding QoE configuration

Proposal: RAN3 to discuss the necessity of introducing another enumerated value “all” and, the necessity of introducing the communication service type indication in the QoE report.

4	References

- R3-250739, MBS Communication Service Type for QMC (Ericsson)

- R3-250026, LS Reply on MBS Communication Service Type (SA4(Huawei))	LS in