3GPP TSG-RAN WG3 #127	R3-250789

Athens, Greece, 17th – 21st , February, 2025

Agenda Item:	11.4

Source:	CMCC (moderator)

Title:	Summary of Offline Discussion on AI/ML NRDC

Document for:	Approval

- Introduction

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

CB: # AIRAN4_NRDC

- Check the open issue above

- Capture agreements and provide TP if agreeable

(moderator - CMCC)

Summary of offline disc R3-250789

- For the Chairman’s Notes

Whether and how to introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information needs further discussion.

- Discussion

In Tuesday’s online discussion companies showed their views on the following as was captured the Chair’s meeting minutes:

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

ZTE, Ofinno, Lenovo, QC : Yes, it’s needed and useful.

HW, LGE: There is no consensus in SI.

Nok: It’s infeasible and complicated

SS: It’s workable.

CATT: Feasible.

Lenovo: The similar infor already has been introduced in mobility cases.

E///: Not only the PSCell but also the SCG bearers

Seems there is no doubts that introducing predicated PSCell as one of assistance information is beneficial on helping SN to select PSCell. However, there are still different views whether MN can predicate PSCell.

Companies are welcome to provide technical opinions on whether it is feasible to provide predicated PSCell in DC procedures (e.g., SN Addition, MN-initiated SN change) in this doc.

| Company | Feasible or not? | Comments |
| --- | --- | --- |
| CMCC | Yes | In legacy SN Addition and MN-initiated SN Change procedures, MN could provide measurement results of PSCells to the target SN for SN to choose and configure the SCG cell(s), which means MN has the enough input data for AI/ML model to generate the predicted PSCell(s). In Rel-18, the UE Trajectory Prediction is transferred from source node to target node for AI/ML based Mobility Optimization of single connectivity. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover. The similar solution could also apply to NR-DC Mobility Optimization. |
| Samsung | Yes | In NR-DC, the SN may not have sufficient information for UE trajectory prediction. In some cases like SN change without inter-MN handover, the time for UE to stay in a SN is short, the collected information from UE is not enough to do the prediction. There is another case that the SN has no AI/ML capability. It can not be assumed that all nodes in the network are equipped with AI/ML function with the reason that AI/ML model requires the high computation power comparing with the normal communication. So, we believe it is beneficial for the MN to predict the PSCell id for SN. |
| Ofinno | Yes | The MN may use its AI/ML model to predict the UE trajectory. The MN may use such prediction to select a target SN for SN addition or change. UE trajectory prediction is cell-level granularity, which assumes that the MN has very good idea about potential PSCell and/or SCG configuration for the UE. Once such inference results are available, it is not clear why they shall be discarded instead of sending them to the SN to assist in selecting PSCell for the UE / configuring SCG for the UE. Alternatively, the MN may send to the SN several PSCell / SCell predicted candidates for the SN to take into account during PSCell/SCell selection. This may be further enhanced by adding priority/preference information to each predicted candidate PSCell/SCell to assist SN in selecting PSCell / configuring SCG for the UE. |

- References

[1] R3-250664, Discussion on AIML based Mobility Optimization for NR-DC (CMCC, CATT, ZTE Corporation, Lenovo)

[2] R3-250089, Mobility Optimization for NR-DC (Ofinno, LLC)

[3] R3-250141, (TP to 38.423) Addition of predicted PSCell ID within DC procedure (ZTE Corporation, Samsung, Qualcomm, CATT, CMCC, China Unicom)

[4] R3-250375, (TP for AI/ML BLCR to TS 38.423) AI/ML-based Mobility Optimization in NR-DC (Huawei)

[5] R3-250142, (TP to 38.423) Discussion on Mobility Optimization for NR-DC (ZTE Corporation)

[6] R3-250172， (TP to BLCR TS38.423) NR-DC mobility optimization (NEC)

[7] R3-250275, Discussion on UE performance metric in AIML for NR-DC (Lenovo)

[8] R3-250277, (TP to BLCR 37.340) AIML for NR-DC (Lenovo, CATT, ZTE Corporation, Samsung)

[9] R3-250343, Discussion on AI/ML based Mobility Optimization for NR-DC (China Telecom)

[10] R3-250344, (TP to TS38.423) Support of AIML based Mobility Optimization for NR-DC (China Telecom)

[11] R3-250381, Discussion on open issue of mobility optimization for NR-DC (LG Electronics Inc.)

[12] R3-250459, AI/ML support for NR-DC (Ericsson, Jio Platforms (JPL), InterDigital, FiberCop)

[13] R3-250460, (TP to TS 38.423) AI/ML support for NR-DC (Ericsson, Jio Platforms (JPL), InterDigital, FiberCop)

[14] R3-250573, Procedures for AI/ML Mobility Optimization in NR-DC (Nokia)

[15] R3-250574, (TP to TS 38.423) AI/ML Mobility for NR-DC (Nokia)

[16] R3-250580, Discussions for AI/ML assisted NR-DC (Jio Platforms JPL)

[17] R3-250595, AI/ML in DC scenario (CATT)

[18] R3-250596, (TP for TS 38.423) Introducing essentially per-DRB UE performance result for DC (CATT)

[19] R3-250618, Discussion on AI/ML for mobility in NR-DC (Samsung)

[20] R3-250665, (TP to 38.423) AIML based Mobility Optimization for NR-DC (CMCC)