3GPP TSG RAN WG3#127	R3-250807

17th February – 21st February - 2025

Athens, Greece

Agenda Item:	13.2

Source:	China Telecom (moderator)

Title:	Summary of offline discussion on MobilityEnh_LTM

Document for:	Approval

Introduction

CB: # ME1_LTM

- continue discuss on stage2 and stage 3 according the online agreements.

- capture the agreements in the TPs.

(moderator - CT)

Summary of offline disc R3-250807

For the Chairman’s Notes

Propose the following:

TP for TS38.300, R3-250058 rev [in R3-250885] – Agreed

TP for TS37.340, R3-250416 rev [in R3-250896] – Agreed

TP for TS38.423, R3-250366 rev [in R3-250892] – Agreed

TP for TS38.473, R3-250297 rev [in R3-250884] – Agreed

TP for TS37.483, [in R3-250904] – Agreed

Propose to capture the following:

The format of the new introduced IE is same with the gNB-DU ID.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

Discussion

In this offline, we plan to continue to discuss the remaining issues on support of inter-CU LTM.

Support of inter-CU LTM in Standalone scenario

SP CSI-RS configuration

During the online section, we agreed that it is the source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell, and agreed to adopt the class-1 procedure for the SP CSI-RS activation/deactivation procedure.

For both inter-CU and intra-CU cases: For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell. For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

In this section, we can start with intra-CU case and discuss the whether to reuse the existing F1AP message or introduce a new class-1 message for the SP CSI-RS activation/deactivation:

- Option 1: Reuse the current F1AP procedure (i.e. UE Context Modification (gNB-CU initiated) and UE Context Modification Required (gNB-DU initiated)).

- Option 2: Introduce a new UE-associated procedure over F1AP.

Similarly, for inter-CU case, we can further analysis whether to reuse the existing XnAP message or introduce a new class-1 message for the SP CSI-RS activation/deactivation:

- Option 1: Reuse the current XnAP procedure (i.e. Handover Request and LTM Configuration Update procedure).

- Option 2: Introduce a new UE-associated procedure over XnAP.

In this session, we can discuss which option to adopt.

Moderator summary:

Nokia, LG, ZTE, HW, CATT: prefer to introduce a new F1AP procedure.

E///, SS, QC: prefer to reuse the current F1AP procedure.

No consensus, to be continued…

Early RACH Configuration

During the online session, we reach following agreement towards the early RACH configuration:

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource(FFS)) to the candidate gNB-CU to request early RACH configuration.

In R18 LTM, the early RACH resources can be shared among UEs belonging to same DU, and we introduced the source gNB-DUs List IE in Early UL Sync Configuration IE in F1AP to indicate candidate DU(s) to allocate the different early RACH resource. Similar mechanism should be introduced in XnAP. However, as a gNB cannot explore its own architecture to neighbor gNB(s), companies suggest to introduce a new ID of early RACH configuration resource. Here, we can finalize the new IE and solve the remaining issue left during the online session.

The new ID of early RACH configuration resource can be named as:

- Option 1: Early UL Sync Configuration Provider ID

- Option 2: Early RACH resource request ID

- Option 3: gNB-DU ID

- Option 4: Others?

In this session, we can discuss which option to adopt.

Moderator summary:

The format of the new introduced IE is same with the gNB-DU ID.

In addition, the pair of (gNB ID, new ID of early RACH configuration resource(FFS)) is also needed to be transferred from candidate gNB-CU to candidate DU(s) during handover preparation, the F1AP impact is foreseen. If we reach consensus on the IE structure of new ID of early RACH configuration resource, we can further analysis the corresponding F1AP impact.

One simple way is to reused the current LTM gNB-DUs List IE and introduce the gNB ID of the corresponding DU(s) in F1AP. In [R3-250297, Huawei], it proposed to introduce the LTM gNB ID IE under the current Early Sync Information Request IE to request early RACH configuration.

| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |

Proposal: The candidate gNB-CU sends the gNB ID of the corresponding gNB-DU(s) to the candidate gNB-DU to request early RACH configuration.

Moderator summary:

Agree the TP for F1AP.

Reference configuration

In RAN2#127 meeting, RAN2 agreed to support one single reference configuration in Rel-19 inter-CU LTM:

Agreements on inter-CU LTM Reference configuration: - Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

In R18, the CU can either request the source DU or the candidate DU to provide a reference configuration, similar mechanism should be introduced for inter-CU case.

Take the F1AP as the baseline, for inter CU case, the source gNB could provide the LTM reference configuration to the candidate gNB explicitly, e.g., via the Handover Request message or LTM configuration message. In addition, if the source gNB doesn’t have the LTM reference configuration, it may request a candidate gNB to provide the LTM reference configuration.

In [R3-250238, Google], it raises following proposals:

Proposal 1: The source gNB can either request a reference configuration or, if available, provide a reference configuration for LTM in a Handover Request message.

Proposal 2: A candidate gNB, if being requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message.

Proposal 3: The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge message.

In this session, we can check whether the above proposal is acceptable.

Moderator summary:

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

Handling of multiple UE associations

Issue1: UE association setup during initial handover preparation

During the online session, we discussed how to identify the same UE from multiple Handover preparation procedure but reached no consensus. Based on the online discussion and companies’ contributions, moderator summarized following options for further discussion:

- Option 1: Reuse the existing mandatory NG-C UE associated signalling reference IE in the XnAP HANDOVER REQUEST. [NEC]

- Option 2: Support one UE association during the initial preparation phase. [LGE, ZTE]

- Option 3: Reuse the C-RNTI information. [Lenovo]

- Option 4: Reuse the same Target NG-RAN node UE XnAP ID assigned when the first LTM candidate is prepared. [E///]

- Option 5: Introduce a new LTM ID to identify a specific LTM procedure. [CATT]

In this session, we can down select from the above options.

Moderator summary:

LG, HW: Option 1 and option3 can be used to identify the UE association. Check whether option2 is acceptable.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Issue2: UE association setup after each LTM cell switch (i.e. UE association between the new serving gNB and candidate gNBs)

For subsequent LTM, after each cell switch, the UE association needs to be established between the new serving gNB and candidate gNB(s). In the last RAN3 meeting, it was agreed to reuse the LTM Configuration Update procedure to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch to support subsequent LTM.

RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

For subsequent LTM, the candidate gNB should identify that the UE is the same as the previously associated between the old serving gNB and the current candidate gNB.

If we reach consensus on the handing of multiple UE associations, we can further analysis how the candidate gNB identify that the UE is the same as the previously associated between the old serving gNB and the current candidate gNB. Companies’ proposals are cited as below:

In [R3-250058, Nokia]: Proposal 9: The Xn LTM CONFIGURATION UPDATE message is used by new Source gNB to exchange Xn UE AP Identifiers and indicate to signal the change of gNB. In [R3-250415, CATT]: Proposal 5: Discuss how to realize the UE context association in-between candidate gNB. Potential solution includes: When the gNB2 establish UE association with gNB3 - Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID. - Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure. - Option 3: gNB2 shall include the assigned target XnAP ID, which is assigned by gNB3 in initial LTM candidate cell request procedure ( gNB1 send HO request to gNB3 for request candidate cell 3). In [R3-250297, Huawei]: Proposal 12: After each cell switch, the new serving gNB signals the new allocated source gNB UE XnAP ID(s) to other candidate gNBs in the LTM Configuration Update message. The candidate gNB replies with the target gNB UE XnAP ID in LTM Configuration Update Acknowledgement message. In addition, the UE XnAP ID allocated by the candidate gNB for the initial source gNB is included in the LTM Configuration Update message. In [R3-250283, Lenovo]: Proposal 6: The source gNB transfers the UE XnAP ID of other candidate gNBs received in the Handover Request Acknowledge message to the new source gNB. Proposal 7: The new source gNB sends the received UE XnAP ID of candidate gNB from the source gNB to the corresponding candidate gNB via the LTM Configuration Update message after the LTM cell switch. In [R3-250641, Samsung]: Proposal 6: Introduce last source node UE XnAP ID in LTM Configuration Update message from new source gNB to other candidate gNBs after inter-CU LTM Cell Switch, so that candidate gNB can identify the same UE and setup the UE associated Xn singnalling connection. In [R3-250340, CT]: Proposal 11: To support subsequent LTM, the old serving gNB needs to send the Candidate node UE XnAP ID(s) to the new serving gNB via Cell Switch Notification message after each LTM cell switch. In [R3-250647, LGE]: Proposal 15: FFS on which UE XnAP IDs to take as reference, either “Source UE XnAP ID” assigned by the old serving CU, or “Target UE XnAP ID” assigned for the old serving CU, or both.

Moderator summary:

Skip.

LTM configuration sync up among candidate gNBs

To support subsequent inter-CU LTM, some LTM configurations, e.g., early sync configuration, configuration ID, and data forwarding addresses, etc may need to sync up among candidate gNBs.

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

(the step 7 is CU->DU: UE CONTEXT MODICATION REQUEST; the step 19 is DU->CU: DU-CU CELL SWITCH NOTIFICATION; the step 22 is DU->CU: ACCESS SUCCESS)

RAN3 to discuss how the LTM configurations are synced up among candidate gNBs in case of subsequent LTM.  Similar as F1AP, there are three possible options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) during the handover preparation phase.

- Option 2: Send the collected LTM information to the target gNB when the source gNB sends the LTM Cell Switch Command to the UE.

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

In this session, we can check whether the following proposal is acceptable:

Proposal 1: Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses.

Proposal 2: It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Moderator summary:

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Data forwarding

In the last RAN3 meeting, we left a remaining issue related to data forwarding:

RAN3#126: Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

During the final CB discussion in the last meeting, it seems most companies think it is reasonable to enable the source gNB to initiated data forwarding after the LTM Cell Switch decision as soon as possible, since the main purpose for LTM is to reduce the data interruption during mobility procedures. The only controversial point is the wording of this data forwarding procedure. In current specification, we defined following data forwarding procedures:

TS38.300: Early Data Forwarding: data forwarding that is initiated before the UE executes the handover. Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

Since the LTM cell switch is initiated by the source gNB and not the UE, so the source gNB can initiate data forwarding as soon as it sends the LTM Cell Switch Command to the UE.

In this section, we can check whether the following proposal is acceptable:

Proposal: Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

Moderator summary:

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

Based on companies’ contribution, another data forwarding issue that need to be solved is the data forwarding address transfer among candidate gNBs for subsequent LTM. Moderator summarized the following options to be further discussed:

- Option 1: The data forwarding address is sent to the candidate node in the XnAP CELL SWITCH NOTIFICATION message

- Option 2: The data forwarding address is sent together with the collected LTM information during the LTM preparation phase or later steps (up to implementation)

- Option 3: The new serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Question: Which option is agreeable? And which message to use?

Moderator summary:

Support of inter-CU LTM in NR-DC scenario.

During the online section, we reached following agreements towards applicable scenarios for inter-CU LTM in NRDC:

RAN3 agree the following scenarios to support LTM with NR-DC: 1.SN initiated inter-CU SCG LTM without MCG changes (high priority) 2.Inter-CU MCG LTM without SN release 3.Inter-CU MCG LTM with SN release 4.Inter-CU MCG LTM with SN addition

SN initiated inter-CU SCG LTM without MCG changes

For inter-CU SCG LTM, following agreements were reached in RAN2 previous meetings:

RAN2#127: - Inter-CU SCG LTM: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. - Inter-CU MCG LTM: 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure. RAN2#127bis: 3. The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9. Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10. Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. 11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM RAN2#128: 4. RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. 5. RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19. 6. From RAN2 perspective, the following coexistence cases in NR-DC can be supported: - Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM - Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations. 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message. 9. RAN2 to support intra-CU MCG LTM with SCG configuration. 10. It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. 11. RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this session, we can discuss the stage-2 signaling design for SN-initiated inter-CU SCG LTM by taking into account the above RAN2 agreements.

The figure of SN initiated inter-CU SCG LTM procedure from the endorsed running CR R2-2410928 in RAN2 #128 is cited as below:

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

LTM Preparation:

In step 1~3, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN. Then the MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. Within the list of cells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare for SCG LTM, considering of the maximum number indicated by the MN. For each prepared PSCell, the candidate SN provides the corresponding SCG radio resource configuration, and may also provide the L1 RS (e.g. SSB or CSI-RS) configuration for L1 measurement, early UL sync configuration, or TCI-state configuration. The candidate SN sends those configurations to the MN via SN Addition Request Acknowledge message. And then the MN may send the Xn-U Address Indication message to the candidate SN(s) to provide the information for data forwarding and/or trigger the early data forwarding.

5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

Proposal 1: For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

Proposal 2: Then the MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Proposal 3: Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

Moderator summary:

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

Then, in step 5-6, the MN initiate the SN modification procedure towards the source SN and candidate SN(s), to transfer the LTM related configuration of candidate cells.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

Proposal 4: The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

Moderator summary:

The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

In step 7-8, in order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), e.g. via MN initiated SN modification procedure. And then the candidate SN can update the candidate SCG configuration (e.g. generating the L1 CSI report configuration), accordingly.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Proposal 5: In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

Moderator summary:

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

In step 9/10, the MN sends the RRCReconfiguration message to the UE including the SCG LTM configuration. And the UE responses an RRCReconfigurationComplete message to the MN. The MN RRCReconfigurationComplete message includes an SN RRCReconfigurationComplete message to the source SN.

In step 11, the MN informs the source SN with the SN RRCReconfigurationComplete message via SN Change Confirm message, to indicate that SCG LTM is prepared.

Early sync:

In step 13/14a, the UE performs DL/UL sync with the LTM candidate cell(s) before receiving the cell switch command. For the PDCCH order triggered early RACH. In step 14b, the candidate SN(s) needs to transfer the calculated TA value of candidate cell(s) to the source SN via the MN. The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information between the MN and the source/candidate SN.

Proposal 6: The TA information transfer message can be reused to transfer the TA information from the candidate SN to the source SN via the MN.

Moderator summary:

To be continued.

LTM cell switch execution:

In step 15, the UE sends the L1 measurement report(s) to the source SN (i.e. source SN DU). In step 16/17, the source SN decides to execute LTM cell switch to the target cell by sending an LTM cell switch command MAC CE to the UE.

In step 18/19, the source SN sends a notification message to the target SN via the MN, to indicate the triggering of LTM to the UE. The message may include the target cell ID, the selected TCI-state ID(s) of the target cell. We can simply extend the existing cell switch notification procedure via F1 interface to Xn interface.

Proposal 7: The Cell Switch Notification message can be reused to inform the selected target cell from the source SN to the target SN via the MN.

Moderator summary:

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

LTM cell switch completion:

In step 20, the UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell. If the UE has valid TA of the target cell, the UE skips random access procedure towards the target cell, i.e. RACH-less LTM. The order the UE sends the MN RRCReconfigurationComplete message and performs the Random Access procedure towards the target SCG is not defined. If the UE has performed a RA procedure in step 20, the UE considers that LTM cell switch execution is successfully completed when the random access procedure towards the target SCG is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the target SN has successfully received its first UL data.

In step 21, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. In step 22, and then the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete message.

In step 27, the data forwarding procedure is performed.

In step 28, if applicable, the update of the UP path towards the core network is performed via a PDU session path update procedure.

In step 29, the MN initiates XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB, to inform the execution of SCG LTM. The detailed procedure is similar to SN initiated inter-SN SCPAC.

Avoid simultaneous MCG and SCG LTM

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

For coexistence of MCG and SCG LTM, following agreements were reached in RAN2#127bis meeting:

- RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Besides, in RAN3#123bis meeting, we reached following agreement related to inter-CU LTM in NR-DC:

- Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Regarding this issue, we need to design solutions to avoid simultaneous configuration for inter-CU MCG and SCG LTM.

During the online section, we mentioned following two options:

- Option 1: The MN/source SN needs to inform the other node once it decides to initiate an inter-CU LTM.

- Option 2: The MN rejects the SN-initiated inter-CU SCG LTM request with a proper cause value.

In this section, we can down select from the above options.

Moderator summary:

To be continued.

TP/BLCRs

Based on the chairman notes’ s guidance, in this offline discussion, we should work on stage2/3 TPs.

TP for TS38.300

The TP for TS38.300 is proposed in [0058, Nokia], [0298, HW], [0366, E///], we can check the contribution if possible.

Nokia

TP for TS37.340

The TP for TS37.340 is proposed in [0416, CATT], [0117, ZTE], [0366, E///], we can check the contributions if possible.

CATT

TP for TS38.423

The TP for TS38.423 is proposed in [0366, E///], [0116/0117, ZTE], [0238, Google], [0283, Lenovo], [0641, SS], [0160, NEC], we can check the contributions if possible.

E///

TP to TS38.473

The TP for TS38.473 is proposed in [0297, HW], we can check the contribution if possible

HW

TP to TS37.483

The TP for TS37.483 is proposed in [0647, LGE], we can check the contribution if possible

LGE

Conclusion, Recommendations [if needed]

If needed

References

| R3-250160 | (TP to BL CR for TS 38.423 on Inter-CU LTM) Rel-19 inter-CU LTM issues (NEC) | other |
| --- | --- | --- |
| R3-250297 | (TP for LTM BLCR for TS38.473): Further discussion on inter-CU LTM procedure (Huawei) | other |
| R3-250365 | Further discussion on inter-CU LTM (Ericsson) | discussion |
| R3-250058 | Discussion on Inter-gNB-CU LTM Procedure (Nokia) | discussion |
| R3-250238 | (TP to BL CR to 38.300 and 38.423) Support for reference configurations for inter-CU LTM (Google) | other |
| R3-250090 | Issues on Data Forwarding for Inter-CU LTM (Ofinno, LLC) | discussion |
| R3-250192 | Signalling enhancements for inter-CU LTM HO (Qualcomm Incorporated) | discussion |
| R3-250340 | Discussion on signaling enhancement for inter-CU LTM (China Telecom) | discussion |
| R3-250539 | Discussion on inter-CU LTM non-DC case (NTT DOCOMO INC..) | discussion |
| R3-250415 | Discussion for Inter-CU LTM (CATT) | discussion |
| R3-250640 | Discussion on Inter-gNB-CU LTM (Samsung) | discussion |
| R3-250647 | Keep continuing discussions on Inter-CU LTM signaling design (LG Electronics Inc.) | discussion |
| R3-250689 | Discussion on inter-CU LTM (CMCC) | discussion |
| R3-250116 | (TP to TS38.423) inter-CU LTM (ZTE Corporation) | other |
| R3-250227 | Initial considerations on Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-250283 | [TP to BLCR for TS 38.423] Inter-CU LTM (Lenovo) | other |
| R3-250059 | Discussion on Inter-gNB-CU LTM Dual Connectivity (Nokia) | discussion |
| R3-250161 | discussion Rel-19 inter-CU LTM in DC scenario (NEC) | discussion |
| R3-250416 | (TP to 37.340) Discussion for Inter-CU LTM in DC (CATT) | discussion |
| R3-250341 | Discussion on support of inter-CU LTM in NR-DC scenario (China Telecom) | discussion |
| R3-250529 | Discussion on Inter-CU LTM Dual Connectivity (Jio Platforms (JPL)) | discussion |
| R3-250117 | (TP to TS37.340 and TS38.423) SN initiated inter-CU SCG LTM (ZTE Corporation) | other |
| R3-250540 | Discussion on inter-CU LTM DC case (NTT DOCOMO INC..) | discussion |
| R3-250284 | General aspects of inter-CU SCG LTM (Lenovo) | discussion |
| R3-250263 | gNB-DU initiated LTM candidate cell reconfiguration (Rakuten Inc, Qualcomm Inc) | discussion |
| R3-250298 | (TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure (Huawei) | other |
| R3-250366 | (TP for TS 38.300, TS 38.423, TS 37.340) – Support for inter-CU LTM (Ericsson) | other |
| R3-250641 | (TP to BLCR for TS 38.423) Xn impact for Inter-gNB-CU LTM (Samsung) | other |
| R3-250259 | gNB-DU initiated LTM candidate cell reconfiguration (Rakuten Mobile, Inc) | Discussion withdrawn |

Appendix

Previous meeting agreements:

RAN3#123bis:

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

RAN3#124:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

RAN3#125:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

RAN3#125bis:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

- Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

RAN3#126:

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

RAN3#127(Online Section):

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource(FFS)) to the candidate gNB-CU to request early RACH configuration.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

RAN3 agree the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition