3GPP TSG-RAN WG3 Meeting #127	R3-250889

Athens, Greece, 17-21 Feb 2025

Agenda Item:	11.2

Source:	ZTE Corporation, China Unicom, China Telecom, Ericsson, Nokia, Huawei, Ofinno, Samsung, Deutsche Telekom, CMCC, Lenovo, Qualcomm, NEC

Title:		[TP to BLCR 38.423] Support of AI/ML assisted Network Slicing

Document for:	Text Proposal

1	Introduction

This TP for the BLCR to TS 38.423 follows discussions about AI/ML network slicing to reflect the agreement:

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

2	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >> Slice Measurement Initiation Result | O |  | 9.2.3.x1 | List of measurement objects that failed to be initiated per slice. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

9.2.3.x1	Slice Measurement Initiation Result

This IE indicates the list of measurement objects that failed to be initiated per slice.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Measurement Initiation Result List |  | 1 |  |  |
| >Slice Measurement Initiation Result Item |  | 1..< maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI Measurement Initiation Result List |  | 1 |  |  |
| >>> S-NSSAI Measurement Initiation Result Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>> S-NSSAI Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the slice. |
| >>>>> S-NSSAI Measurement Failure Cause Item |  | 1 .. <maxFailedSliceMeasObjects> |  |  |
| >>>>> S-NSSAI Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node1. |
| >>>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN IDs broadcast in a cell. Value is 12. |
| maxFailedSliceMeasObjects | Maximum number of measurement objects that can fail per slice. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>