3GPP TSG-RAN WG3 Meeting #128	R3-253145

St Julian’s, Malta, May 19 - 23, 2025

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	(TP to BL CR for TS 38.455 and TS 38.305) Support of "Channel Measurement" for Case 3b and Stage 2 Support for Case 3a/3b

Document for:		Discussion and Decision

# 1.	Introduction

In this contribution, we discuss the RAN1 agreements related to the "Channel Measurement" for Case 3b, and propose corresponding changes to the NRPPa and Stage 2 (38.305) baseline CRs.

# 2.	Discussion of RAN1 Agreements on "Channel Measurement"

## 2.1	RAN1 Agreements on Case 3b

The list below summarizes the RAN1 agreements related to the "Channel Measurement" in Case 3b. Each agreement is associated with a number (#1 – #14) for reference in the following discussion.

#1 Agreement (RAN1#116)

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

#2 Agreement (RAN1#116)

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time.

- FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time

#3 Agreement (RAN1#116bis)

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215.

FFS: whether it is applicable when Case 3b is used to support multi-RTT

#4 Agreement  (RAN1#118)

For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

#5 Agreement  (RAN1#118)

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

#6 Agreement  (RAN1#118bis)

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

#7 Agreement (RAN1#119)

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

#8 Agreement (RAN1#120)

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting:

- k = {0...5}

#9 Agreement (RAN1#120)

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting:

- Nt' = {8, 16, 24}

#10 Agreement (RAN1#120)

For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B):

(A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b).

- Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly.

- The descriptive info of the channel measurement includes at least:

  - measurement type (A) or type (B) and

  - measurement parameters.

- Regarding measurement parameters,

  - For (A) path-based measurement, the measurement parameters include: number of additional paths, k.

  - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k.

    - FFS: Nt

- Note: Type (A) and Type (B) are used for discuss purpose only.

#11 Conclusion (RAN1#120)

For Rel-19 AI/ML based positioning Case 3b,

- If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type.

  - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested.

- It is up to RAN3 to decide the signaling details.

  - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

#12 Conclusion (RAN1#120)

- The offset (used in the agreement made in RAN1#119) refers to the duration (in samples) between the reference time and the starting time of the list of Nt consecutive values.

#13 Agreement (RAN1#120)

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by:

- No offset is transmitted from gNB to LMF

#14 Conclusion (RAN1#120)

For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.

## 2.2	Discussion

The Model Input for Case 3b is the time domain "Channel Measurement", which consists of: (#1)

-	Timing Information;

-	paired timing information and power information.

The "timing information" of the "Channel Measurement" is represented relative to a "reference time" (#2). This reference time is the existing UL RTOA reference time, as defined in TS 38.215 (#3).

FFS whether the UL RTOA reference time is also used to support multi-RTT (#3).

The "power information" of the "Channel Measurement" is the existing UL SRS-RSRPP (#4).

The "Channel Measurement" report includes the (#6):

-	"Channel Measurement" 						M

-	Quality of the "Channel Measurement"		O

-	Details are FFS (#6)

-	Time stamp of the "Channel Measurement"	O

According to RAN1, there are two types of "Channel Measurements":

(a)	"path-based measurements", which is referring to the existing specification (#7):

-	The existing NRPPa specification supports this in IEs Additional Path List (9.2.41) and Extended Additional Path List (9.2.74).

The difference between the two IEs is only in the number of additional paths supported (2 in Additional Path List, and 8 in Extended Additional Path List).

-	RAN1 refers to this as "Channel Measurement "Type A" (#10)

-	In existing specifications "Type A" can be reported along with UL RTOA Measurement (9.2.39) and gNB Rx-Tx Time Difference (9.2.40).

-	A LMF can request the (existing) "Channel Measurement Type A" via the IE Measurement Characteristics Request Indicator (9.2.81).

-	No enhancements are needed for the number of additional paths (i.e., can remain up to 8) (#14)

(b)	"sample-based measurements", which is an enhancement to the "path-based measurement" (#7):

-	The channel response consists of Nt consecutive channel response samples, with timing granularity T

-	The timing granularity is T = 2k × Tc (#7), which is the same as in NRPPa IEs Additional Path List and Extended Additional Path List.

-	The starting point of the Nt consecutive channel response samples is the 1st detected path (rounded down to T)

-	From the Nt consecutive channel response samples, the "Channel Measurement" is reported as follows (#7):

-	The measurement consists of Nt' values, selected from the Nt samples

-	The selected Nt' values are reported with timing granularity T

-	The selected Nt' values are those with the highest power

-	The power (if reported) consists of the power of each Nt' sample

-	The timing information is defined relative to the same reference time as the path-based measurement ("Type A") (#7).

-	LMF can signal the values (#7, #8, #9):

-	Nt' (8, 16, 24), Nt (32, 64, 128), k (0…5)

-	RAN1 refers to this as "Channel Measurement Type B" (#10)

Observation 1:	The difference between "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) is on how the reported values are selected by the TRP: 
-	For "Type A", the (up to 8) paths are selected as the (up to 8) detected strongest path timings.
-	For "Type B", the (up to 24) samples are selected as the (up to 24) detected strongest path samples within a specified window.

Observation 2:	For both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) the detected paths/samples are reported with timing granularity T.

Observation 3:	For both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) the timing information is represented relative to the UL RTOA reference time.

Observation 4:	For both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) the power of the reported path/sample values can be expressed via existing UL SRS-RSRPP measurement.

Based on the above discussion and observations, there is no need to add a new TRP Measurement Type to the NRPPa Measurement Request message. The additional reporting information for "Channel Measurement Type B" can be handled in the same way as for the existing "Channel Measurement Type A" (path-based) via the (existing) NRPPa Measurement Characteristics Request Indicator. This would allow reporting the enhanced "Channel Measurement Type B" together with any timing measurement (e.g., RTOA, gNB-RxTxTimeDiff (which is still FFS in RAN1)), and would also allow to report both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) simultaneously, if desired.

Proposal 1:	Support for "Sample-based Channel Measurements" can be achieved via the existing TRP Measurement Type for 'UL-RTOA' (and possibly gNB-RxTxTimeDiff (which is still FFS in RAN1)) and the existing 'Measurement Characteristics Request Indicator'.

A corresponding TP is provided in the Annex A.

Proposal 2:	 Endorse the TP in the Annex A of this contribution.

# 3.	Discussion of Stage 2 (38.305) Impacts for Case 3a and Case 3b

## 3.1	Case 3a

For Case 3a, the TRPs report the existing timing measurements (UL RTOA and gNB Rx-Tx time difference) and the LOS/NLOS indicator:

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, - Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

Therefore, Case 3a could be integrated into the Multi-RTT and UL-TDOA descriptions to clarify that the UL RTOA, gNB Rx-Tx time difference, and LOS/NLOS indicator may also be predicted by using a trained machine learning model hosted by the NG-RAN node.

Proposal 1:	Integrate Case 3a into the UL-TDOA and Multi-RTT descriptions in Stage 2 TS 38.305.

## 3.2	Case 3b

For Case 3b, the additional TRP reporting information includes the sample-based channel measurement, as discussed in section 2 above. If the sample-based channel measurement is reported (at least) along with the UL-RTOA measurements, Case 3b could also be integrated into the UL-TDOA descriptions in Stage 2 TS 38.305. Whether the sample-based channel measurement is also applicable to Multi-RTT seems unclear at the moment, e.g., per RAN1 agreement:

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT

Proposal 2:	Integrate Case 3b (at least) in UL-TDOA description in Stage 2 TS 38.305.

In addition, small updates to the gNB and LMF functional descriptions would be required to clarify that machine learning models may be used at these nodes to predict measurements or locations. Some small clarifications to the Positioning Data Collection Procedure description are also proposed.

Proposal 3: 	Endorse the TP in the Annex B of this contribution.

# 4.	Summary

In this contribution, we discussed the RAN1 agreements related to the "Channel Measurement" for Case 3b and proposed corresponding changes to the NRPPa and Stage 2 (38.305) baseline CRs.

NRPPa updates for Case 3b:

Observation 1:	The difference between "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) is on how the reported values are selected by the TRP: 
-	For "Type A", the (up to 8) paths are selected as the (up to 8) detected strongest path timings.
-	For "Type B", the (up to 24) samples are selected as the (up to 24) detected strongest path samples within a specified window.

Observation 2:	For both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) the detected paths/samples are reported with timing granularity T.

Observation 3:	For both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) the timing information is represented relative to the UL RTOA reference time.

Observation 4:	For both, "Channel Measurement Type A" (path-based) and "Channel Measurement Type B" (sample-based) the power of the reported path/sample values can be expressed via existing UL SRS-RSRPP measurement.

Proposal 1:	Support for "Sample-based Channel Measurements" can be achieved via the existing TRP Measurement Type for 'UL-RTOA' (and possibly gNB-RxTxTimeDiff (which is still FFS in RAN1)) and the existing 'Measurement Characteristics Request Indicator'.

Proposal 2:	Endorse the TP in the Annex A of this contribution.

Stage 2 (38.305) updates for Case 3a/3b:

Proposal 1:	Integrate Case 3a into the UL-TDOA and Multi-RTT descriptions in Stage 2 TS 38.305.

Proposal 2:	Integrate Case 3b (at least) in UL-TDOA description in Stage 2 TS 38.305.

Proposal 3: 	Endorse the TP in the Annex B of this contribution.

# References

[1]	R3-252501, "(BL CR to 38.455) Introduction of AI/ML air", Ericsson

[2]		R3-252500, "(BL CR to 38.305) Introduction of AI/ML air", CATT et al.

# Annex A: TP to BL CR to 38.455 Introduction of AI/ML air

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the Channel Response Window Size IE and the Channel Response Number IE are included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN may take it into account when configuring measurements including Sampled Additional Path List.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Channel Response Window Size | O |  | ENUMERATED (32, 64, 128, …) | Represents the window size over which the channel response samples are requested to be selected. If this IE is included, the IE Channel Response Number is also included. | Yes | Ignore |
| >>Channel Response Number | O |  | ENUMERATED (8, 16, 24, …) | Represents the number of channel response samples requested within the Channel Response Window Size. If this IE is included, the IE Channel Response Window Size is also included. | Yes | Ignore |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Editor's Note: Bit 3 can be applicable to all 3 Additional Path lists. Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |  |  |
| >k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |  |  |
| >k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |  |  |
| >k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |  |  |
| >k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |  |  |
| >k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |  |  |
| >kminus1 |  |  | INTEGER (0..3940097) | TS 38.133 [16] | YES | ignore |
| >kminus2 |  |  | INTEGER (0..7880193) | TS 38.133 [16] | YES | ignore |
| >kminus3 |  |  | INTEGER (0.. 15760385) | TS 38.133 [16] | YES | ignore |
| >kminus4 |  |  | INTEGER (0.. 31520769) | TS 38.133 [16] | YES | ignore |
| >kminus5 |  |  | INTEGER (0.. 63041537) | TS 38.133 [16] | YES | ignore |
| >kminus6 |  |  | INTEGER (0.. 126083073) | TS 38.133 [16] | YES | ignore |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.2.85 |  | YES | ignore |
| Sampled Additional Path List | O |  | 9.2.x |  | YES | ignore |

### 9.2.x	Sampled Additional Path List

This IE contains the sampled additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |  |
| --- | --- | --- | --- | --- | --- |
| Sampled Additional Path Item |  | 1..<maxNoSamples> |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |  |
| >>k0 |  |  | INTEGER(0..16351) | TS 38.133 [16] |  |
| >>k1 |  |  | INTEGER(0..8176) | TS 38.133 [16] |  |
| >>k2 |  |  | INTEGER(0..4088) | TS 38.133 [16] |  |
| >>k3 |  |  | INTEGER(0..2044) | TS 38.133 [16] |  |
| >>k4 |  |  | INTEGER(0..1022) | TS 38.133 [16] |  |
| >>k5 |  |  | INTEGER(0..511) | TS 38.133 [16] |  |
| >Path Quality | O |  | Measurement Quality (FFS) 9.2.43 |  |  |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  |  |

| Range bound | Explanation |
| --- | --- |
| maxNoSamples | Maximum no. of selected path samples. Value is 24. |

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-SCS-SpecificCarrier,

id-MeasBasedOnAggregatedResources,

id-UEReportingInterval-milliseconds,

id-ChannelResponseWindowSize,

id-ChannelResponseNumber,

id-SampledAdditionalPathList

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

CGI-NR ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nRcellIdentifier			NRCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-NR-ExtIEs} } OPTIONAL,

...

}

CGI-NR-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ChannelResponseWindowSize ::= ENUMERATED { 32, 64, 128, ... }

ChannelResponseNumber ::= ENUMERATED { 8, 16, 24, ... }

RelativePathDelay ::= CHOICE {

k0			INTEGER(0..16351),

k1			INTEGER(0..8176),

k2			INTEGER(0..4088),

k3			INTEGER(0..2044),

k4			INTEGER(0..1022),

k5			INTEGER(0..511),

choice-Extension		ProtocolIE-Single-Container { { RelativePathDelay-ExtIEs} }

}

RelativePathDelay-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ID id-ReportingGranularitykminus1AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus1AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus2AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus2AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus3AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus3AdditionalPath PRESENCE mandatory}|
	{ID id-ReportingGranularitykminus4AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus4AdditionalPath PRESENCE mandatory}|
	{ID id-ReportingGranularitykminus5AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus5AdditionalPath PRESENCE mandatory}|
	{ID id-ReportingGranularitykminus6AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus6AdditionalPath PRESENCE mandatory},

...

}

RelativePathDelay2 ::= CHOICE {

k0			INTEGER(0..16351),

k1			INTEGER(0..8176),

k2			INTEGER(0..4088),

k3			INTEGER(0..2044),

k4			INTEGER(0..1022),

k5			INTEGER(0..511),

choice-Extension		ProtocolIE-Single-Container { { RelativePathDelay2-ExtIEs} }

}

RelativePathDelay2-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SampledAdditionalPathList ::= SEQUENCE (SIZE (1..maxNoSamples)) OF SampledAdditionalPathList-Item

SampledAdditionalPathList-Item ::= SEQUENCE {

relativeTimeOfPath			RelativePathDelay2,

pathQuality					TrpMeasurementQuality		OPTIONAL, -- FFS

pathPower					UL-SRS-RSRPP				OPTIONAL,

ChannelResponseWindowSize	ChannelResponseWindowSize	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAdditionalPathList-Item-ExtIEs} } OPTIONAL,

...

}

ExtendedAdditionalPathList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended 	PRESENCE optional}|

{ID id-ChannelResponseWindowSize				CRITICALITY ignore EXTENSION ChannelResponseWindowSize 						PRESENCE optional}|

{ID id-ChannelResponseNumber					CRITICALITY ignore EXTENSION ChannelResponseNumber 								PRESENCE optional},

...

}

TRPMeasurementType::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

...,

multiple-UL-AoA,

uL-SRS-RSRPP,

ul-RSCP

}

UL-RTOAMeasurement ::= SEQUENCE {

uLRTOAmeas			ULRTOAMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { UL-RTOAMeasurement-ExtIEs } }	OPTIONAL,

...

}

UL-RTOAMeasurement-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRP-Rx-TEGInformation		CRITICALITY ignore EXTENSION TRP-Rx-TEGInformation		PRESENCE optional}|

{ ID id-SampledAdditionalPathList	CRITICALITY ignore EXTENSION SampledAdditionalPathList	PRESENCE optional},

...

}

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-PointA														ProtocolIE-ID ::= 154

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-ChannelResponseWindowSize									ProtocolIE-ID ::= XX1

id-ChannelResponseNumber										ProtocolIE-ID ::= XX2

id-SampledAdditionalPathList									ProtocolIE-ID ::= XX3

-- **************************************************************

--

-- Lists

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

maxnoAggCombinations						INTEGER ::= 2

maxnoAggregatedPosSRSCombinations			INTEGER ::= 32

maxNoSamples								INTEGER ::= 24

# Annex B: TP to BL CR to 38.305 Introduction of AI/ML air

## 5.4	Functional Description of Elements Related to UE Positioning in NG-RAN

[…]

### 5.4.2	gNB

The gNB is a network element of NG-RAN that may provide measurement information for a target UE and communicates this information to an LMF.

To support NR RAT-Dependent positioning, the gNB may make measurements of radio signals for a target UE, and provide measurement results for position estimation. A gNB may serve several TRPs, including for example remote radio heads, and UL-SRS only RPs and DL-PRS-only TPs. For NTN, a TRP may be located on board the satellite.

A gNB may broadcast assistance data information, received from an LMF, in positioning System Information messages.

A gNB may host machine learning models to predict the measurements from radio signals transmitted by a target UE. The machine learning model that is used for measurement prediction by the gNB may have been trained by the gNB.

### 5.4.3	ng-eNB

The ng-eNB is a network element of NG-RAN that may provide measurement results for position estimation and makes measurements of radio signals for a target UE and communicates these measurements to an LMF.

The ng-eNB makes its measurements in response to requests from the LMF (on demand or periodically).

An ng-eNB may serve several TPs, including for example remote radio heads and PRS-only TPs for PRS-based TBS positioning for E-UTRA.

An ng-eNB may broadcast assistance data information, received from an LMF, in positioning System Information messages.

### 5.4.4	Location Management Function (LMF)

The LMF manages the support of different location services for target UEs, including positioning of UEs and delivery of assistance data to UEs. The LMF may interact with the serving gNB or serving ng-eNB for a target UE in order to obtain position measurements for the UE, including uplink measurements made by an NG-RAN and downlink measurements made by the UE that were provided to an NG-RAN as part of other functions such as for support of handover.

The LMF may interact with a target UE in order to deliver assistance data if requested for a particular location service, or to obtain a location estimate if that was requested.

The LMF may interact with multiple NG-RAN nodes to provide assistance data information for broadcasting. The assistance data information for broadcast may optionally be segmented and/or ciphered by the LMF. The LMF may also interact with AMFs to provide ciphering key data information to the AMF as described in greater detail in TS 23.273 [35].

For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.

The LMF may interact with the AMF to provide (updated) UE Positioning Capability to AMF and to receive stored UE Positioning Capability from AMF as described in TS 23.273 [35].

For NTN, the LMF is configured by the OAM with satellite related information (described in TS 38.300 [52]), as well as the association between TRP(s) and satellite(s), the association between gNB and TRP(s).

A LMF may host machine learning models to predict the target UE location from measurement information received from gNBs. The machine learning model that is used for UE location prediction by the LMF may have been trained by the LMF.

### 5.4.5	Positioning Reference Unit (PRU)

A Positioning Reference Unit (PRU) at a known location can perform positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, DL-RSCPD, DL-RSCP, etc.) and report these measurements to a location server. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, UL-RSCP, etc.) from PRU at a known location. The PRU measurements can be compared by a location server with the measurements expected at the known PRU location to determine correction terms for other nearby target devices. The DL- and/or UL location measurements for other target devices can then be corrected based on the previously determined correction terms.

PRU measurements may also be provided to the target device in the assistance data as described in clause 8.12.

From a location server perspective, the PRU functionality is realized by a UE with known location.

## 6.3	NG-RAN Node terminated protocols

### 6.3.1	NR Positioning Protocol A (NRPPa)

The NR Positioning Protocol A (NRPPa) carries information between the NG-RAN Node and the LMF. It is used to support the following positioning functions:

-	E-CID for E-UTRA where measurements are transferred from the ng-eNB to the LMF.

-	Data collection from ng-eNB's and gNB's for support of OTDOA positioning for E-UTRA.

-	Cell-ID and Cell Portion ID retrieval from gNB's for support of NR Cell ID positioning method.

-	Exchange of information between LMF and NG-RAN node for the purpose of assistance data broadcasting.

-	NR E-CID where measurements are transferred from the gNB to the LMF.

-	NR Multi-RTT where measurements are transferred from the gNB to the LMF.

-	NR UL-AoA where measurements are transferred from the gNB to the LMF.

-	NR UL-TDOA where measurements are transferred from the gNB to the LMF.

-	Data collection from gNBs for support of DL-TDOA, DL-AoD, Multi-RTT, UL-TDOA, UL-AoA.

-	Measurement Preconfiguration Information Transfer which allows the LMF to request the NG-RAN node to pre-configure and activate/deactivate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer which allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Transfer of data collection information from LMF to gNB which assists the gNB to train machine learning models.

The NRPPa protocol is transparent to the AMF. The AMF routes the NRPPa PDUs transparently based on a Routing ID corresponding to the involved LMF over NG-C interface without knowledge of the involved NRPPa transaction. It carries the NRPPa PDUs over NG-C interface either in UE associated mode or non-UE associated mode.

In case of a split gNB architecture, the NRPPa protocol is terminated at the gNB-CU.

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

4.	The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicate that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

## 8.10	Multi-RTT positioning

### 8.10.1	General

In the Multi-RTT positioning method, the UE position is estimated based on measurements performed at both, UE and TRPs. The measurements performed at the UE and TRPs are UE/gNB Rx-Tx time difference measurements (and optionally DL-PRS-RSRP, DL-PRS-RSRPP, UL-SRS-RSRP, UL-SRS-RSRPP, and/or DL-RSCP/UL-RSCP) of DL-PRS and UL-SRS, which are used by an LMF to determine the RTTs.

The gNB Rx-Tx time difference measurements may also be predicted by using a trained machine learning model hosted by the NG-RAN node.

For network verification of UE location in NTN, the measurements can be performed at a single TRP at different time instances. The additional measurements performed at UE are the UE Rx – Tx time difference subframe offset in unit of subframe and the DL timing drift due to Doppler in service link between UE and satellite as defined in TS 38.215 [37].

The UE may require measurement gaps to perform the Multi-RTT measurements from NR TRPs. The UE may request measurement gaps from a gNB using the procedure described in clause 7.4.1.1. The UE may also request to activate pre-configured measurement gaps as described in clause 7.7.2.

NOTE: Multi-RTT positioning with aperiodic or semi-persistent SRS is not supported for a U2N Remote UE.

[…]

#### 8.10.2.3	Information that may be transferred from the gNB to LMF

[…]

The measurement results that may be signalled from gNBs to the LMF is listed in Table 8.10.2.3-3.

Table 8.10.2.3-3: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

gNB Rx-Tx time difference measurementNOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information of the measurement

LoS/NLoS information for each measurementNOTE 2

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be predicted by using a trained machine learning model hosted by the NG-RAN node,

Editor's Note: FFS whether time domain channel measurements are also applicable to Multi-RTT.

[…]

### 8.10.3	Multi-RTT Positioning Procedures

#### 8.10.3.0	General

The procedures described in this clause support Multi-RTT positioning measurements obtained by the UE and TRPs/gNB.

The procedures for data collection to train a machine learning model at the NG-RAN node for Multi-RTT positioning measurement predictions are described in Clause 7.x.

[…]

## 8.13	UL-TDOA positioning

### 8.13.1	General

In the UL-TDOA positioning method, the UE position is estimated based on UL-RTOA (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP and/or UL-RSCP) measurements taken at different TRPs of uplink radio signals from UE, along with other configuration information.

The UL-RTOA measurements taken at different TRPs of uplink radio signals from UE may also be predicted by using a trained machine learning model hosted by the NG-RAN node.

The LMF may also use a trained machine learning model hosted by the LMF to predict directly the UE location using the TRP measurements.

The specifics of any UL-TDOA positioning methods or techniques used to estimate the UE's location from these measurements are beyond the scope of this specification.

In order to obtain uplink measurements, the TRPs need to know the characteristics of the SRS signal transmitted by the UE for the time period required to perform uplink measurement. These characteristics should be static over the periodic transmission of SRS during the uplink measurements. Hence, the LMF will indicate to the serving gNB the need to direct the UE to transmit SRS signals for uplink positioning. It is up to the serving gNB to make the final decision on resources to be assigned and to communicate this SRS configuration information back to the LMF so that LMF can forward the SRS configuration to the TRPs. The gNB may decide (e.g., in case no resources are available) to configure no resources for the UE and report the empty resource configuration to the LMF.

NOTE: UL-TDOA positioning with aperiodic or semi-persistent SRS is not supported for a U2N Remote UE.

[…]

### 8.13.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and gNB/TRPs.

[…]

#### 8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOANOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

LoS/NLoS information for each measurementNOTE 2

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

Time domain channel measurements (additional path list or sampled additional path list)NOTE 3

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be predicted by using a trained machine learning model hosted by the NG-RAN node. NOTE 3: These measurements may also be used by an LMF to directly predict the UE location using a trained machine learning model hosted by the LMF.

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Bandwidth aggregation request indication

Positioning validity area cell list

Validity area specific SRS information

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity and amount

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information for measurements

Information on time window size and number of channel response samples for channel measurements

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

### 8.13.3	UL-TDOA Positioning Procedures

#### 8.13.3.0	General

The procedures described in this clause support UL-TDOA positioning measurements obtained by the gNB and provided to the LMF using NRPPa.

The procedures for data collection to train a machine learning model at the NG-RAN node for UL-TDOA measurement predictions are described in Clause 7.x.

The procedures for data collection to train a machine learning model at the LMF for direct UE location prediction are described in TS 23.273 [35].

[…]