3GPP TSG-RAN3 Meeting #128	R3-253265

Malta, MT, May 19-23, 2025

Agenda Item:	14.3

Source: 		Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Xiaomi, China Telecom, LG Electronics

Title: 	(TP for TS 38.300) Discussion on Hard FLSO in NR NTN Regenerative Payload

Document for:	Discussion/Decision

# Background

In the last RAN3 meeting we had good progress on most of the open issues in NTN Regenerative Payload. In this paper we provide our inputs on remaining open issue on hard feeder link switch over from last meeting.

RAN3#127bis Support RRC_INACTIVE UE in NTN by implementation in R19. No enhancements on location based CHO for NTN in R19. In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. No new NGAP suspend/resume procedures in R19. NG Connection interruption due to hard feeder link switch over Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

# Discussion

## Hard Feeder Link Switch Over

In the last RAN3#127bis meeting a new IE “Downlink NG Transmission Control” was added to NGAP RAN Configuration Update message from gNB to AMF to indicate to the AMF, to stop and start DL transmission due to hard feeder link switch over in a stage 3 TP 252409 to TS 38.413. Corresponding Stage 2 text to TS 38.300 was endorsed in R3-252454. Some companies needed more time to agree to the solution and hence an Editor’s note “Editor’s Note: FFS whether this IE/text is needed.” was added to both Stage 2 and Stage 3.

TS 38.300 TP R3-252454 16.14.4.3 Procedures The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function. For Regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the NGAP Configuration Update message when the Hard Feeder Link switch occurs. Editor’s Note: FFS whether this text is needed.

As stated in our earlier contributions time synchronised stop and start of DL transmission is needed in the Application layer between both gNB and the AMF during hard feeder link switch over which cannot be handled via OAM. The interruption of NG due to hard FLSO needs to be handled in NG application layer as this needs handshaking between peers at both the end of NG connection to stop and start transmission synchronously. Hence, the current NGAP signalling solution to indicate over NGAP to stop and start DL transmission during hard FLSO is required. We propose to remove the editor’s notes in the stage 2 and stage 3 which has FFS on the text and IE.

Time synchronised start and stop of DL transmission over NGAP between gNB and AMF due to Hard Feeder Link Switch Over is needed at NGAP application layer which cannot be handled via OAM.

Remove the Editor’s Note with “FFS whether this text is needed” in stage 2 TS 38.300 and Editor’s Note in stage 3 TS 38.413 “FFS whether this IE is needed. Corresponding Stage 2 TP to TS 38.300 is provided below. Stage 3 TP can be found in R3-253392

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Time synchronised start and stop of DL transmission over NGAP between gNB and AMF due to Hard Feeder Link Switch Over is needed at NGAP application layer which cannot be handled via OAM.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Remove the Editor’s Note with “FFS whether this text is needed” in stage 2 TS 38.300 and Editor’s Note in stage 3 TS 38.413 “FFS whether this IE is needed. Corresponding Stage 2 TP to TS 38.300 is provided below. Stage 3 TP can be found in R3-253392

# TP for TS 38.300

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

For Regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the NGAP Configuration Update message when the Hard Feeder Link switch occurs.