3GPP TSG-RAN WG3 Meeting #128	R3-253313

Malta, MT, May 19 – 23, 2025

Agenda item:	 10.3.1

Source:		Lenovo

Title:		Discussion on SON/MDT for NTN

Document for:		Discussion and Decision

1	Introduction

RAN3#125 meeting agreed that:

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

RAN3#125bis meeting agreed that:

For location based trigger condition:

Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell.

Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For earth fixed case, there is no need to report the measured distance from the UE.

For joint trigger condition:

Agree on the following definition of the use cases and agreement:

- Case 1: Too Late CHO Execution: RLF happens in the source cell. The UE re-establishes in the cell other than the source cell.

- Case 2: Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

RAN3#126 meeting agreed that:

For single location based trigger condition:

WA: There is no need to report any other info in the RLF report for the location based CHO.

For multiple trigger conditions:

No consensus on further enhancement.

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

RAN3#127 meeting agreed that:

Time based CHO:

Existing timers can be reused to identify when RLF happens with 100ms accuracy.

No need to identify whether RLF happens before or after T1 for time-based CHO.

Location based CHO:

Revert the WA: There is no need to report any other info in the RLF report for the location based CHO.

For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged.

RAN3#127bis meeting further agreed that:

No further discussion on Time-based CHO.

MRO for RACH-less Handover for NTN:

Divide the MRO for RACH-less handover case into two scenarios:

- RACH-less handover was attempted but failed;

- UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

MDT enhancement for NTN:

Distance-based event trigger for logged MDT?

In this paper, we would further discuss SON/MDT for NTN.

2	Discussion

MRO for RACH-less Handover for NTN:

Last RAN3#127bis meeting agreed to divide the MRO for RACH-less handover case into two scenarios:

- Case 1: RACH-less handover was attempted but failed;

- Case 2: UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

For Case 1, it may happen due to low RSRP threshold for RACH-less HO or inappropriate HO related configuration (e.g. target cell is wrong, or RACH-less HO related configuration is not proper), since RSRP threshold for RACH-less HO is configured by network, it can know the RSRP threshold as well as whether SSB measurement is higher than the RSRP threshold or not, based on its stored/retrieved UE context and existing information (e.g. cell/SSB measurement results, last serving cell ID, failed cell ID and etc) included in RLF report. Further, network can determine the failure cause and do corresponding MRO.

For Case 2, UE did not attempt the configured RACH-less HO because measured RSRP is below RSRP threshold, similar as Case 1, network can know the RSRP threshold and whether SSB measurement is lower than the RSRP threshold.

Therefore, there is no need for the UE to report the RSRP threshold for RACH-less HO in RLF report.

Proposal 1: No need to include the RSRP threshold for RACH-less HO in RLF report.

When RACH-less handover was attempted but failed, fallback from RACH-less HO to RACH-based HO could be performed. If RA-InformationCommon is included in RLF report, it means that fallback from RACH-less HO to RACH-based HO is performed, then network may optimize RACH-less HO related configuration. So, an explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in RLF report.

Proposal 2: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in RLF report.

MDT enhancement for NTN:

For the FFS on whether to introduce distance-based event trigger for logged MDT, the intention of distance-based event trigger is to avoid logging DL pilot strength measurements in the NTN cell. Currently, event L1 is supported for measurement based logged MDT, in which the UE performs logging of MDT measurements when the triggering condition (e.g. serving cell quality is lower than the configured threshold as configured in the event L1) is met, measurement based logged MDT can be reused for NTN. Additionally, introducing distance-based event trigger for logged MDT in NTN would cause complexity and RAN2 WG should also be involved (e.g. RAN2 needs to discuss UE capability, network configuration, UE logging behaviour, and etc), considering it is late stage in R19 and only two meetings left with limited TU especially in RAN2, it is unnecessary to introduce distance-based event trigger for logged MDT in NTN.

Proposal 3: No need to introduce distance-based event trigger for logged MDT in NTN.

For the issue about whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, last RAN2 meeting discussed and replied the LS in [1] that “RAN2 confirms that the geographical area defined for MBS NTN can be used as a reference for Area Scope of logged MDT over Uu for NTN, including both reference location/radius and polygon-based area indication. RAN2 also agreed that only geographical area scope is used to indicate applicable logging area to the UE over Uu, i.e., no mapped cell ID(s)”. Based on RAN2’s agreements, RAN3 should introduce the geographical area (e.g. including both reference location/radius and polygon-based area indication) as the Area Scope for the logged MDT for NTN over NGAP.

Proposal 4: Introduce geographical area (e.g. including both reference location/radius and polygon-based area indication) as the Area Scope for the logged MDT for NTN over NGAP.

3	Conclusion

In this paper, SON/MDT enhancements for NTN are discussed. We have the following proposals:

Proposal 1: No need to include the RSRP threshold for RACH-less HO in RLF report.

Proposal 2: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in RLF report.

Proposal 3: No need to introduce distance-based event trigger for logged MDT in NTN.

Proposal 4: Introduce geographical area (e.g. including both reference location/radius and polygon-based area indication) as the Area Scope for the logged MDT for NTN over NGAP.

Reference

[1] R2-2503196, Relay LS on MDT for NTN, Qualcomm