3GPP TSG-RAN WG3 Meeting #128	             R3-253670

St Julian, Malta, 19-23 May 2025

Agenda Item:	21.3

Source:	CMCC

Title:	Support of Uplink Congestion Signaling

Document for:	Discussions & Approval

# Introduction

In the previous RAN3 127Bis meeting [1], the following agreements regarding support of uplink congestion signalling and rate control have been reached:

For Specify uplink congestion signalling: Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control. Introduce the same indication in XnAP to inform target gNB. FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows. For avoid RLC unnecessary retransmission FFS on whether the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

# Discussions

## Support of Uplink Congestion Signalling

In addition to the agreed “Indication of Bitrate Adaptation” to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control, the CN may provide the gBN with a recommended uplink (UL) bit rate for specific QoS flows. This allows the gNB to adjust uplink rate control to match the CN recommendations as well as according to its available resources.

The CN could inform gNB the recommended UL rate for specific QoS flows.

In the current 3GPP architecture, the gNB-DU operates at the DRB level and the gNB-CU is responsible for mapping QoS flows to DRBs and providing the gNB-DU with DRB-level QoS parameters. However, there are scenarios where it may be beneficial for the gNB-DU to have more granular control over QoS flows, such as deriving recommended bit rates for individual QoS flows based on available resources and QoS requirements.

The F1AP protocol already supports the transfer of QoS flow-level QoS parameters (e.g., 5QI, GFBR, MFBR) from the gNB-CU to the gNB-DU during the UE Context Setup or UE Context Modification procedures. This information could potentially be leveraged to enable the gNB-DU to make more informed decisions about resource allocation and scheduling.

The gNB-CU can configure the gNB-DU to enforce uplink bit rate limits at the DRB level. If a DRB carries only one QoS flow, this effectively controls the uplink bit rate at the QoS flow level.

In CU-DU split architecture, the gNB-CU can configure the gNB-DU to enforce uplink bit rate control at the DRB level.

The gNB-DU sends ﻿Recommended bit rate MAC CE to UE for the DRB.

According to TS 38.323 clause 5.8 [2], it states that the SDAP header will not be ciphered. Thus, the gNB-DU is able to utilize the QFI carried in SDAP header to differentiate different QoS flows mapped to the same DRB.

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

The gNB-DU could have the information of the QoS flows that are mapped to a specific DRB, thus the gNB-DU is capable of reconfiguring the uplink data rate for each QoS flow mapped to the DRB based on the available uplink resource, congestion status of the DRB, as well as the QoS requirements contained in the ﻿QoS Flow Level QoS Parameters. If the gNB-DU perceives/estimates the current data rate of each QoS flow by leveraging the QFI contained in the SDAP header, the uplink data rate reconfiguration by the gNB-DU would be feasible at the QoS flow level.

The QFI carried in the SDAP header can be considered visible to the gNB-DU, which can be utilized by the gNB-DU for uplink data rate reconfiguration at the QoS flow level.

With the above proposal being made, if we stick to the functional and protocol stack split of gNB-CU and gNB-DU, QFI is likely not be leveraged by the gNB-DU. In the following, we discuss the solutions to provide QoS flow level in CU-DU split architecture if we assume the gNB-DU would not utilize the QFI contained in the SDAP header.

Solution I:

Since the PDCP layer is handled by the gNB-CU in the 3GPP functional split, the gNB-CU can enforce QoS flow-level bit rate control directly. The gNB-CU can request the gNB-DU to provide updates on:

- The available uplink resources for a specific DRB.

- The congestion status of the DRB (e.g., whether the DRB is experiencing high load or latency).

This feedback can be sent from the gNB-DU to the gNB-CU over the F1. The gNB-CU can use to reconfigure the uplink data rate for each QoS flow mapped to the DRB. The gNB-CU could use RRC to notify the UE regarding the uplink rate control for the QoS flows if the update is not frequent. The gNB-CU could also send the uplink rate control for the QoS flows to the gNB-DU, which forward the information to UE through MAC CE.

The gNB-DU sends ﻿available uplink resource, congestion status of a specific DRB to the gNB-CU, the gNB-CU reconfigures the uplink data rate for each QoS flow based on the information.  ﻿

The gNB-CU could send the recommended uplink data rate for a QoS Flow by RRC to the UE.

The gNB-CU could send the recommended uplink data rate for a QoS Flow to the gNB-DU, which notify the UE through MAC CE.

Solution II:

The gNB-DU sends the Recommended bit rate MAC CE to UE for the DRB. The gNB-CU notifies the UE which QoS flows are subject to uplink rate control by RRC, how to update uplink rate for those QoS flows is up to the UE.

The gNB-CU informs the UE via RRC which QoS flows are subject to uplink rate control. The UE decides how to adjust the uplink rate for the affected QoS flows.

## Avoiding Unnecessary RLC Retransmission

In RAN2#129 meeting, RAN2 made the following agreement on a combined approach to avoid unnecessary RLC retransmission:

Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

The PDCP entity resides in the gNB-CU, while the transmitting RLC entity is located in the gNB-DU. To align with RAN2’s agreement, coordination between the gNB-CU and gNB-DU is required. Specifically, when a PDCP SDU is discarded due to timer expiration, the gNB-CU shall notify the gNB-DU with a discard indication. This indication could notify the gNB-DU to halt both any ongoing or scheduled transmission and retransmission of the corresponding RLC SDU or its segments. This mechanism ensures efficient resource usage by preventing unnecessary transmissions of data that has already been discarded at the PDCP layer.

To avoid unnecessary RLC retransmissions, the gNB-CU should send an indication to the gNB-DU instructing it to stop both transmission and retransmission of a specific RLC SDU or its segments.

# Conclusion

In this submission, we discussed the support of uplink congestion signalling and rate control. The following proposals have been made:

- The CN could inform gNB the recommended UL rate for specific QoS flows.

In CU-DU split architecture, the gNB-CU can configure the gNB-DU to enforce uplink bit rate control at the DRB level.

The gNB-DU sends ﻿Recommended bit rate MAC CE to UE for the DRB.

The QFI carried in the SDAP header can be considered visible to the gNB-DU, which can be utilized by the gNB-DU for uplink data rate reconfiguration at the QoS flow level.

The gNB-DU sends ﻿available uplink resource, congestion status of a specific DRB to the gNB-CU, the gNB-CU reconfigures the uplink data rate for each QoS flow based on the information.  ﻿

The gNB-CU could send the recommended uplink data rate for a QoS Flow by RRC to the UE.

The gNB-CU could send the recommended uplink data rate for a QoS Flow to the gNB-DU, which notify the UE through MAC CE.

The gNB-CU informs the UE via RRC which QoS flows are subject to uplink rate control. The UE decides how to adjust the uplink rate for the affected QoS flows.

To avoid unnecessary RLC retransmissions, the gNB-CU should send an indication to the gNB-DU instructing it to stop both transmission and retransmission of a specific RLC SDU or its segments.

# References

- Draft_MeetingReport_RAN3#127-bis_v20250419