3GPP TSG-RAN WG3 Meeting #128	R3-253788

Malta, MT, 19 – 23 May, 2025

Title:	Summary of Offline Discussion on intra-CU Conditional LTM

Agenda Item:	10.3.1

Source:	Huawei (moderator)

Document for:	Approval

# Introduction

CB: # MobilityEhn_CLTM

- offline discussion focus essential functions.

- capture the agreements for stage 2 and stage3 TPs.

- list the open issues for next meeting.

(Moderator - HW)

# For the Chairman’s Notes

Propose the following:

R3-25xxxa, R3-25xxxc merged

R3-25xxxc rev [in xxxg] – agreed

R3-25xxxd rev [in xxxh] – agreed

R3-25xxxe rev [in xxxi] – agreed

R3-25xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Issue 1: Simultaneously configuration of LTM and C-LTM:

RAN3#127bis meeting FFS whether a cell can be simultaneously configured for LTM and CLTM.

RAN2#129bis meeting Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

R3-253138 (Nokia), R3-253251(ZTE), R3-253470 (CATT), R3-253484 (Ericsson), R3-253656 (Samsung), R3-253661(LGE), R3-253601 (Huawei) propose to follow RAN2 decision.

Therefore, the moderator propose:

RAN3 confirms that the network can trigger a LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration.

Further discussion on the following 2 questions:

Q1: Can we configure a C-LTM on top of a already prepared LTM for a candidate cell?

Q2: Can we change a L1-LTM to L3-LTM and vice vasa for a given candidate cell?

## Issue 2: C-LTM indicator to the candidate DU

The candidate DU needs to be aware that the LTM preparation for a candidate cell is a conditional LTM for the following reasons:

- To inform the candidate DU to generate the C-LTM TAT which is going to configured to the UE and to the source DU

- To inform the candidate DU not to expect any cell switch notification from the CU (if the source DU does not trigger a CSC to a UE with C-LTM configured)

- To not mandatorily initiate the CSC to the UE when the candidate cell becomes a source cell in subsequence

- To wait for the CU to trigger the early RACH for those other candidate cells with L3 CLTM configured when the candidate cell becomes a source in subsequence LTM

The (initial) source DU also needs to be aware of the purpose of conditional LTM for the reasons of:

- To delivery the TA to the UE in a new MAC CE which is different from legacy LTM.

- To not mandatorily initiate the CSC to the UE

- To wait for the CU to trigger the early RACH for those candidate cells with L3 C-LTM configured.

- Option 1: one C-LTM indicator for both L1 measurement based C-LTM and L3 measurement based C-LTM

| LTM Information Setup/Modify |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, conditional,…) |  | - |  |

- Option 2: two separate C-LTM indicators for L1 measurement based C-LTM and L3 measurement based C-LTM

| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, L1-CLTM, L3-CLTM, …) |  | - |  |

Option 3: new IE with 2 indicators, L1 C-LTM, both L1 LTM and L1 C-LTM

| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >L1 C-LTM Indicator | O |  | ENUMERATED (L1 C-LTM, both L1 LTM and L1 C-LTM, …) |  |  |  |

Option 4: new IE in Early Sync Information Request with two indicators

| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| >>>Request for CLTM Information | O |  | ENUMERATED (L1 CLTM, L3 CLTM…) |  | - |  |

Option 5: 2 indicators, C-LTM only ,  LTM and C-LTM

| LTM Information Setup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM-only, LTM-and-C-LTM) |  | - |  |

Q0: Is the nature of the C-LTM indicator(s) is a configuration parameter of the candidate cell?

Q1: Do the L1 C-LTM and L3-CLTM need to be indicated separately to the candidate cell?

Q2: Is an indicator like both LTM and C-LTM needed?

Q3: New IE or to extend the legacy LTM Indicator IE?

## Issue 3: C-LTM indicator to the initial source DU

Furthermore, for the very first C-LTM execution, the initial source DU needs to be informed explicitly as well in case that no candidate cells are configured in the initial source DU.

| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| >Conditional LTM (option 1) |  |  | ENUMERATED (true,…) |  |  |  |
| Conditional LTM (option 2) |  |  | ENUMERATED (true,…) |  |  |  |

Q1: issue confirmed?

Q2:which option to agree?

## Issue 4: Request for L1 execution condition

As per RAN2 agreement, for a given source cell, the CU needs to request the (initial) source DU to generate the L1 execution condition for each indicated other candidate cell.

The related RAN2 agreement:

RAN2#129 meeting Agreements on C-LTM: 1. From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

Therefore, it can be concluded that the Request for L1 execution condition is a configuration of the source cell.

Option 1: add an indicator in the LTM Configuration ID Mapping List IE

#### 9.3.1.294	LTM Configuration ID Mapping List

This IE indicates the list of LTM cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >L1 Trigger Event Required | O |  | ENUMERATED (true, ...) |  | YES | ignore |

Option 2: a new IE in LTM Information Setup

| LTM Information Setup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM-only, LTM-and-C-LTM) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >L1 Execution Condition Request | O |  | 9.3.1.294 | Can be included only when the LTM Indicator IE is set to “C-LTM-only” or “LTM-and-C-LTM”. | - |  |

Q1: Is the nature of the Request for L1 execution condition is a configuration parameter of the source cell?

Q2: Which option is perferred?

## Issue 5: The definition of L1 Execution Condition Information IE

R3-253200 proposes that the L1 Execution Condition Information IE provided by the DU to CU, a list of cell and correspond to each cell the LTM-CSI-Report-Config-ID as OCTET STRING(includes the LTM-CSI-ReportConfigId-r18 as defined in subclause 6.3.3 in TS 38.331)

R3-253326 (lenovo) and R3-253661 (LGE) propse to refer to LTM-CSI-ReportConfigId-r18 IE in RRC.

R3-253484 (Ericsson) proposes to includes the ltm-ExecutionCondition RRC IE.

Discussion:

## TAT generation

R3-253138 (Nokia) proposes to follow Rel-18 LTM framework, the Source DU based on implementation determines whether an acquired TA value is valid.

R3-253601 (Huawei), R3-253326 (lenovo) and R3-253484 (Ericsson) R3-253562 (QC)propse to transfer the TAT from candidate to source via UE Context Setup/Modification procedure or TA Information Transfer procedure.

R3-253367 (China Telecom) and R3-253470 (CATT) proposes source gNB-DU to generate the TA timer and provide it to the CU via UE context Modification Response.

## CSI-RS based L1 measurement:

R3-253562 (QC) proposes to enhance the Rel-18 LTM CSI configuration procedure for support of CSI-RS based L1 measurement and reporting.

In detail:

- The CU requests a candidate gNB-DU and the source gNB-DU to provide the CSI-RS resources and configuration for the UE, for a suggested candidate cell, in UE Context Setup Request and UE Context Modification Request, respectively.

- A candidate gNB-DU and the source gNB-DU provide the CSI-RS resources and configuration for the UE for the prepared candidate cell, in UE Context Setup Response and UE Context Modification Response, respectively.

- The CU generates the common CSI configuration, i.e., the list of CSI-RS resources for all the LTM candidate cells and provides them to a candidate gNB-DU and the source gNB-DU, which then respond with the corresponding CSI reporting configurations for each LTM candidate cell.

Moderator proposal:

For the CSI-RS based L1 measurement in intra-CU conditional LTM, follow the agreements made for the inter-CU LTM on F1.

## L3 measurement based CLTM

R3-253138 (Nokia) proposes to introduce a new Class 2 F1AP procedure from the gNB-CU to the gNB-DU to trigger PDCCH-ordered early RACH procedure

R3-253326 (lenovo) and R3-253601 (Huawei): propse to reuse the Mobility Initiation procedure in rel-18.

R3-253251(ZTE) thinks that we need a F1AP message for this purpose.

R3-253367 (China Telecom) thinks that it is the CU to trigger the early RACH, FFS on the detailed F1AP message.

## C-LTM Completion

Inform the source DU

R3-253661 (LGE) :To inform the previous serving DU and stop data transmission after Inter-DU C-LTM execution happens, re-use the existing Transmission Action Indicator IE (used for S-CPAC) in UE CTXT MOD REQ message.

## C-LTM modification and cancelation

R3-253367 (China Telecom):

Proposal 5: The gNB-CU can use the UE Context Modification procedure to modify or release the prepared resources of candidate cells in the (candidate) gNB-DU and use the UE Context Release procedure to release the UE context in the (candidate) gNB-DU.

Proposal 6: The (candidate) gNB-DU can use the UE Context Modification Required procedure to request to cancel the prepared resources of a subset of candidate cells in it and use the UE Context Release Request procedure to request to release all candidate cells in it.

## TAT transfer

R3-253251(ZTE) proposes that the source gNB-DU transfers the TAT information such as the remaining duration of the TAT after cell switch execution to the target gNB-DU by UE CONTEXT MODIFICATION RESPONSE /REQUEST message after the successful cell change.

Moderator proposal:

Check RAN2 progress

# Conclusion, Recommendations [if needed]

If needed

# References