3GPP TSG RAN WG3 Meeting #128	R3-253010

Malta, MT, 19 - 23 May, 2025

3GPP TSG RAN WG2 Meeting #129bis              	                	                          R2-2503169

Wuhan, China, 7th – 11th Apr, 2025

Title:	Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R2-2500015 (R1-2410922)

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN2

To:	RAN1, RAN3, SA2, SA5

Cc:	SA3

Contact person:	Ziyi Li

liziyi5@xiaomi.com

Marco Belleschi

marco.belleschi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

RAN2 thanks RAN1 for the LS on signalling feasibility of dataset and parameter sharing. RAN2 discussed the candidate solutions and feasibility of standardized signaling for NW-side sharing dataset/model parameter to UE or UE-side training entity for below options:

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

Based on RAN1 LS, RAN2 studied the principles and approaches of NW-side sharing dataset/model parameter transfer to UE or UE-side training entity, and reached following agreements:

Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles:

- A1 - Size: From RAN2 point of view, RAN2 aims to define a unified solution (e.g. OTA, non-OTA, or a combined) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs);

- A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported;

- A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server);

- A4 - Latency: Relaxed latency requirement and infrequent update;

- A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server).

- A6: Proprietary information of the network and UE should be respected and not disclosed.

RAN2 concluded the following approaches for NW-side sharing dataset/model parameter transfer asked by RAN1:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1).  From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption. Furthermore, it does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

For OTA approaches, i.e., ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions:

- gNB -> UE via CP, where gNB is NW-side dataset/model parameter collection entity

- CN -> UE via gNB, where CN is NW-side dataset/model parameter collection entity

- OAM -> UE via gNB, where OAM is NW-side dataset/model parameter collection entity

RAN2 further have following conclusion on challenges and potential suitable scenarios for above OTA solutions (attached in Table 2), but RAN2 has no consensus on feasibility.

Table 2. OTA candidate solutions

| gNB -> UE via CP | Challenges | - UE is only required to support 45kB RRC buffer size, according to TS 38.306. - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) |
| --- | --- | --- |
|  | Potential suitable scenario | - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs. |
| CN -> UE via gNB | Challenges | - Same challenges as OTA solution ‘gNB->UE via CP’, if OTA solution ‘CN -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desirable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution ‘CN -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘CN -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA2. |
| OAM -> UE via gNB | Challenges | - Same challenges as OTA solution ‘gNB -> UE via CP’, if OTA solution ‘OAM -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution ‘OAM -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘OAM -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA5. |

# Action

To RAN1:

Action: RAN2 respectfully requests RAN1 to take above replies into account in future discussion.

To RAN3, SA2, SA5:

Action: RAN2 respectfully asks RAN3/SA2/SA5 to confirm RAN2 assumption on non-OTA solutions.

# Dates of the next TSG RAN WG2 meetings

TSG RAN2 Meeting #130		19th – 23rd May, 2025	Malta, MT

TSG RAN2 Meeting #131		25th – 29th  Aug, 2025	India