3GPP TSG-RAN WG3 Meeting #128	R3-253053

Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1265 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RRC Establishment Cause for CIoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, ZTE, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the NB-IoT UE connected with 5GC, it is specified that the RRC Establishment Cause IE indicates the EstablishmentCause-NB IE as defined in TS 36.331, and this is incorrect. As specified in TS 36.331 below, in the RRCConnectionRequest-NB message, the establishmentCause-r16 should be referred. RRCConnectionRequest-5GC-NB-r16-IEs ::= SEQUENCE { ue-Identity-r16 InitialUE-Identity-5GC-NB-r16, establishmentCause-r16 ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, spare4, spare3, spare2, spare1}, cqi-NPDCCH-r16 CQI-NPDCCH-NB-r14, spare BIT STRING (SIZE (11)) } Similarly, in the RRCConnectionResumeRequest-NB message, the resumeCause-r16 should be referred: RRCConnectionResumeRequest-5GC-NB-r16-IEs ::= SEQUENCE { resumeID-r16 I-RNTI-r15, shortResumeMAC-I-r16 ShortMAC-I, resumeCause-r16 EstablishmentCause-NB-r13, cqi-NPDCCH-r16 CQI-NPDCCH-NB-r14, spare BIT STRING (SIZE (4)) } And in case of NB-IoT EDT, in the RRCEarlyDataRequest-NB message, the establishmentCause-r16 should be referred: RRCEarlyDataRequest-5GC-NB-r16-IEs ::= SEQUENCE { ng-5G-S-TMSI-r16 NG-5G-S-TMSI-r15, establishmentCause-r16 ENUMERATED {mo-Data, mo-ExceptionData, mt-Access, spare1}, cqi-NPDCCH-r16 CQI-NPDCCH-NB-r14 OPTIONAL, dedicatedInfoNAS-r16 DedicatedInfoNAS, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } On the other hand, for CIoT connecting to 5GC, in the RRCEarlyDataRequest message, the establishmentCause-r16 should be referred: RRCEarlyDataRequest-5GC-r16-IEs ::= SEQUENCE { ng-5G-S-TMSI-r16 NG-5G-S-TMSI-r15, establishmentCause-r16 ENUMERATED {mo-Data, spare3, spare2, spare1}, dedicatedInfoNAS-r16 DedicatedInfoNAS, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the RRC Establishment Cause to refer to the correct field names in TS 36.331 for NB-IoT UE connected with 5GC. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the RRC Establishment Cause for NB-IoT UE. This CR is functional NBC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong cause will be provided from the ng-eNB to 5GC in case of CIoT/NB-IoT UE access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.111 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: update coverpage and content based on online comments. Rev 2: resubmission. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.111	RRC Establishment Cause

This IE indicates the reason for RRC Connection Establishment as received from the UE in the EstablishmentCause IE defined in TS 38.331 [18] or the EstablishmentCause-5GC IE defined in TS 36.331 [21], or the reason for RRC Connection Resume as received from the UE in the ResumeCause IE defined in TS 38.331 [18] or the ResumeCause-r15 IE defined in TS 36.331 [21] or the reason for Early Data Request as received from the UE in the establishmentCause-r16 contained in the RRCEarlyDataRequest-5GC-r16-IEs IE defined in TS 36.331 [21].

In case of NB-IoT, this IE indicates the reason for RRC Connection Establishment as received from the UE in the establishmentCause-r16 contained in the RRCConnectionRequest-5GC-NB-r16-IEs IE defined in TS 36.331 [21], or the reason for RRC Connection Resume as received from the UE in the EstablishmentCause-NB-13 IE defined in TS 36.331 [21], or the reason for Early Data Request as received from the UE in the establishmentCause-r16 contained in the RRCEarlyDataRequest-5GC-NB-r16-IEs IE defined in TS 36.331 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Establishment Cause | M |  | ENUMERATED (emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess, …, notAvailable, mo-ExceptionData) | The notAvailable value is used in case the UE is re-establishing an RRC connection but there is fallback to RRC connection establishment as described in TS 38.331 [18], or the UE is resuming an RRC connection and the cause value received from the UE does not map to any other value of the RRC Establishment Cause IE. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>