3GPP TSG-RAN WG3 Meeting #128	R3-253081

# Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1263 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.413 for WAB) Support for Wireless Access Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support the Location Services for UEs served by WAB-nodes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added a WAB abbreviation and definitions for WAB related terms. - Added Additional ULI IE in the User Location Information IE. - Added Additional ULI IE in some non-UE associated procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE location services for WAB not supprted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 8.7.1.2, 8.7.2.2, 9.2.6.1, 9.2.6.4, 9.3.1.16, 9.3.1.X(new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: R3-251551, updated based on the latest spec version, and submitted to RAN3#127-bis meeting. Rev 2: R3-252531, merged the agreed TP R3-252456, which was agreed in RAN3 #127-bis meeting. Rev 3: R3-253081, submit to RAN3 #128 meeting. |  |  |  |  |  |  |  |  |

Start of Change

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ACL functionality: as defined in TS 36.413 [16].

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

Public Network Integrated NPN: as defined in TS 23.501 [9].

Stand-alone Non-Public Network: as defined in TS 23.501 [9].

WAB-gNB: as defined in TS 38.401 [2].

WAB-node: as defined in TS 38.401 [2].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

AMF	Access and Mobility Management Function

AUN3	Authenticable Non-3GPP

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTM	Point to Multipoint

PTP	Point to Point

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

RSPP	Ranging/SL Positioning Protocol

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TSS	Timing Synchronisation Status

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

WAB	Wireless Access Backhaul

Next Change

8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message. If the IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and use it for further AMF selection for the IAB-MT.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF. If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the Additional ULI IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, store this information, and take it into account for determining the location of the UEs served by the NG-RAN node, as specified in TS 23.501 [9].

Next Change

8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the Additional ULI IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, store this information, and take it into account for determining the location of the UEs served by the NG-RAN node, as specified in TS 23.501 [9].

Next Change

9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Additional ULI | O |  | 9.3.1.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

Next Change

9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Additional ULI | O |  | 9.3.1.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

Next Change

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>Additional ULI | O |  | 9.3.1.X | Indicates the Additional User Location Information for a UE served by a WAB-gNB. | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |

Next Change

9.3.1.X	Additional ULI

This IE contains the NR CGI and the TAI that reflect the location information of WAB-node which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 |  |
| TAI | M |  | 9.3.3.11 |  |

Next Change

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

>>>>>>>>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<<<<<<<<

FROM NGAP-Containers

id-A2X-PC5-QoS-Parameters,

id-AerialUEsubscriptionInformation,

id-AdditionalULI,

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

>>>>>>>>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- INTERFACE MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- NG Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-RANNodeName					CRITICALITY ignore	TYPE RANNodeName					PRESENCE optional	}|

{ ID id-SupportedTAList				CRITICALITY reject	TYPE SupportedTAList				PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX						PRESENCE mandatory	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX		PRESENCE optional	}|

{ ID id-Extended-RANNodeName		CRITICALITY ignore	TYPE Extended-RANNodeName			PRESENCE optional	}|

{ ID id-AdditionalULI				CRITICALITY ignore	TYPE AdditionalULI 					PRESENCE optional	},

...

}

Next Change

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	}|

{ ID id-AdditionalULI							CRITICALITY ignore	TYPE AdditionalULI 									PRESENCE optional	},

...

}

Next Change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

-------------------skip the unchanged parts-------------------

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-AdditionalULI,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

Next Change

#### -- A

AdditionalDLUPTNLInformationForHOList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF AdditionalDLUPTNLInformationForHOItem

AdditionalDLUPTNLInformationForHOItem ::= SEQUENCE {

additionalDL-NGU-UP-TNLInformation				UPTransportLayerInformation,

additionalQosFlowSetupResponseList				QosFlowListWithDataForwarding,

additionalDLForwardingUPTNLInformation			UPTransportLayerInformation 							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalDLUPTNLInformationForHOItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalDLUPTNLInformationForHOItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalRedundantDL-NGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional		},

...

}

AdditionalQosFlowInformation ::= ENUMERATED {

more-likely,

...

}

AerialUEsubscriptionInformation ::= ENUMERATED {

allowed,

not-allowed,

...

}

AdditionalULI ::= SEQUENCE {

nRCGI							NR-CGI,

tAI								TAI,

iE-Extensions					ProtocolExtensionContainer { { AdditionalULI-ExtIEs} }	OPTIONAL,

...

}

AdditionalULI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

A2X-PC5-FlowBitRates ::= SEQUENCE {

a2X-GuaranteedFlowBitRate		BitRate,

a2X-MaximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { {A2X-PC5-FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

A2X-PC5-FlowBitRates-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Next Change

-- U

-------------------skip the unchanged parts-------------------

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation						CRITICALITY ignore	EXTENSION NGRAN-CGI											PRESENCE optional	}|

{ ID id-NID										CRITICALITY reject	EXTENSION NID												PRESENCE optional	}|

{ ID id-NRNTNTAIInformation						CRITICALITY ignore	EXTENSION NRNTNTAIInformation 							PRESENCE optional	}|

{ ID id-MobileIAB-MTUserLocationInformation		CRITICALITY ignore	EXTENSION MobileIAB-MTUserLocationInformation 		PRESENCE optional	}|

{ ID id-AdditionalULI						CRITICALITY ignore	EXTENSION AdditionalULI  		 					PRESENCE optional	},

...

}

Next Change

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-------------------skip the unchanged parts-------------------

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-AdditionalULI										ProtocolIE-ID ::= xxx

End of Change