3GPP TSG-RAN3 Meeting #128	R3-253146

Malta, MT, 19 – 23 May, 2025

Agenda Item:	20.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on LS Response for R3-253010 and R3-253011

Document for:	Discussion/Decision

# Introduction

In this paper we discuss the LS response and provide draft LS response for RAN2 LSes R3-253010 and R3-253011.

# LS Response for R3-253010

In this LS RAN2 has listed OTA based and non-OTA based solutions for NW dataset and model parameter transfer. RAN2 has asked RAN3/SA2/SA5 to confirm RAN2 assumption on non-OTA solution listed in the LS.

R3-253010 Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1). From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption. Furthermore, it does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Table 1. non-OTA candidate solutions Option; Impacted WG; Specification impact/Implementation impact. OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2). gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5). To RAN3, SA2, SA5: Action: RAN2 respectfully asks RAN3/SA2/SA5 to confirm RAN2 assumption on non-OTA solutions.

From the different non-OTA solution listed above, only gNB  OAM/CN  UE solution3 has RAN3 impact. Hence RAN3 should consider only solution 3 for LS response to RAN2.

Proposal 1: RAN3 should focus on, only gNB  OAM/CN  UE non-OTA solution for LS response to RAN2.

With the existing architecture, it is feasible for gNB to transfer NW dataset/model parameters to OAM or to CN. There may be stage 2 and stage 3 impacts with both the solutions.

Proposal 2: RAN3 to confirm to RAN2 that it is feasible to support data transfer from gNB to OAM/CN with the existing architecture. Corresponding draft LS is provided in Annex A.

# LS Response for R3-253011

RAN2 has sent the agreements made on OAM centric solution for NW-side data collection and has asked RAN3 to consider the RAN2 agreements for RAN3 work.

R3-253011 # 1 Overall description For OAM-centric solution for NW-side data collection, RAN2 has made the following agreements: RAN2 assumes to only adopt management based immediate MDT for NW-side data collection. For NR-DC, the configuration and reporting of NW sided data collection can only be via MN. # 2 Actions To SA5 and RAN3: ACTION: RAN2 respectfully asks SA5 and RAN3 to take these agreements into account for their work and notify RAN2 if there are any concerns to support them for Rel-19.

The first agreement states that RAN2 adopts only Management based immediate MDT for NW side data collection. The second agreement states that data collection can only be done at the MN. Both of these are generic agreements and RAN3 shall note this agreement with no further action. RAN3 does not have a WI to work on NW side data collection related agreements.

Proposal 3: RAN3 to note the agreements from RAN2 with no further action. RAN3 to inform RAN2 that RAN3 does not have a WI to work on NW side data collection. Corresponding draft LS is provided in Annex B.

# Summary

Based on the discussion above, we have following proposals:

R3-253010

Proposal 1: RAN3 should focus on, only gNB  OAM/CN  UE non-OTA solution for LS response to RAN2.

Proposal 2: RAN3 to confirm to RAN2 that it is feasible to support data transfer from gNB to OAM/CN with the existing architecture. Corresponding draft LS is provided in Annex A.

R3-253011

Proposal 3: RAN3 to note the agreements from RAN2 with no further action. RAN3 to inform RAN2 that RAN3 does not have a WI to work on NW side data collection. Corresponding draft LS is provided in Annex B.

# Annex A Draft LS Reply for R3-253010

Title:	Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R3-253010

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN2, RAN1

Cc:	SA2, SA5

Contact person:	Geetha Rajendran

geethapr@qti.qualcomm.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 thanks RAN2 LS on signalling feasibility of dataset and parameter sharing.

RAN3 confirms it is feasible to support the transfer of dataset/model parameters from gNB to OAM/CN in non-OTA approach, with existing architecture.

# 2	Actions

To RAN2

ACTION: 	RAN3 respectfully requests RAN2 to take above reply into account for their work.

# 3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129	25-28 Aug 2025			 Bangalore, IN

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			 Prague, CZ

# Annex B	Draft LS Reply for R3-253011

Title:	Reply LS on OAM-centric solution for NW-side data collection

Response to:	R3-253011

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN2

Cc:	SA5

Contact person:	Geetha Rajendran

geethapr@qti.qualcomm.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 thanks RAN2 LS on OAM-centric solution for NW-side data collection.

RAN3 acknowledges the RAN2 agreements on OAM-centric solution for NW-side data collection. However, RAN3 would like to inform RAN2 that RAN3 does not have a WI in Rel-19 to work on OAM-centric solution for NW-side data collection.

# 2	Actions

To RAN2

ACTION: 	RAN3 respectfully requests RAN2 to take above into consideration.

# 3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129	25-28 Aug 2025			 Bangalore, IN

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			 Prague, CZ