3GPP TSG-RAN WG3 Meeting #128	R3-253147

Malta, MT, 19 – 23 May 2025

Agenda Item:	22.2

Source: 				LG Electronics Inc., FirstNet

Title:			(TP for TS 38.401 and 38.473) Support of Multi-hop relay

Document for:		Discussion and Decision

1.	Introduction

In current RAN3 BL CR to TS 38.401 [1], there are some remaining open issues to be solved for the multi-hop relay. Also, RAN2 currently considers in [2] that in Approach 1, the SRB0 message (i.e., RRCSetupRequest or RRCResumeRequest) is forwarded by the U2N Relay UE in RRC_INACITVE (i.e., fast/parallel RRC setup procedure). In this contribution, therefore, we will discuss these open issues for U2N Remote UE’s initial access via the multi-hop relay and the RAN3 impact on the fast/parallel RRC setup in Approach 1, and then provide our views on it.

2.	Discussion

In current RAN3 BL CR to TS 38.401 [1], there are some remaining open issues to be solved for the multi-hop relay. First FFS is to capture the definition of terminologies related to the Multi-hop relay operation based on RAN2 progress. In RAN2#129bis meeting, the followings were captured into the Chair’s minute [3]:

Agreement in RAN2#129bis [3]:

The term “U2N relay UE” can include first/intermediate/last relay UEs in multihop, if not otherwise qualified.  We can distinguish explicitly when a requirement applies only to single-hop or only to certain multihop roles.

The term “U2N remote UE” can include multihop remote UEs, if not otherwise qualified.  We can distinguish explicitly when a requirement applies only to single-hop or only to multihop.

Expand the definitions of “U2N relay UE” and “U2N remote UE” in the CR definition sections to include multihop.  This drafting can be done initially in the 38.300 running CR and migrated later into the other CRs.

The existing multihop definitions, e.g., first/intermediate/last relay UE, are kept.  FFS if they need debugging (business as usual).  The intention is that the first relay UE is an intermediate relay UE, as originally agreed.

The terms “parent” and “child” UE can be used in CR drafting.  FFS if they need to be defined, based on how we end up using them.

Based on these agreements, the following definitions related to the Multi-hop relay operation for TS 38.300 were endorsed in [4]:

R2-2503088 in [4]: ## 3.2 Definitions For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2]. … Child UE: A U2N Relay UE’s next hop in downstream direction for serving a U2N Remote UE in U2N Relay communication. Child UE can be the U2N Remote UE or a U2N Relay UE. Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met. CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling. DAPS Handover: a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time, as defined in TS 23.501 [3]. Direct Path: a type of UE-to-Network transmission path, where data is transmitted between a UE and the network without sidelink relaying. Downstream: direction toward child node or UE in IAB-topology or U2N Remote UE in U2N Relay communication. Early Data Forwarding: data forwarding that is initiated before the UE executes the handover. Earth-centered, earth-fixed: a global geodetic reference system for the Earth intended for practical applications of mapping, charting, geopositioning and navigation, as specified in NIMA TR 8350.2 [51]. eRedCap UE: a UE with enhanced reduced capabilities as specified in clause 4.2.22.1 in TS 38.306 [11]. Feeder link: wireless link between the NTN Gateway and the NTN payload. First U2N Relay UE: an Intermediate U2N Relay UE having both PC5 connection to a parent UE and PC5 connection to a U2N Remote UE for serving the U2N Remote UE in case of multi-hop L2 U2N Relay communication. … Intermediate U2N Relay UE: a U2N Relay UE having both PC5 connection to a parent UE and PC5 connection to a child UE or a U2N Remote UE for serving the U2N Remote UE in case of multi-hop L2 U2N Relay communication. Inter-donor partial migration: migration of an IAB-MT to a parent node underneath a different IAB-donor-CU while the collocated IAB-DU and its descendant IAB-node(s), if any, are terminated at the initial IAB-donor-CU. The procedure renders the said IAB-node as a boundary IAB-node. Intra-system Handover: handover that does not involve a CN change (EPC or 5GC). Inter-system Handover: handover that involves a CN change (EPC or 5GC). Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node. Last U2N Relay UE: a U2N Relay UE having both Uu connection to the network and PC5 connection to a child UE for serving a U2N Remote UE in case of L2 U2N Relay communication. Editor’s note: FFS whether the term ‘Last U2N Relay UE’ can also cover single-hop U2N Relay UE, if needed, i.e. single-hop U2N Relay UE can be also called ‘Last U2N Relay UE’ in Rel-19 specifications. … Parent UE: A U2N Remote UE or U2N Relay UE’s next hop U2N Relay UE in upstream direction for serving the U2N Remote UE in U2N Relay communication. PC5 Relay RLC channel: an RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, between L2 U2N Relay UEs (in case of multi-hop L2 U2N relay communication), or between L2 U2U Remote UE and L2 U2U Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network/UE-to-UE Relay. … U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s). Up to three L2 U2N Relay UEs (i.e. one Last U2N Relay and up to two Intermediate U2N Relays including one First U2N Relay) can be configured for serving a L2 U2N Remote UE in multi-hop L2 U2N Relay communication in this release. U2N Remote UE: a UE that communicates with the network via one or more U2N Relay UEs on an indirect path. U2U Relay UE: a UE that provides functionality to support connectivity between two U2U Remote UEs. U2U Remote UE: a UE that communicates with other UE(s) via a U2U Relay UE. UE-to-Network Relay communication: A mode of communication in which a UE communicates with the network through an indirect path involving only one U2N Relay UE for single-hop L2 U2N Relay communication or multiple L2 U2N Relay UEs for multi-hop L2 U2N Relay communication. UE-to-Network Relay discovery: A mode of NR sidelink discovery in which a UE disovers other UEs for U2N Relay communication. …

Based on RAN2 progress, RAN3 can refer to the terminologies in the running CR for TS 38.300. Therefore, the BL CRs to TS 38.401 and TS 38.473 need to be updated to cover these terminologies. However, the BL CR to TS 38.470 already takes into account the RAN2 agreements for the terminologies. For BL CRs to TS 38.413 and TS 38.423, there is no need to capture the multi-hop relay related terminologies because these specs have a minor impact on the multi-hop relay authorization.

Proposal 1: The BL CRs to TS 38.401 and TS 38.473 refer to the Multi-hop relay related terminologies in the RAN2 running CR to TS 38.300.

In last RAN3 meeting, it was discussed whether the RBs (including SRB0) of the U2N Remote UE can be multiplexed over the existing egress PC5/Uu Relay RLC channel for the different RB(s) of different Remote UE(s). According to RAN2 running CR to TS 38.300 [4], it seems that the RBs of different U2N Remote UEs can be mapped into the same PC5/Uu Relay RLC channel in PC5 hop and Uu hop as follows:

R2-2503088 [4]: … #### 16.12.2.1 L2 UE-to-Network Relay The protocol stacks for the user plane and control plane of single-hop L2 U2N Relay architecture are illustrated in Figure 16.12.2.1-1 and Figure 16.12.2.1-2. The protocol stacks for the user plane and control plane of multi-hop L2 U2N Relay architecture are illustrated in Figure 16.12.2.1-3 and Figure 16.12.2.1-4. The SRAP sublayer is placed above the RLC sublayer for both CP and UP at both PC5 interface and Uu interface. The Uu SDAP, PDCP and RRC are terminated between L2 U2N Remote UE and gNB, while SRAP, RLC, MAC and PHY are terminated in each hop (i.e., the link between L2 U2N Remote UE and the L2 U2N Relay UE, the link between L2 U2N Relay UEs, and the link between L2 U2N Relay UE and the gNB). For L2 U2N Relay, the SRAP sublayer over PC5 hop is only for the purpose of bearer mapping. The SRAP sublayer is not present over PC5 hop for relaying the L2 U2N Remote UE's message on BCCH and PCCH. For L2 U2N Remote UE's message on SRB0, the SRAP header is not present over PC5 hop, but the SRAP header is present over Uu hop for both DL and UL. Figure 16.12.2.1-1: User plane protocol stack for single-hop L2 UE-to-Network Relay Figure 16.12.2.1-2: Control plane protocol stack for single-hop L2 UE-to-Network Relay Figure 16.12.2.1-3: User plane protocol stack for multi-hop L2 UE-to-Network Relay Figure 16.12.2.1-4: Control plane protocol stack for multi-hop L2 UE-to-Network Relay For L2 U2N Relay, for uplink: - The Uu/PC5 SRAP sublayer at the U2N Relay UE performs UL bearer mapping between end-to-end Uu Radio Bearers of L2 U2N remote UE (identified for the purposes of this mapping by the local Remote UE ID and an associated bearer ID) and egress Uu/PC5 Relay RLC channels over the L2 U2N Relay UE Uu/PC5 interface. For uplink relaying traffic, the different end-to-end Uu Radio Bearers (SRBs or DRBs) of the same L2 U2N Remote UE and/or different L2 U2N Remote UEs can be multiplexed over the same egress Uu/PC5 Relay RLC channel; - The Uu/PC5 SRAP sublayer at the U2N Relay UE supports L2 U2N Remote UE identification for the UL traffic. The identity information of L2 U2N Remote UE end-to-end Uu Radio Bearer and a local Remote UE ID are included in the Uu SRAP header at UL in order for gNB to correlate the received packets for the specific PDCP entity associated with the right end-to-end Uu Radio Bearer of the L2 U2N Remote UE; - The PC5 SRAP sublayer at the L2 U2N Remote UE supports UL bearer mapping between L2 U2N Remote UE end-to-end Uu Radio Bearers and egress PC5 Relay RLC channels. For L2 U2N Relay, for downlink: - The Uu/PC5 SRAP sublayer at the U2N Relay UE performs DL bearer mapping at gNB to map end-to-end Uu Radio Bearer (SRB, DRB) of L2 U2N Remote UE (identified for the purposes of this mapping by the local Remote UE ID and an associated bearer ID) into Uu/PC5 Relay RLC channel. The Uu/PC5 SRAP sublayer at the U2N Relay UE performs DL bearer mapping and data multiplexing between multiple end-to-end Radio Bearers (SRBs or DRBs) of a L2 U2N Remote UE and/or different L2 U2N Remote UEs and one Uu Relay RLC channel over the L2 U2N Relay UE Uu/PC5 interface; - The Uu/PC5 SRAP sublayer at the U2N Relay UE supports L2 U2N Remote UE identification for DL traffic. The identity information of L2 U2N Remote UE end-to-end Uu Radio Bearer and a local Remote UE ID are included into the Uu SRAP header by the gNB at DL for the L2 U2N Relay UE to identify the corresponding end-to-end Uu Radio Bearer(s) of L2 U2N Remote UE; - The PC5 SRAP sublayer at the L2 U2N Relay UE performs DL bearer mapping between end-to-end Uu Radio Bearers of L2 U2N remote UE and egress PC5 Relay RLC channels; - The PC5 SRAP sublayer at the L2 U2N Remote UE correlates the received packets with the right PDCP entity associated with the given end-to-end Uu Radio Bearer of the L2 U2N Remote UE based on the identity information included in the PC5 SRAP header. A local Remote UE ID is included in both PC5 SRAP header and Uu SRAP header. L2 U2N Relay UE is configured by the gNB with the local Remote UE ID(s) to be used in SRAP header. L2 U2N Remote UE obtains the local Remote ID from the gNB via Uu RRC messages including RRCSetup, RRCReconfiguration, RRCResume and RRCReestablishment. The end-to-end DRB(s) or end-to-end SRB(s), except SRB0, of L2 U2N Remote UE can be multiplexed to the PC5 Relay RLC channels and Uu Relay RLC channels in both PC5 hop and Uu hop, but an end-to-end DRB and an end-to-end SRB can neither be mapped into the same PC5 Relay RLC channel nor be mapped into the same Uu Relay RLC channel. It is the gNB responsibility to avoid collision on the usage of local Remote UE ID. The gNB can update the local Remote UE ID by sending the updated local Remote UE ID via RRCReconfiguration message. The serving gNB can perform local Remote UE ID update independent of the PC5 unicast link L2 ID update procedure. …

Therefore, RAN3 needs to discuss how to support the mapping of the RBs of different U2N Remote UEs to same PC5/Uu Relay RLC channel in F1 interface.

Observation 1: The mapping of the RBs of different U2N Remote UEs to same PC5/Uu Relay RLC channel needs to be supported in F1 interface.

Since the different RB(s) of different Remote UE(s) can be already multiplexed over the same Uu Relay RLC channel in Rel-17 single-hop relay operation, this can be reused for Rel-19 multi-hop relay operation. Therefore, there is no RAN3 issue to support the multiplexing for the Uu Relay RLC channel in Rel-19 multi-hop relay operation. However, for PC5 Relay RLC channel, it is unclear how to handle the multiplexing of the RBs of different U2N Remote UEs to the same egress PC5 Relay RLC channel over F1 interface. In Rel-17 single-hop relay operation, since each PC5 Relay RLC channel configuration provided to U2N Relay UE is uniquely associated with one U2N Remote UE, there is no multiplexing between the RBs of different U2N Remote UEs over the same PC5 Relay RLC channel.

Observation 2: From the RAN3 point of view, the different RB(s) of different Remote UE(s) can be multiplexed over the same Uu Relay RLC channel.

Observation 3: It is unclear how to handle the multiplexing of the RBs of different U2N Remote UEs to the same egress PC5 Relay RLC channel over F1 interface.

RAN3 needs to discuss whether the gNB-CU indicates to the gNB-DU that the existing PC5 Relay RLC channel is also used for a new U2N Remote UE. Since the gNB-CU already has the PC5 Relay RLC channel configurations established for other U2N Remote UE, it is possible to generate the RRCReconfiguration message to configure the PC5 Relay RLC channel to a new U2N Relay UE without the help of gNB-DU. In this case, the gNB-DU is difficult to notice that the new U2N Remote UE is additionally mapped to the existing PC5 Relay RLC channel. Therefore, our view is that the list of U2N Remote UEs mapped to the same PC5 Relay RLC channel should be provided to the gNB-DU. To this end, when the existing PC5 Relay RLC channel is used for relaying the SRB message of a new U2N Remote UE, the gNB-CU needs to include the local ID of a new U2N Remote UE to the PC5 RLC Channel to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message. One possible option is to reuse the existing Remote UE Local ID IE in the PC5 RLC Channel to Be Modified List IE. However, since the gNB-DU is difficult to differentiate between the wrong local ID value (i.e., error case) and the additional local ID value (i.e., reuse of existing PC5 Relay RLC channel), it is better to introduce a new IE to indicate to the gNB-DU the local ID of the new Remote UE which is mapped to the existing PC5 Relay RLC channel.

Proposal 2: The gNB-CU includes the Remote UE Local ID to Add IE (i.e., local ID for a new U2N Remote UE) to the PC5 RLC Channel to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message to indicate that the existing PC5 Relay RLC channel is used for a new U2N Remote UE.

Similarly, it is possible that one of the U2N Remote UEs mapped to the same PC5 Relay RLC channel is deleted due to e.g., the UE inactivity or the remapping to other PC5 Relay RLC channel. Therefore, it is also needed to introduce a new IE to indicate to the gNB-DU the Remote UE Local ID to delete.

Proposal 2a: The gNB-CU includes the Remote UE Local ID to Delete IE to the PC5 RLC Channel to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message to indicate that the PC5 Relay RLC channel is no longer used for the U2N Remote UE.

The following Editor’s Notes are related to the gNB-DU’s awareness on the multi-hop relay operation.

- FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message

- FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE

In Rel-17 U2N Relay, the sidelink configuration container in the INITIAL UL RRC MESSAGE TRANSFER message is the configuration for the PC5 Relay RLC channel between U2N Remote UE and U2N Relay UE for relaying of U2N Remote UE’s SRB1. In Rel-19 Multi-hop Relay, our understanding is that for relaying of U2N Remote UE’s SRB1, this information should include the PC5 Relay RLC channel between First Relay UE and U2N Remote UE. However, in order to correctly include this information in the INITIAL UL RRC MESSAGE TRANSFER message in Rel-19, the gNB-DU needs to know the U2N Remote UE is directly connected to the First Relay UE. For example, as shown in Figure 1, the SRB0 of U2N Remote UE#1 (connected to gNB via multi-hop relay) and U2N Remote UE#4 (connected to gNB via single-hop relay) can be multiplexed over the same egress Uu Relay RLC channel. In this case, the local ID enables the gNB-DU to differentiate between the U2N Remote UE#1 and the U2N Remote UE#4. However, without a priori knowledge, the gNB-DU cannot know that the RRCSetupRequest message from U2N Remote UE#1 is delivered via First Relay UE, the Intermediate Relay UE, and the Last Relay UE. In other words, the gNB-DU may have the wrong knowledge on that the U2N Remote UE#1 is connected to the gNB via the single Relay UE (i.e., Last Relay UE). Therefore, the gNB-DU may allocate the PC5 Relay RLC channel for single-hop relay operation, thus causing misconfiguration of the PC5 Relay RLC channel for the U2N Remote UE#1’s SRB1. If RAN2 introduces a new Remote UE local ID or a new SRAP header format for multi-hop relay operation, it is natural for the gNB-DU to know that the U2N Remote UE#1 is connected to the gNB via multi-hop relay. However, for now, it seems that RAN2 reuses the existing Remote UE local ID and SRAP header format.

Proposal 3: Upon the reception of the RRCSetupRequest message, the gNB-DU should know that the U2N Remote UE is directly connected to the First Relay UE.

Figure 1: Example of coexistence of single hop relay operation and multi-hop relay operation

According to previous RAN3 agreement, the gNB-CU can request to the gNB-DU to establish a new PC5/Uu Relay RLC channel for relaying of U2N Remote UE’s SRBs (i.e., SRB0, SRB1, and SRB2). If new PC5 Relay RLC channel of the U2N Relay UE for relaying of U2N Remote UE#1’s SRB0 is established, the gNB-CU provides the Remote UE Local ID to the gNB-DU in the PC5 RLC Channel to Be Setup List IE of the UE CONTEXT MODIFICATION REQUEST message. Therefore, upon the reception of the RRCSetupRequest message of U2N Remote UE#1, the gNB-DU can check whether the RRC message is delivered via the multi-hop relay, based on the Remote UE Local ID.

Proposal 4: If a new PC5 Relay RLC channel is established, the gNB-DU becomes aware that the U2N Remote UE is directly connected to the First Relay UE based on the Remote UE Local ID.

If the existing PC5 Relay RLC channel is used for relaying the U2N Remote UE#1’s SRB0 message, the gNB-CU might not include the PC5 RLC Channel to Be Setup List IE in the UE CONTEXT MODIFICATION REQUEST message. Therefore, since the Remote UE Local ID is not provided, the gNB-DU cannot identify the parent node of the U2N Remote UE#1 (i.e., First Relay UE) upon the reception of the RRCSetupRequest message of U2N Remote UE#1. If Proposal 2 is agreed, the Remote UE Local ID to Add IE enables the gNB-DU to identify the parent node of the U2N Remote UE#1. If Proposal 2 is not agreed (i.e., Remote UE Local ID is not provided in the PC5 RLC Channel to Be Setup List IE or the PC5 RLC Channel to Be Modified List IE), the gNB-CU should at least provide the Remote UE Local ID to the gNB-DU in the UE CONTEXT MODIFICATION REQUEST message of the First Relay UE.

Proposal 4a: In the case where the existing PC5 Relay RLC channel is reused, if Proposal 2 is agreed, the gNB-DU becomes aware that the U2N Remote UE is directly connected to the First Relay UE based on the additional Remote UE Local ID.

Proposal 4b: In the case where the existing PC5 Relay RLC channel is reused, if Proposal 2 is not agreed, the gNB-CU should at least provide the Remote UE Local ID to the gNB-DU in the UE CONTEXT MODIFICATION REQUEST message of the First Relay UE.

From the gNB-CU’s point of view, each UE is connected to the gNB in sequence. That is, the gNB-CU already knows that the Intermediate Relay UE in RRC_CONNECTED is connected to the gNB via the Last Relay UE. Upon reception of the SidelinkUEInformationNR message from the Intermediate Relay UE, the gNB-CU becomes aware that the First Relay UE is connected to the gNB via the Intermediate Relay UE and the Last Relay UE. Similarly, when the First Relay UE in RRC_CONNECTED sends its own SidelinkUEInformationNR message to the gNB-CU, the gNB-CU becomes aware that the Remote UE is connected to the gNB via the First Relay UE, the Intermediate Relay UE and the Last Relay UE. Therefore, there is no need to provide the gNB-DU UE F1AP IDs of the First Relay UE and Intermediate Relay UE(s) to the gNB-CU in the INITIAL UL RRC MESSAGE TRANSFER message.

Proposal 5: There is no need to provide the gNB-DU UE F1AP IDs of the First Relay UE and Intermediate Relay UE(s) to the gNB-CU in the INITIAL UL RRC MESSAGE TRANSFER.

Proposal 6: The gNB-CU can have the knowledge on the whole path for Remote UE when entering all the Relay UEs into RRC_CONNECTED.

In RAN3 #127bis meeting, RAN3 made the following working assumption on the counterpart information.

- WA: Reuse the Peer UE ID IE as counterpart information

In Approach #1, both gNB-CU and gNB-DU become aware of the whole path information consisting of the L2 IDs for U2N Relay UEs and U2N Remote UE since each UE is connected to the gNB in sequence. Therefore, when receiving the L2 ID of the parent UE or child UE along with the PC5 RLC Channel to Be Setup List IE, the gNB-DU can identify the counterpart of the PC5 Relay RLC channel to be established towards the downstream or the upstream. Since the existing Peer UE ID IE can contain the destination L2 ID of each PC5 hop, there is no critical issue to reuse this IE for the multi-hop relay operation. However, as shown in TS 38.473, the existing Peer UE ID IE is introduced only for the Rel-18 single-hop U2U relay operation. Therefore, the semantics description needs to be updated to cover the Rel-19 multi-hop U2N relay operation. In addition, since the PC5 Relay RLC channel between the First Relay UE and U2N Remote UE is uniquely associated with one U2N Remote UE, the Peer UE ID IE is only included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2N Relay UE in the U2N multi-hop relay operation.

TS 38.473: … >>Peer UE ID; O; BIT STRING (SIZE(24)); Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE, defined in TS 38.331 [8]. This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE.; YES; reject. …

Proposal 7: It is proposed to turn the following WA into the agreement and update the semantics description of the Peer UE ID IE to cover the multi-hop relay operation.

- WA: Reuse the Peer UE ID IE as counterpart information

Proposal 7a: The Peer UE ID IE is only included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2N Relay UE in the U2N multi-hop relay operation.

In RAN2#129bis meeting, it was confirmed in RAN2 that Approach 2 is not supported in this release.

Agreement in RAN2#129bis [3]:

Control plane design considers only approach 1 (along with the fast forwarding enhancement for SRB0 if agreed), and approach 2 is not further pursued in Rel-19.

Observation 4: Approach 2 is not supported in this release.

However, it was also considered whether and how to support the fast/parallel RRC setup procedure for Approach 1 [2]. That is, RAN2 had the discussion to forward the SRB0 messages of the U2N Remote UE by U2N Relay UEs not in RRC_CONNECTED in Approach 1. RAN2 will conclude whether and how to support the fast/parallel RRC setup procedure for Approach 1 in RAN2#130 meeting as follows:

Agreement in RAN2#129bis [3]:

Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1.

TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged).

Other approaches are not precluded (contribution-driven) but should be shown at a mature stage considering the time left.

Strive to avoid additional RAN3 impact specific to fast forwarding.

FFS if applicable to DL.

FFS what level of gNB awareness of the path information would be needed.

FFS if fast forwarding is optional/mandatory for UEs to support.

In RAN2, two options are considered to support the fast forwarding of the SRB0 messages [5][6].

- Option 1 [5]: SRAP header adding a L2 ID field to indicate U2N Remote UE in a new header format.

- Option 2 [6]: gNB allocated Local ID(s) to Intermediate Relay UE in the RRCRelease message when the Intermediate Relay UE enters into RRC_INACTIVE. The Intermediate Relay UE then assigns this local ID to represent U2N Remote UE when creating SRAP header.

The overall procedure for the fast/parallel RRC setup can be illustrated as follows:

Figure 2: Overall Procedure for Fast/Parallel RRC Setup with Option 1 and 2

In Option 1, the First Relay UE in RRC_IDLE/RRC_INACTIVE state does not withhold the RRCSetupRequest message of the U2N Remote UE until it enters into RRC_CONNECTED, but immediately sends it to the parent UE (i.e., Intermediate Relay UE). In addition, the First Relay UE in RRC_IDLE/RRC_INACTIVE state generates its own RRCSetupRequest message for the state transition to RRC_CONNECTED and then sends it to the parent UE as well. Similarly, the Intermediate Relay UE in RRC_IDLE/RRC_INACTIVE state forwards the RRCSetupRequest messages of the U2N Remote UE and First Relay UE to the Last Relay UE, and also sends its own RRCSetupRequest message towards the gNB. During Steps 1~3, each Relay UE remembers which PC5 link received the first SRB0 message in UL and uses it for DL egress link determination.

In Option 2, when entering into RRC_INACTIVE state, the local ID list is provided to the First Relay UE. Upon the reception of the RRCSetupRequest message from the U2N Remote UE, the First Relay UE selects one from the local ID list and assigns it to the U2N Remote UE. The First Relay UE forwards the RRCSetupRequest message of the U2N Remote UE together with the SRAP header containing the assigned local ID for the U2N Remote UE. Then, the First Relay UE generates its own RRCResumeRequest message and sends it to the Last Relay UE. Similarly, the Intermediate Relay UE performs a similar behaviour (i.e., forwarding of the SRB0 message of the U2N Remote UE and the First Relay UE, and transmission of its own SRB0 message). During Steps 1~3, each Relay UE remembers the local ID of the child UE in UL and uses it for DL egress link determination.

The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the relay operation for the U2N Remote UE, First Relay UE, and Intermediate Relay UE. For Option 2, the Last Relay UE also includes the local IDs received from the child UEs in the SidelinkUEInformationNR message. Therefore, for Option 2, the gNB-CU does not need to assign the local IDs to the U2N Remote UE and First Relay UE. For the Intermediate Relay UE, the gNB-CU may allocate the local ID as in Rel-17 single-hop relay operation.

Observation 5: For Option 2, the gNB-CU does not allocate the local IDs to the U2N Remote UE and First Relay UE.

In Step 5, the gNB-CU requests to the gNB-DU the Uu Relay RLC channel configuration for relaying the SRB0 message (i.e., RRCSetupRequest or RRCResumeRequest) for all UEs. If RAN2 decides to apply the fast forwarding of the SRB0 message only on the uplink (i.e., forwarding of the RRCSetupRequest message or the RRCResumeRequest message), the gNB-CU may also request the PC5 Relay RLC channel configuration for relaying the SRB0/1 message (i.e., RRCSetup or RRCResume) to the gNB-DU. However, since all UEs in the multi-hop relay operation try to access to the gNB at the same time, the gNB-CU does not have the path information for each UE. Since the gNB-CU cannot decide the parent UE and child UE of each UE accordingly, it is difficult to provide the Peer UE ID IE together with the PC5 RLC Channel to Be Setup List IE to the gNB-DU. For now, RAN2 considers the inclusion of the path information for each UE into the SidelinkUEInformationNR message. In this case, the gNB-CU can receive the path information from the Last Relay UE. Also, the gNB-DU can know the path information because the gNB-CU also forwards the SidelinkUEInformationNR message to the gNB-DU.

Observation 6: In case where the fast/parallel RRC setup is applied to the UL, the gNB-CU has to acquire the path information to configure the PC5 Relay RLC channel.

If RAN2 decides to apply the fast forwarding of the SRB0/1 message on the downlink (i.e., forwarding of the RRCSetup message or the RRCResume message) as well, the gNB-CU might not request to the gNB-DU the SRB0/1 relaying PC5 Relay RLC channel configuration because the specified PC5 Relay RLC channel (i.e., SL-RLC0/1 or a new SL-RLC) is used for relaying the SRB0/1 message (i.e., RRCSetup or RRCResume). Therefore, there is no chance to provide additional information (e.g., local ID for the UE) by the gNB-CU to the gNB-DU.

Observation 7: In case where the fast/parallel RRC setup is also applied to the DL, the gNB-CU is difficult to provide additional information (e.g., local ID for the UE) by the gNB-CU to the gNB-DU.

After the configuration of the Uu Relay RLC channel by gNB, the Last Relay UE forwards all RRC messages to the gNB-DU. In Steps 1b-1 and 2b-1 of Option 1, the U2N Relay UE uses a new SRAP header format containing the L2 ID of the UE in the PC5 hop. However, it is unclear if this new SRAP header format is also used for in Uu hop. Our understanding is that for now, RAN2 tries to avoid the impact on the Uu SRAP operation. That is, since the legacy SRAP header is reused in Uu hop, the gNB-DU includes the BEARER ID and the Local ID for the UE in the SRAP header for DL as in Rel-17 single-hop relay operation. However, if RAN2 decides to change the Uu SRAP operation, the gNB-DU should remember the L2 ID received in the SRAP header of the RRCSetupRequest message and include it together with the Local ID into the SRAP header of the RRCSetup message. This is a new gNB-DU’s behaviour compared to Rel-17 single-hop relay operation.

Observation 8: In case where a new SRAP header format is used for Uu hop in Option 1, the gNB-DU should include the L2 ID and Local ID into the SRAP header of the RRCSetup message.

As opposed to Option 1, Option 2 reuses the existing SRAP header which contains the BEARER ID and the Local ID for the UE. Therefore, there is no RAN3 impact on the Uu SRAP operation.

Then, in Step 9, the gNB-DU needs to identify the parent node of the UE sending the SRB0 message (i.e., RRCSetupRequest or RRCResumeRequest) to configure the SRB1 relaying PC5 Relay RLC channel between the UE and the parent node. In the basic procedure (i.e., all U2N Relay UEs are in RRC_CONNECTED) for Approach 1, each UE is connected to the gNB in sequence. Hence, the gNB-DU can identify the parent node based on the information (e.g., local ID for the UE) provided by the gNB-CU and the whole path knowledge in the gNB-DU. However, in the fast/parallel RRC setup, the gNB-DU does not have the knowledge on the whole path for each UE. As mentioned earlier, if the path information is included in the SidelinkUEInformationNR message, this problem can be resolved.

However, in case where the fast/parallel RRC setup is also applied to the DL, the way the gNB-CU provides the additional information (e.g., local ID for the UE) to the gNB-DU is still missing. To this end, we think that if the mapping information between the L2 ID of the UE and the local ID of the UE is provided in Step 5 of Figure 2, it enables the gNB-DU to identify the parent node of the UE. For example, upon the reception of the RRCSetupRequest message of the First Relay UE in Option 1, the gNB-DU becomes aware that the First Relay UE is directly connected to the Intermediate Relay UE based on the path information and the mapping information between the L2 ID and Local ID.

For Option 2, if the path information in the SidelinkUEInformationNR message is consisted of the local IDs, then the gNB-DU may deduce the parent node of the UE sending the SRB0 message (i.e., RRCSetupRequest or RRCResumeRequest) in Step 9. However, since the gNB-DU does not have the L2 ID of the parent node, we are not sure that it can establish the SRB1 relaying PC5 Relay RLC channel between the UE and the parent node and then include it into the INITIAL UL RRC MESSAGE TRANSFER message. As a result, we think that in Option 2, the mapping information between the L2 ID of the UE and the local ID of the UE should be also provided to the gNB-DU.

Observation 9: In case where the fast/parallel RRC setup is also applied to the DL, the gNB-CU should provide the mapping information between the L2 ID of the UE and the local ID of the UE to the gNB-DU.

According to RAN2 agreements, the uniqueness of the local ID within the cell should be guaranteed by the gNB by implementation.

Agreement in RAN2#129bis [3]:

Reuse the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

•	First relay UE reports the L2 ID of the remote UE to the gNB to request the local ID allocation, the uniqueness of the local ID within the cell is assumed to be guaranteed by the gNB by implementation.

•	The remote UE local ID is 8 bits.

In Option 2, when the First Relay UE enters into the RRC_INACTIVE state, the gNB-CU pre-allocates to the First Relay UE the candidate Local ID(s) used in current cell. When the U2N Remote UE initiates the state transition to RRC_CONNECTED in this cell, the First Relay UE selects one of the candidate Local ID(s) on the behalf of the gNB-CU and uses it for the U2N Remote UE. However, the serving cell of the First Relay UE in RRC_INACTIVE may be changed due to cell change by the parent UE (e.g., Intermediate Relay UE or Last Relay UE). In this case, the First Relay UE should not use the candidate Local ID(s) configured by the gNB-CU because the changed cell is different from the cell whose candidate Local ID(s) are configured by the gNB-CU. In this case, the gNB-CU should keep this information because the First Relay UE can move back to the cell in which the UE most recently received RRCRelease message. Another possible scenario is that when the RNA for the First Relay UE is consisted of multiple cells, different candidate Local ID(s) per cell are configured by the gNB-CU. Therefore, when the serving cell is changed within the RNA, the First Relay UE can accordingly use the candidate Local ID(s) configured for the changed cell. This means that the gNB-CU needs to keep all candidate Local ID(s) configured for the First Relay UE in RRC_INACTIVE within the RNA. As a result, considering the Remote UE Local ID is 8 bits, the remaining local ID space in the gNB-CU may not be enough to avoid collision.

Observation 10: In Option 2, the local ID space in the gNB-CU may not be enough to avoid collision.

Based on observations, it seems that both options for the support of the fast/parallel RRC setup have RAN3 specification impact. Therefore, if RAN2 decides to support the fast/parallel RRC setup, RAN3 needs to enhance the F1AP signalling.

Proposal 8: It is proposed to wait for RAN2 progress on the fast/parallel RRC setup in Approach 1.

The FFS whether Step 18 can be performed earlier (e.g., via Steps 6-13) in current BL CR to TS 38.401 [1] is related to the following RAN2 discussion.

- FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2

Regarding this issue, RAN2 concluded that the principle in Rel-17 U2N Relay is also applied to the Rel-19 Multi-hop Relay as highlighted below:

Agreement in RAN2#129 [7]:

RAN2 assumes that discovery and PC5 connection establishment can be performed in each intermediate UE prior to processing the received RRCSetupRequest by the remote UE.  The related FFS can be removed from the stage 2 description.

For the baseline solution, the last relay sending SUI on behalf of other relay UEs is not supported.  The related FFS can be removed from the stage 2 description.

For the baseline solution, Rel17 SUI message and format is re-used.

No further clarification in stage 2 description is needed to clarify that a relay UE can establish its RLC channel for relaying of SRB1 from its immediate child node during its own connection establishment and details will be clarified in stage 3 (i.e., follow legacy U2N procedure from Rel-17 to set up the RLC channel for SRB1).  The related FFS can be removed from the stage 2 description.

Therefore, in Figure 8.19.xx-1 of BL CR to TS 38.401, Step 18 can be performed via Steps 6-13. This FFS can be changed into the Note as in Rel-17 U2N Relay.

For the last Editor’s Note (i.e., FFS whether this step can be performed earlier), we also think that the additional configuration of the PC5/Uu Relay RLC channel to the U2N Relay UEs for relaying of U2N Remote UE’s DRBs and SRBs can follow the legacy U2N procedure from Rel-17. There is no issue to support this functionality in Rel-19 multi-hop relay operation. Therefore, we propose to capture the same Note (i.e., This step may be performed earlier) in Rel-17 U2N Relay into the BL CR to TS 38.401.

Proposal 9: It is proposed to change the following Editor’s Notes into the Notes.

- Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13.

- Editor’s Note: FFS whether this step may be performed earlier.

In order to capture the above observations and proposals, the following proposal is also suggested to RAN3:

Proposal 10: It is proposed to agree the corresponding TP for BL CR to TS 38.401 in Appendix #1.

Proposal 11: It is proposed to agree the corresponding TP for BL CR to TS 38.473 in Appendix #2.

3.	Conclusion

In this contribution, we focused on the remaining issues for U2N Remote UE’s initial access via the multi-hop relay and the RAN3 impact on the fast/parallel RRC setup in Approach 1, and then provide our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: The BL CRs to TS 38.401 and TS 38.473 refer to the Multi-hop relay related terminologies in the RAN2 running CR to TS 38.300.

Proposal 2: The gNB-CU includes the Remote UE Local ID to Add IE (i.e., local ID for a new U2N Remote UE) to the PC5 RLC Channel to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message to indicate that the existing PC5 Relay RLC channel is used for a new U2N Remote UE.

Proposal 2a: The gNB-CU includes the Remote UE Local ID to Delete IE to the PC5 RLC Channel to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message to indicate that the PC5 Relay RLC channel is no longer used for the U2N Remote UE.

Proposal 3: Upon the reception of the RRCSetupRequest message, the gNB-DU should know that the U2N Remote UE is directly connected to the First Relay UE.

Proposal 4: If a new PC5 Relay RLC channel is established, the gNB-DU becomes aware that the U2N Remote UE is directly connected to the First Relay UE based on the Remote UE Local ID.

Proposal 4a: In the case where the existing PC5 Relay RLC channel is reused, if Proposal 2 is agreed, the gNB-DU becomes aware that the U2N Remote UE is directly connected to the First Relay UE based on the additional Remote UE Local ID.

Proposal 4b: In the case where the existing PC5 Relay RLC channel is reused, if Proposal 2 is not agreed, the gNB-CU should at least provide the Remote UE Local ID to the gNB-DU in the UE CONTEXT MODIFICATION REQUEST message of the First Relay UE.

Proposal 5: There is no need to provide the gNB-DU UE F1AP IDs of the First Relay UE and Intermediate Relay UE(s) to the gNB-CU in the INITIAL UL RRC MESSAGE TRANSFER.

Proposal 6: The gNB-CU can have the knowledge on the whole path for Remote UE when entering all the Relay UEs into RRC_CONNECTED.

Proposal 7: It is proposed to turn the following WA into the agreement and update the semantics description of the Peer UE ID IE to cover the multi-hop relay operation.

- WA: Reuse the Peer UE ID IE as counterpart information

Proposal 7a: The Peer UE ID IE is only included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2N Relay UE in the U2N multi-hop relay operation.

Proposal 8: It is proposed to wait for RAN2 progress on the fast/parallel RRC setup in Approach 1.

Proposal 9: It is proposed to change the following Editor’s Notes into the Notes.

- Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13.

- Editor’s Note: FFS whether this step may be performed earlier.

Proposal 10: It is proposed to agree the corresponding TP for BL CR to TS 38.401 in Appendix #1.

Proposal 11: It is proposed to agree the corresponding TP for BL CR to TS 38.473 in Appendix #2.

4.	Reference

- R3-253126, “(BL CR to 38.401) Support of Multi-hop relay”, LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei, May, 2025.

- R2-2503078, “Fast/Parallel RRC Setup for MH relay”, Apple, April, 2025.

- R2-2502984, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), April, 2025.

- R2-2503088, “Multi-hop U2N Relay in TS 38.300”, LG Electronics (Rapporteur), April, 2025.

- R2-2502189, “Fast & Parallel RRC Establishment/Configuration for Multi-hop U2N relaying”, Apple, FirstNet, Ericsson, AT&T, Kyocera, April, 2025.

- R2-2502778, “Consideration on CP issues for multi-hop SL relay”, Samsung, April, 2025.

- R2-2501334, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), February, 2025.

5.	Appendix #1: TP for TS 38.401

This appendix provides the Text proposal for TS 38.401 based on the proposals of this contribution.

----------------Start of the First Change---------------

## 3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [34].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [34].Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Addition: as defined in TS 37.340 [12].

Conditional PSCell Change: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

eNB-CP: as defined in TS 36.401 [28].

eNB-UP: as defined in TS 36.401 [28].

en-gNB: as defined in TS 37.340 [12].

Early Data Forwarding: as defined in TS 38.300 [2].

F1-terminating IAB-donor: Refers to the IAB-donor that terminates F1 for the boundary IAB-node or a mobile IAB-node.

First U2N Relay UE: as defined in TS 38.300 [2].

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU. For DC operation, the MgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as master node, and the SgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as secondary node.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU. For DC operation, the MgNB-CU-CP designates the gNB-CU-CP of the gNB-CU for an en-gNB or a gNB acting as master node, and the SgNB-CU-CP designates the gNB-CU-CP of the gNB-CU for an en-gNB or a gNB acting as secondary node.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU. For DC operation, the MgNB-CU-UP designates the gNB-CU-UP of the gNB-CU for an en-gNB or a gNB acting as master node, and the the SgNB-CU-UP designates the gNB-CU-UP of the gNB-CU for an en-gNB or a gNB acting as secondary node.

IAB-node: as defined in TS 38.300 [2].

IAB-donor: as defined in TS 38.300 [2].

IAB-donor-CU: the gNB-CU of an IAB-donor, terminating the F1 interface towards IAB-nodes and IAB-donor-DU.

IAB-donor-DU: the gNB-DU of an IAB-donor, hosting the IAB BAP sublayer (as defined in TS 38.340 [22]), providing wireless backhaul to IAB-nodes.

IAB-DU: as defined in TS 38.300 [2].

IAB-MT: as defined in TS 38.300 [2].

IAB Topology: as defined in TS 38.300 [2].

Intermediate U2N Relay UE: as defined in TS 38.300 [2].

Last U2N Relay UE: as defined in TS 38.300 [2].

Mapped QoS flows: Unicast QoS flows requested to be established, i.e. included in the legacy QoS flow lists in a way, that non-support RAN nodes would attempt to establish unicast QoS flows and supporting RAN nodes can identify them as mapped QoS flows based on the associated QoS information.

Master node: as defined in TS 37.340 [12].

Master gNB: see TS 37.340 [12].

MBS session resource: This term is used for specification of NG, Xn, F1 and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support an MBS Session.

MP Relay UE: as defined in TS 38.300 [2].

MP Remote UE: as defined in TS 38.300 [2].

Multi-path: as defined in TS 38.300 [2].

NCR-MT: as defined in TS 38.300 [7].

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

ng-eNB-CU-Control Plane (ng-eNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-CP terminates the E1 interface connected with the ng-eNB-CU-UP and the W1-C interface connected with the ng-eNB-DU.

ng-eNB-CU-User Plane (ng-eNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol and the SDAP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-UP terminates the E1 interface connected with the ng-eNB-CU-CP and the W1-U interface connected with the ng-eNB-DU.

NG-RAN node: as defined in TS 38.300 [2].

Non-F1-terminating IAB-donor of boundary IAB-node: Refers to the IAB-donor that has an RRC connection with the boundary node but does not terminate F1 with this boundary node.

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

Public Network Integrated NPN: as defined in TS 23.501 [3].

RRC-terminating IAB-donor: Refers to the IAB-donor that terminates the RRC connection of the mobile IAB-node. The RRC-terminating IAB-donor may also be an F1-terminating IAB-donor.

Secondary gNB: see TS 37.340 [12].

Stand-alone Non-Public Network: as defined in TS 23.501 [3].

Subsequent CPAC: see TS 37.340 [12].

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

[Unchanged text skipped]

### 8.19.xx	Remote UE initial access for Multi-hop Layer-2 UE-to-Network Relay

Editor’s Note: Current signaling flows are based on the agreements that the intermediate Relay UE is in RRC_CONNECTED state. Whether the intermediate Relay UE in RRC_IDLE/RRC_INACTIVE state is supported or not is pending to RAN2.

The signalling flow for Remote UE Initial access is shown in Figure 8.19.xx-1.

Figure 8.19.xx-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the First U2N Relay UE via PC5 Relay RLC channel.

3.	The First U2N Relay UE withholds the received RRC message. If the First U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the Intermediate U2N Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the Intermediate U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the First U2N Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped.

4.	The First U2N Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate U2N Relay UE and Last U2N Relay UE.

5.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the First U2N Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

6.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the Last U2N Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0.

7.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the Last U2N Relay UE to gNB-CU, which may contain the PC5 Relay RLC channel configuration towards the Intermediate U2N Relay UE and the Uu Relay RLC channel configuration towards the gNB-DU.

8.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the Last U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The gNB-DU sends the RRCReconfiguration message to the Last U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

10.	The Last U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the Last U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

12.	The gNB-CU configures the Intermediate U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the Intermediate U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

13.	The gNB-CU configures the First U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the First U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

14.	After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the First U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the Intermediate U2N Relay UE and the Last U2N Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the Last U2N Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration between the U2N Remote UE and the First U2N Relay UE for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

16.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

17.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

18.	The gNB-CU configures the Last U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the Last U2N Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First U2N Relay UE and the Intermediate U2N Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First U2N Relay UE and the Intermediate U2N Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

NOTE 1: This step may be performed earlier, e.g., via steps 6~13.

19.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

20.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

21.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

22.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

23.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

24.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

25.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The U2N Remote UE responds with the SecurityModeComplete message.

27.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

28.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

29.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

30.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

31.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

32.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the Last U2N Relay UE, and additional PC5 Relay RLC channels for the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE.

NOTE 2: This step may be performed earlier.

-----------------End of the First Change---------------

6. Appendix #2: TP for TS 38.473

This appendix provides the Text proposal for TS 38.473 based on the proposals of this contribution.

----------------Start of the First Change---------------

## 3.1	Definitions

elementary procedure: F1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU and gNB-DU. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several F1AP EPs together is specified in stage 2 specifications (e.g., TS 38.470 [2]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

BH RLC channel: as defined in TS 38.300 [6].

Child UE: as defined in TS 38.300 [6].

CG-SDT-CS-RNTI: as defined in TS 38.300 [6].

Complete candidate configuration: one type of a candidate configuration as defined in TS 38.331 [8].

Conditional handover: as defined in TS 38.300 [6].

Conditional PSCell Addition: as defined in TS 37.340 [7].

Conditional PSCell Change: as defined in TS 37.340 [7].

DAPS Handover: as defined in TS 38.300 [6].

EN-DC operation: Used in this specification when the F1AP is applied for gNB-CU and gNB-DU in E-UTRAN.

F1-terminating IAB-donor: as defined in TS 38.401 [4].

First U2N Relay: as defined in TS 38.300 [6].

gNB: as defined in TS 38.300 [6].

gNB-CU: as defined in TS 38.401 [4].

gNB-CU UE F1AP ID: as defined in TS 38.401 [4].

gNB-DU: as defined in TS 38.401 [4].

gNB-DU UE F1AP ID: as defined in TS 38.401 [4].

en-gNB: as defined in TS 37.340 [7].

IAB-DU: as defined in TS 38.300 [6].

IAB-MT: as defined in TS 38.300 [6].

IAB-donor: as defined in TS 38.300 [6].

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-node: as defined in TS 38.300 [6].

Intermediate U2N Relay: as defined in TS 38.300 [6].

Last U2N Relay: as defined in TS 38.300 [6].

MBS-associated signalling: When F1AP messages associated to one MBS session uses the MBS-associated logical F1-connection for association of the message to the MBS session in gNB-DU and gNB-CU.

MBS-associated logical F1-connection: The MBS-associated logical F1-connection uses the identities GNB-CU MBS F1AP ID and GNB-DU MBS F1AP ID according to the definition in TS 38.401 [4]. For a received MBS-associated F1AP message the gNB-CU identifies the associated MBS session based on the GNB-CU MBS F1AP ID IE and the gNB-DU identifies the associated MBS session based on the GNB-DU MBS F1AP ID IE.

MBS Session context in a gNB-DU: as defined in TS 38.401 [4].

MBS session resource: as defined in TS 38.401 [4].

Mobile IAB-DU: as defined in TS 38.300 [6].

Mobile IAB-MT: as defined in TS 38.300 [6].

Mobile IAB-node: as defined in TS 38.300 [6].

MP Relay UE: as defined in TS 38.300 [6].

MP Remote UE: as defined in TS 38.300 [6].

Multi-path: as defined in TS 38.300 [6].

Multicast F1-U Context: as defined in TS 38.401 [4].

Other SI: as defined in TS 38.300 [6].

Parent UE: as defined in TS 38.300 [6].

PC5 Relay RLC channel: as defined in TS 38.300 [6].

Public network integrated NPN: as defined in TS 23.501 [21].

RRC-terminating IAB-donor: as defined in TS 38.401 [4].

SRAP: Sidelink relay adaptation protocol, as defined in TS 38.300 [6].

Stand-alone Non-Public Network: as defined in TS 23.501 [21].

UE-associated logical F1-connection: The UE-associated logical F1-connection uses the identities GNB-CU UE F1AP ID and GNB-DU UE F1AP ID according to the definition in TS 38.401 [4]. For a received UE associated F1AP message the gNB-CU identifies the associated UE based on the GNB-CU UE F1AP ID IE and the gNB-DU identifies the associated UE based on the GNB-DU UE F1AP ID IE. The UE-associated logical F1-connection may exist before the F1 UE context is setup in gNB-DU.

UE-associated signalling: When F1AP messages associated to one UE uses the UE-associated logical F1-connection for association of the message to the UE in gNB-DU and gNB-CU.

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s). Up to three L2 U2N Relay UEs (i.e. one Last U2N Relay and up to two Intermediate U2N Relays including one First U2N Relay) can be configured for serving a L2 U2N Remote UE in multi-hop L2 U2N Relay communication in this release.

U2N Remote UE: a UE that communicates with the network via one or more U2N Relay UE(s) on an indirect path.

U2U Relay UE: as defined in TS 38.300 [6].

U2U Remote UE: as defined in TS 38.300 [6].

Uu Relay RLC channel: as defined in TS 38.300 [6].

[Unchanged text skipped]

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

[Unchanged text skipped]

If the musim-CandidateBandList IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for temporary capability restriction.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the gNB-CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], , and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the Remote UE Local ID To Add IE is contained within the PC5 RLC Channel To Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-CU shall, if supported, add the information in the Remote UE Local ID To Add IE for the PC5 Relay RLC channel accordingly. If the Remote UE Local ID To Delete IE is contained within the PC5 RLC Channel To Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-CU shall, if supported, delete information of local ID indicated by Remote UE Local ID to Delete IE accordingly.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

[Unchanged text skipped]

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31, ...) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based or multi-path relay based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 |  | YES | ignore |
| >>UL Configuration | O |  | 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based or multi-path relay based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator Modify | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. | - |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. | - |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | - |  |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | - |  |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| >S-CPAC Request | O |  | ENUMERATED (initiation, …) | Indicates that SN change is for S-CPAC preparation. | YES | reject |
| >S-CPAC Lower Layer Reference Config Request | O |  | ENUMERATED (true, …) |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| Uplink TxDirectCurrentTwoCarrierList Information | O |  | 9.3.1.283 |  | YES | ignore |
| IAB Conditional RRC Message Delivery Indication | O |  | ENUMERATED (true, …) | Indicates whether the RRC message within should be withheld. This IE is only applicable if the UE is an IAB-MT, and the gNB-DU is an IAB-DU. | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 | This IE is only applicable if the UE is an IAB-MT. | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Updated Remote UE Local ID | O |  | Remote UE Local ID 9.3.1.267 | This IE indicates the updated Remote UE Local ID for the U2N Remote UE associated with the F1AP-IDs | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | O |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to Be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …, SRB0) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>>U2U RLC Channel QoS |  |  |  |  | YES | reject |
| >>>>U2U RLC Channel QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>Peer UE ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE, defined in TS 38.331 [8] (in case of single-hop L2 U2N relay communication), or the destination L2 ID of the parent UE or the child UE (in case of multi-hop L2 U2N relay communication). This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE (in case of single-hop L2 U2N relay communication), or a L2 U2N Relay UE (in case of multi-hop L2 U2N relay communication). | YES | reject |
| PC5 RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | O |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …, SRB0) | This IE indicate the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>>U2U RLC Channel QoS |  |  |  |  | YES | reject |
| >>>>U2U RLC Channel QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>Remote UE Local ID to Add | O |  | 9.3.1.267 | This IE indicates the Local ID of the U2N Remote UE to be added whose SRB or DRB is conveyed via the PC5 Relay RLC Channel. | YES | ignore |
| >>Remote UE Local ID to Delete | O |  | 9.3.1.267 | This IE indicates the Local ID of the U2N Remote UE to be removed whose SRB or DRB is no longer conveyed via the PC5 Relay RLC Channel. | YES | ignore |
| PC5 RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| Multicast MBS Session Remove List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has left. | YES | reject |
| UE Multicast MRB to Be Setup at Modify List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup at Modify Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| >SL DRX Cycle Item IEs |  | 1 .. <maxnoofSLdestinations > |  |  | EACH | ignore |
| >>RX UE ID | M |  | BIT STRING (SIZE(24)) | Indicates the destination L2 ID of RX UE associated to this UE. | - |  |
| >>CHOICE SL DRX Information | M |  |  |  | - |  |
| >>>SL DRX Cycle |  |  |  |  |  |  |
| >>>>SL DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Indicates the desired SL DRX cycle for RX UE associated to this UE. | - |  |
| >>>No SL DRX |  |  |  |  | - |  |
| >>>>SL DRX configuration indicator | M |  | ENUMERATED(release,...) |  | - |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.274 |  | YES | ignore |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| DAPS HO status | O |  | ENUMERATED(initiation, …) | This IE is used if DAPS HO is initiated. | YES | ignore |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |
| Uplink TxDirectCurrentMoreCarrierList Information | O |  | 9.3.1.284 |  | YES | ignore |
| CPAC MCG Information |  | 0..1 |  | This IE is used at the MN for MCG configuration as specified in TS 37.340 [7] for CPAC. | YES | ignore |
| >CPAC Trigger | M |  | ENUMERATED (CPAC-preparation, CPAC-executed, …) |  | - |  |
| >PSCell ID | M |  | NR CGI 9.3.1.12 | The PSCell corresponding to the included CG-Config IE at CPAC-preparation or the selected PSCell by the UE at CPAC-executed. | - |  |
| Network Controlled Repeater Authorized | O |  | 9.3.1.288 |  | YES | ignore |
| SDT Volume Threshold | O |  | INTEGER(1.. 192000,...) | Unit: byte. | YES | ignore |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE shall be present if the CHO Trigger IE is present and set to "CHO-cancel". |
| ifEarlyUL | This IE shall be present if the Early UL Sync Configuration IE or the Early UL Sync Configuration for SUL IE is present. |

[Unchanged text skipped]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

[snip]

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-FiveGProSeLayer3MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,

id-FiveGProSeLayer2MHRemote,

id-RemoteUELocalIDToAdd,

id-RemoteUELocalIDToDelete,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

[Unchanged text skipped]

PC5RLCChannelID ::= INTEGER (1..512, ...)

PC5RLCChannelQoSInformation ::= CHOICE {

pC5RLCChannelQoS				QoSFlowLevelQoSParameters,

pC5ControlPlaneTrafficType		ENUMERATED {srb1,srb2,...,srb0},

choice-extension				ProtocolIE-SingleContainer { { PC5RLCChannelQoSInformation-ExtIEs} }

}

[Unchanged text skipped]

PC5RLCChannelToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeModifiedItem

PC5RLCChannelToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

pC5RLCChannelQoSInformation		PC5RLCChannelQoSInformation			OPTIONAL,

rLCMode							RLCMode			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RemoteUELocalIDToAdd		CRITICALITY ignore		EXTENSION RemoteUELocalID		PRESENCE optional }|

{ ID id-RemoteUELocalIDToDelete		CRITICALITY ignore		EXTENSION RemoteUELocalID		PRESENCE optional },

...

}

[Unchanged text skipped]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-FiveGProSeLayer3MHUEtoNetworkRelay				ProtocolIE-ID ::= 996 – to be assigned by MCC

id-FiveGProSeLayer2MHUEtoNetworkRelay				ProtocolIE-ID ::= 997 – to be assigned by MCC

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay	ProtocolIE-ID ::= 998 – to be assigned by MCC

id-FiveGProSeLayer2MHRemote							ProtocolIE-ID ::= 999 – to be assigned by MCC

id-RemoteUELocalIDToAdd								ProtocolIE-ID ::= xxx

id-RemoteUELocalIDToDelete							ProtocolIE-ID ::= yyy

-----------------End of the First Change---------------