3GPP TSG-RAN WG3 Meeting #128	R3-253159

Malta, MT, 19 – 23 May 2025

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Further discussion on Continuous MDT Collection

Document for:	Discussions & Approval

1	Introduction

SA5 has responded to RAN3’s LS regarding continuous MDT collection. In the reply LS [1], SA5 provided several guidelines addressing the questions raised by RAN3 and also posed some additional questions for clarification.

In this contribution, we present our understanding of SA5’s input and outline potential solutions for enabling continuous MDT collection.

2	Discussion

For Measurement Continuity Problem, regarding RAN3 agreements:

RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.”

Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.”

Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

For Trace Correlation Problem,

RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping.

SA5 provided several clarifications in response to RAN3's LS and also raised a few questions that require further clarification. The details are as follows:

2.1 Measurement Continuity

Regarding the Measurement Continuity issue, SA5 has confirmed that it is feasible for OAM to provide the necessary configuration mechanisms to the RAN in order to identify Management-based MDT sessions intended for Continuous MDT. RAN3 may further specify the detailed configuration parameters as part of OAM requirements.

In alignment with this, RAN3 has also agreed that OAM is responsible for configuring Management-based MDT for Continuous MDT purposes towards the NG-RAN.

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Therefore, it is essential that the Management-based MDT configuration delivered by OAM to the NG-RAN includes an explicit indication for Continuous MDT collection, clearly signaling that the MDT session is intended for continuous data collection.

A straightforward solution to address the measurement continuity problem is for OAM to include a dedicated parameter in the MDT configuration indicating that the session is intended for continuous MDT collection when configuring the management-based solution for RAN.

The proposed approach introduces a “Continuous MDT” indication within the MDT configuration. Additionally, a measurement duration parameter may optionally be included to define the active period of the session.

The procedure proceeds as follows:

- Step 1: OAM configures the management-based MDT, including both Immediate MDT and Logged MDT, with the explicit “Continuous MDT” indication.

- Step 2: While the UE is in a connected state, Immediate MDT measurements are performed, and the NG-RAN node reports the collected data to the TCE.

- Step 3: When the UE transitions to an idle or inactive state, it begins Logged MDT measurement collection.

- Step 4: Upon re-entry into the connected state, the UE sends the logged measurement report, with the Continuous MDT indication, to the (new) serving NG-RAN node.

- Step 5: The (new) NG-RAN node, recognizing that the UE is configured for Continuous MDT and report to the OAM. Then OAM re-applies the previously assigned MDT configuration to NG-RAN and NG-RAN selects the same UE.

- Step 6: Once the configured measurement duration expires, the Continuous MDT session is terminated.

- To resolve Problem A (Measurement Continuity), it is proposed to introduce a “Continuous MDT” parameter in the MDT configuration, with an optional measurement duration to control the continuous MDT collection session.

Importantly, SA5 has agreed that the proposed solution should minimize any impacts on the existing Management-Based MDT framework, ensuring backward compatibility and avoiding unnecessary complexity in existing implementations.

SA5 has requested clarification regarding the specific issue referred to as the “Measurement Continuity Problem across different RRC states.” In response, RAN3 refers to the description provided in TR 38.743, which outlines the issue as follows:

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

- Respond to SA5 by clarifying that the Measurement Continuity Problem across different RRC states is as described above in TR 38.743.

Both SA5 and RAN3 have reached a common understanding that the solution to support Continuous MDT collection must minimize impacts on the existing Management-Based MDT framework.

However, in contrast to this principle, some companies have proposed a CN-based solution in which the AMF is responsible for allocating a temporary indication for the purpose of maintaining measurement continuity. Specifically, the AMF would assign a persistent or random UE ID that is passed to the RAN, and store the corresponding MDT configuration.

While this approach may offer a means to track the same UE across states, it introduces several significant concerns:

- Increased complexity and burden on the CN: The CN would be required to consistently allocate and manage a persistent UE ID for each participating UE, adding complexity to core network procedures for management-based MDT.

- Limited applicability in inter-AMF handover scenarios: Persistent UE IDs may not be maintained across AMF boundaries, thereby undermining the objective of continuous measurement continuity in such scenarios.

Therefore, the CN-based approach deviates from the agreed-upon principle of minimizing the impact on the Management-Based MDT framework, and may not be a suitable direction for standardization. Taking the limited time in R19 into account, we propose to adopt the solution with minized standard impact.

Observation 1: AMF-based solution, is not an ideal approach for resolving the issue of continuous data collection due to its significant burden on the CN and its infeasibility in scenarios involving inter-AMF handovers.

- It is proposed to adopt the solution with minimized standard impact to support continuous MDT collection in R19.

2.2 Trace Correlation

In the reply LS, SA5 agrees that at least TR and TRSR—both of which do not expose UE identity—can be used for correlating management-based MDT reports at the TCE.

From our perspective, if TR and TRSR can be used to link multiple management-based MDT reports, and if the TCE can further utilize the timestamp and the “Continuous MDT” indication embedded in the measurement reports, then it becomes feasible to associate multiple measurement reports from the same UE across different RRC states.

This approach represents the least intrusive solution to the existing Management-Based MDT framework, as it does not rely on persistent UE identifiers from the CN and avoids introducing new core network procedures. It aligns fully with the shared goal of both SA5 and RAN3 to minimize the impact while enabling measurement continuity.

- Following the SA5’s guidelines, RAN3 agrees to re-using TR and TRSR to resolve the Trace Correlation Problem while the details on configuration can be left to SA5.

Agree the Reply LS to SA5 [2].

3	Conclusion

Following is our proposals and observations:

- To resolve Problem A (Measurement Continuity), it is proposed to introduce a “Continuous MDT” parameter in the MDT configuration, with an optional measurement duration to control the continuous MDT collection session.

- Respond to SA5 by clarifying that the Measurement Continuity Problem across different RRC states is as described above in TR 38.743.

Observation 1: AMF-based solution, is not an ideal approach for resolving the issue of continuous data collection due to its significant burden on the CN and its infeasibility in scenarios involving inter-AMF handovers.

- It is proposed to adopt the solution with minimized standard impact to support continuous MDT collection in R19.

- Following the SA5’s guidelines, RAN3 agrees to re-using TR and TRSR to resolve the Trace Correlation Problem while the details on such TR and TRSR configuration can be left to SA5.

4	Reference

[1] S5-252114, Reply LS to LS on Continuous MDT, SA5

[2] R3-253517, [Draft] Reply LS to SA5 on Continuous MDT Collection, ZTE Corporation