3GPP TSG-RAN WG3 Meeting #128                                                              R3-253168

Malta, MT, 19 - 23 May, 2025

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	Remaining issues for WAB

Document for:		Discussion

# 1	Introduction

This contribution discusses remaining issues related to WAB:

- Avoiding multi-hop topology during WAB-node access

- Connecting to gNBs that support resource coordination for inband operation.

# 2	Discussion

## 2.1	Avoiding multi-hop topology during WAB-node access

RAN3 has been discussing means to avoid multi-hop WAB topologies for a few meetings. To avoid multi-hop WAB topology resulting from WAB-MT handover, RAN3 agreed in RAN3#126:

Agree on Solution 2: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

To avoid multi-hop WAB topology during WAB-node access, the following two solutions have been discussed:

- Solution 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

- Solution 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

In the last meeting, RAN3 established the following understanding:

Solution 1 is implementation specific and it is not subject to specifications changes. There is no consensus in RAN3 on an agreement for solution 3 and make the decision in next meeting. To be continued...

The following observations can be made on this discussion:

Observation 1-1: Solution 1 provides a mechanism to avoid multi-hop topologies during WAB-node access.

Observation 1-2: For out-of-band operation, WAB-MT will always identify BH RAN cells based on frequency band, i.e., apply solution 1.

Observation 1-3: For in-band operation, the WAB-MT needs to connect to a cell that supports resource coordination, which automatically preclude WAB-gNBs, and which cannot be indicated by Solution 3.

Observation 1-4: Solution 3 is therefore useless.

Proposal 1: RAN3 to discontinue discussion on Solution 3 for multi-hop prevention since (1) it is suboptimal for out-of-band operation and (2) insufficient for in-band operation as it does not indicate the BH gNBs that provide resource coordination.

## 2.2	Connecting to gNBs that support resource coordination

For inband operation, the WAB-MT needs to connect to a BH gNB that supports resource coordination. It is possible, of course, to enhance all BH gNBs with support for WAB-related resource coordination. This, however, may not be cost-effective. It therefore needs to be assumed that not all gNBs have been upgraded to WAB-related resource coordination.

The following aspects need to be supported:

1.) During cell (re)selection and/or initial access, the WAB-MT needs to be able to identify whether a BH gNB supports WAB-related resource coordination.

2.) A BH gNB with support for WAB-related resource coordination also needs to know whether a neighbour node supports WAB-related resource coordination, so that it can make appropriate WAB-MT handover decisions.

Both of these aspects need to be supported. Otherwise, the WAB-MT might end up at a non-WAB-enhanced BH gNB which neither provides WAB-related resource coordination nor performs a handover of the WAB-MT to a WAB-enhanced BH gNB since it is not simply not WAB-aware.

Proposal 2-1: RAN3 to discuss how the WAB-MT identifies a BH gNB that supports WAB-related resource coordination during cell (re)selection and/or initial access,.

Proposal 2-2: RAN3 to discuss how a BH gNB with support of WAB-related resource coordination knows whether neighbour nodes also support WAB-related resource coordination, so that it can perform appropriate WAB-MT handover.

The following options can be considered:

Option 1: WAB-nodes and WAB-enhanced BH gNBs are configured with BH gNB IDs that support WAB resource coordination.

Option 2: BH gNBs indicate support for WAB-related resource coordination OTT for WAB-MT’s cell (re)selection and/or initial access.

Option 3: BH gNBs exchange information about WAB-related resource coordination via Xn.

RAN3 should discuss these options.

Proposal 2-3: RAN3 to consider the following options:

- Option 1: WAB-nodes and WAB-enhanced BH gNBs are configured with BH gNB IDs that support WAB resource coordination.

- Option 2: BH gNBs indicate support for WAB-related resource coordination OTT.

- Option 3: BH gNBs exchange information about WAB-related resource coordination via Xn.

# 3	Conclusion

This contribution discussed remaining issues related to WAB. The following observations and proposals have been made:

On avoidance of multi-hop topology during WAB-MT access:

Observation 1-1: Solution 1 provides a mechanism to avoid multi-hop topologies during WAB-node access.

Observation 1-2: For out-of-band operation, WAB-MT would always select BH RAN cells based on frequency band, i.e., apply solution 1.

Observation 1-3: For in-band operation, the WAB-MT needs to connect to a cell that supports resource coordination which automatically preclude WAB-gNBs, and which cannot be indicated by Solution 3.

Observation 1-4: Solution 3 is therefore useless.

Proposal 1: RAN3 to discontinue discussion on Solution 3 for multi-hop prevention since (1) it is suboptimal for out-of-band operation and insufficient for in-band operation as it does not indicate the BH gNBs that provide resource coordination.

On connecting to gNBs that support resource coordination for in-band operation:

Proposal 2-1: RAN3 to discuss how the WAB-MT identifies a BH gNB that supports WAB-related resource coordination during cell (re)selection and/or initial access,.

Proposal 2-2: RAN3 to discuss how a BH gNB with support of WAB-related resource coordination knows whether neighbour nodes also support WAB-related resource coordination, so that it can perform appropriate WAB-MT handover.

Proposal 2-3: RAN3 to consider the following options:

- Option 1: WAB-nodes and WAB-enhanced BH gNBs are configured with BH gNB IDs that support WAB resource coordination.

- Option 2: BH gNBs indicate support for WAB-related resource coordination OTT.

- Option 3: BH gNBs exchange information about WAB-related resource coordination via Xn.