3GPP TSG-RAN WG3 #128	R3-253196

St Julian, Malta, May 19th – 23rd 2025

Agenda Item:	10.4

Source:	Ericsson

Title:	(TP for BL CR to 38.413 for SON) UHI for SCG activation and deactivation

Document for:	Discussion, Decision

# 1	Introduction

In the Rel-19 SON/MDT Work Item, MHI enhancement for SCG deactivation/activation is listed as one of the objectives for the Work Item. Both a UE based and a network based solution has been discussed, and in RAN3#126 it was agreed to support a network-based solution for enhancing PSCell activation/deactivation. Questions left for further study were whether an absolute time or a relative time spent in the PSCell with SCG state set to activated should be logged, and whether any additional information would be needed. In RAN3#127 and RAN3#127-bis, the topic of UHI for SCG activation and deactivation was not discussed, as RAN3 awaited RAN2 progress on whether to use absolute time or relative time to express the time spent in the PSCell with SCG state set to activated. In RAN2#129-bis it was agreed to log the absolute time the UE has spent in the PSCell with SCG state set to activated in the MHI.

In this contribution we continue the discussion on UHI for SCG activation and deactivation, and provide a text proposal on how to log the time spent in a PSCell with SCG state in the UHI.

# 2	Discussion

The history information on visited cells, logged by the UE in the Mobility History Information (MHI), and by the network in the UE History Information (UHI), helps the network to understand the movement of the UE and identify ping-pong or coverage issues in the network. From Rel-17, not only time spent in the PCell, but also time spent in the PSCell is logged in the history information. In Rel-17, SCG activation and deactivation was also introduced, which enables power saving while still allowing fast access to the SCG when needed.

Currently there is no support in the history information (UHI or MHI) for tracking how often or how long the SCG state has been set to activated or deactivated. Such information could be used by the network to perform mobility optimization as it tracks the actual usage of the SCG leg. It could also play an important role for traffic and pattern analysis, and the MN could track whether a particular SCG related PSCell is configured often or is seldom used. In addition to this, the information on SCG activation/deactivation could play an important role for mobility and traffic prediction in future AI/ML applications.

To increase the understanding of the secondary cell usage, information on the time with SCG state set to activated could be logged in the UE history information (UHI) for each PSCell entry.

Logging the time with SCG state set to activated in the UHI would increase the understanding of the secondary cell usage.

Historically, the UHI and MHI have been designed in analogy, i.e. logging the corresponding values from UE and network point of view respectively, in MHI and UHI. We propose to follow this principle also for the additions made for SCG activation and deactivation.

Historically, the UHI and MHI have been designed in analogy, i.e. logging the corresponding values from UE and network point of view respectively, in MHI and UHI.

In RAN2#129-bis, the following was agreed:

UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

During previous RAN3 meetings, RAN3 agreed to wait for RAN2 decision, the main reason being the advantage of keeping MHI and UHI aligned. Therefore, it is recommended that RAN3 follows the RAN2 technical solution agreed for MHI in order to keep this alignment.

As a reminder of the technical discussion, and in addition to the analogy between MHI and UHI, several benefits from logging the absolute time compared to the percentage of time with SCG state set to activated can be seen. Below a discussion on the pros and cons of the two options is given.

Percentage of time

Logging the percentage of time spent in the PSCell with SCG state set to activated would give an understanding of how large part of the total time spent in the PSCell that the SCG state was set to activated. To be able to derive the value, the network needs to keep track of the total time spent in the PSCell. This is logged for the UHI and included in the field in the field Time Stay in the Last Visited PSCell Information, but since the Time Stay is capped to 4095 s, it cannot be assumed that a current network implementation will continue running a timer longer than that. Logging the percentage of time spent in the PSCell with SCG state set to activated in UHI would impose a requirement on the network to continue counting the time spent in the PSCell with no upper limit. Further, since Time Stay is capped, the time spent in the PSCell will be unknown to the node receiving the UHI in case it is above 4095 s. Therefore, the absolute time spent in the PSCell with SCG state set to activated can also not be derived based on the logged values.

Using the percentage of time with SCG activated in UHI would impose a requirement on the network to continue counting the time spent in the PSCell after 4095 s.

If the time with SCG activated is logged as a percentage in UHI, the absolute time spent in the PSCell with SCG state set to activated cannot be derived based on the logged values.

Another implication of using percentage is that the granularity required from the timer capturing the time spent in the PSCell will be different depending on whether the UE has stayed for a long time or a short time in the PSCell. This is easiest illustrated by some examples as listed in the table below. In the example, we assume a granularity of 1% for the logged percentage of time spent in the PSCell with SCG state set to activated, but similar differences would arise also with other granularities.

| Case | Time spent in PSCell (s) | Time with SCG state set to activated (s) | Percentage of time with SCG activated (%) |
| --- | --- | --- | --- |
| A | 4095 | 21 | 1 |
| B | 4095 | 61 | 1 |
| C | 1 | 0,014 | 1 |
| D | 1 | 0,015 | 2 |

As can be seen, for a long stay in the PSCell (4095 s), a time with SCG state set to activated of 21 s (case A) and a time with SCG state set to activated of 61 s (case B) both results in a percentage value of 1%, although the time with SCG state set to activated is almost three times as high in case B than in case A. For a longer time spent in the PSCell the difference in time in SCG activated could be even larger and still result in the same percentage value.

In case of a short stay (1 s) in the PSCell on the other hand, a time with SCG state set to activated of 14 ms (case C) and a time with SCG state set to activated of 15 ms (case D) will give different percentage values (1% and 2% respectively), even though the difference between the time with SCG state set to activated is as small as 1 ms. This means, that in case of a short stay in the PSCell, a granularity of 1% for the percentage value would require a precision of milliseconds to log the correct percentage of time with SCG state set to activated in the UHI.

Hence from the above examples it is clear that depending on the total “time stay”, the percentage may in some cases give very inaccurate information (as in the examples A and B above), and in some other cases unnecessarily accurate information (as in the examples C and D above).

The granularity needed from a timer measuring the time with SCG activated would be dependent on the time spent in the PSCell in case the percentage of time with SCG activated is to be logged in UHI. In some cases, the percentage would give very inaccurate information (as it is in the examples A and B above), and in some other cases, it will give unnecessarily accurate information (as it is in the examples C and D above).

Absolute time

Logging an absolute time with SCG state set to activated would give a more “objective” understanding of whether the time spent in the PSCell with SCG state set to activated is short or long. If the total time spent in the PSCell is less than 4095 s, how large part of the total time spent in the PSCell that the SCG state was set to activated will also be possible to derive. For time spent in the PSCell equal to or longer than 4095 s however, the percentage of time with SCG state set to activated will be unknown. With this solution, no changes are needed to the requirements on the existing time spent in PSCell timer, and the granularity needed from a timer measuring the time with SCG state set to activated is independent of the length of the stay in the PSCell.

If the absolute time spent with SCG activated is logged in UHI, the percentage of time spent with SCG activated is possible to derive from the logged values only if the total time spent in the PSCell is less than 4095 s.

Using the absolute time with SCG activated in UHI does not change the requirements on the existing time spent in PSCell timer.

The granularity needed from a timer measuring the time with SCG state set to activated is independent of the length of the stay in the PSCell.

The implications of the two different options are captured in the below table.

|  | Percentage of time | Absolute time |
| --- | --- | --- |
| Requirement on the network to continue counting the time spent in the PSCell after 4095 s. | Yes | No |
| Granularity needed from a timer measuring the time with SCG state set to activated | Depends on time spent in PSCell. | Independent of time spent in PSCell. |
| Possible to understand percentage of time with SCG state set to activated. | Yes. | Only if time spent in PSCell < 4095 s. |
| Possible to understand absolute time with SCG state set to activated. | Only if time spent in PSCell < 4095 s. | Yes, with the exception of times above a possible cap of the value. |

Based on the above reasoning, our view is that logging the absolute time with SCG state set to activated is to be preferred over logging the percentage of time with SCG state set to activated.

For each PSCell entry of the UHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

In RAN3#125-bis, it was proposed in [1] to also include UE average throughput over SCG in the UHI. But since the UE average throughput can depend on multiple factors, such as UE capability, load, and radio conditions, this is not useful information for mobility optimization or traffic and pattern analysis, which are the main use cases for MHI/UHI. It is also not clear how collection of the UE average throughput is linked to the SCG activation and deactivation; hence the collection of UE average throughput is out of scope of the objective on MHI Enhancement for SCG Deactivation/Activation, and should not be considered within this Work Item.

Collection of UE average throughput is out of scope of the Rel-19 SON/MDT Work Item.

A Text Proposal implementing Proposal 1 is provided in Appendix A below.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Logging the time with SCG state set to activated in the UHI would increase the understanding of the secondary cell usage.

Observation 2	Historically, the UHI and MHI have been designed in analogy, i.e. logging the corresponding values from UE and network point of view respectively, in MHI and UHI.

Observation 3	Using the percentage of time with SCG activated in UHI would impose a requirement on the network to continue counting the time spent in the PSCell after 4095 s.

Observation 4	If the time with SCG activated is logged as a percentage in UHI, the absolute time spent in the PSCell with SCG state set to activated cannot be derived based on the logged values.

Observation 5	The granularity needed from a timer measuring the time with SCG activated would be dependent on the time spent in the PSCell in case the percentage of time with SCG activated is to be logged in UHI. In some cases, the percentage would give very inaccurat information (as it is in the examples A and B above), and in some other cases, it will give unnecessarily accurate information (as it is in the examples C and D above).

Observation 6	If the absolute time spent with SCG activated is logged in UHI, the percentage of time spent with SCG activated is possible to derive from the logged values only if the total time spent in the PSCell is less than 4095 s.

Observation 7	Using the absolute time with SCG activated in UHI does not change the requirements on the existing time spent in PSCell timer.

Observation 8	The granularity needed from a timer measuring the time with SCG state set to activated is independent of the length of the stay in the PSCell.

Observation 9	Collection of UE average throughput is out of scope of the Rel-19 SON/MDT Work Item.

Based on the discussion in the previous section we propose the following:

Proposal 1	For each PSCell entry of the UHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

# References

R3-245665, (TP to BL CR for TS38.413, TS38.300, TS38.423) Rel-18 leftovers, RAN3, 3GPP TSG-RAN WG3 Meeting #125-bis, Hefei, China, 14th - 18th Oct 2024

# Appendix A - Text Proposal to 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Overall Activation Timer | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell while the SCG was in active state, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | YES | ignore |

[Unchanged parts omitted]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

[Unchanged parts omitted]

id-NR-PagingeDRXInformation,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-OnboardingSupport,

id-OverallActivationTimer,

id-PagingAssisDataforCEcapabUE,

[Unchanged parts omitted]

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-OverallActivationTimer	CRITICALITY ignore	EXTENSION OverallActivationTimer	PRESENCE optional },

...

}

[Unchanged parts omitted]

OverallActivationTimer ::= INTEGER (0..40950)

[Unchanged parts omitted]

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-OverallActivationTimer								ProtocolIE-ID ::= xxx

[End of changes]