3GPP TSG-RAN WG3 #128	R3-253198

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:	   13.2 Support for inter-CU LTM

Title:	(TP to BL CR for TS 38.423 and 38.473 on Inter-CU LTM) Rel-19 inter-CU LTM issues

Source:	NEC

Document for:	   Discussion and Decision

# 1. Introduction

For the Rel-19 Inter-CU LTM, so far RAN3 has progressed quite well. The TP was reflected and merged into R3-252517(BL to 38.300), R3-252520 (BL CR to 38.423) and R3-252522 (BL CR to 38.473).

This contribution continues to discuss and proposal to progress the project.

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-250339 (target: RAN #109) [TU: 1 ( 1, 1, 0.5)] QUOTA: 3

13.2. Support for inter-CU LTM Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: As secondary priority, support the case where CU is acting as SN and MN is unchanged As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support RAN3#127bis: Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM. The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: Candidate Cell ID TA value Preamble index RA-RNTI FFS on the XnAP IDs. Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB. For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB. Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s). Include the following information in SN Change Required (Source SN to MN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for each candidate SN (FFS) CSI Resource configuration request (explicit way) Early sync information request (explicit way) Include the following information in SN Addition Request (MN to Candidate SN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for the candidate SN (explicit way) CSI Resource configuration request (explicit way) Early sync information request (explicit way) It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message. For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN. The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN. Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message. Continue to work on the solutions comparison and details…

2. Discussion

2.1	Security information in F1AP

The security handling for the inter-CU LTM in RAN3#127bis has reached agreement that “The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.”, this is due to the agreement in RAN2 that the NCC is to be included in the LTM cell switch command MAC CE. This is further confirmed in the incoming RAN2 LS (R3-253009) that showed:

“NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.”.

A remaining issue is “Whether to define additional information from CU to DU to deliver NCC value.”, as in the incoming RAN2 LS has shown that if the R19 set ID is same for both target and source cells, the NCC will not be included, consequently, the R19 set ID that corresponds to a set of cells, need to indicate from gNB-CU to gNB-DU, in order for the gNB-DU to judge whether to include NCC in LTM cell switch command MAC CE, therefore it would be feasible for the gNB-CU to indicate the R19 Set ID to the gNB-DU.

The RAN2 LS shows further that:

“The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update.”

It can then include the R19 Set ID (the “ltm-NoSecurityChangeID” in RRC) in the added LTM Security Information IE in the UE Context Modification Request message.

Proposal 1: To include the R19 Set ID (ltm-NoSecurityChangeID in RRC) in the added LTM Security Information IE in the UE Context Modification Request message., from gNB-CU to gNB-DU so for the gNB-DU to judge whether to include NCC in LTM cell switch command MAC CE.

2.2	Reference configuration in XnAP

For the reference configuration, it has been agreed in RAN3#127 that:

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

The open issue “FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.” is still not solved yet in RAN3#127bis and still FFS.

It would be generally to have a good understanding that the source gNB is the one who control the handover, and for the inter-CU LTM the similar principle can be applied. Therefore “For inter-CU LTM, only the source gNB can provide the reference configuration” is a good approach. Then for the question whether the source gNB can request a candidate gNB to provide a reference configuration, based on the principle that source gNB is the one who control the whole handover, it would be feasible that the source gNB can request a candidate gNB to provide a reference configuration.

Proposal 2: It can let the source gNB-CU to be a center control so can request candidate gNBs to provide a reference configuration, then the source gNB can provide the selected reference configuration.

2.3	TA Information Transfer in XnAP

After some meetings discussion for the TA information transfer, and finally agreed that “a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.”, this is following the Rel-18 intra-CU LTM aspect. It would not be wise to extend more attempts to apply per UE specific in a non-UE associated signalling procedure, therefore it would be good to keep the consistency with intra-CU LTM, only to have non-UE associated signalling procedure to transfer the TA information, with NO per UE specific, which is then per DU under the source CU.

So for the “FFS on the XnAP IDs.”, we would propose to remove such FFS and close this issue.

Proposal 3: For the inter-CU TA information transfer, only to have non-UE associated signalling procedure to transfer the TA information, with NO per UE specific, which is then per DU under the source CU.

For the detail information element in the TA Information Transfer message, still need to discuss.

The allocated Early UL Sync Configuration is valid for a list of DU in the source gNB-CU, and the Preamble Index List is similar to intra-CU LTM case that it is valid for a DU. It has been agreed in last RAN3#127bis meeting to have

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

Additionally the information in the Handover related messages and LTM configuration Update related message each is per UE associated, the candidate CU can easily identify the source DU when receive the information from the candidate DUs in F1AP. The information to be in the XNAP TA Information Transfer message, can be same as what in the existing F1AP e.g. DU-CU TA Information Transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- Tag ID Pointer

- source gNB-DU ID

The baseline CR of 38.423 has included Candidate Cell ID, TA Value, Preamble Index and RA-RNTI, other information (Tag ID Pointer, source gNB-DU ID) are also needed.

Proposal 4: The information to be in the XNAP TA Information Transfer message, it is proposed to include also Tag ID Pointer and source gNB-DU ID which is same as what in the existing F1AP e.g. DU-CU TA Information Transfer message. The TP is provided.

2.4	Handling of multiple UE associations in XnAP

Last RAN3#127bis meeting finally agreed that “use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.”, therefore there is no additional UE needed.

For such use of a single UE association for multiple LTM handover request, is actually same as condition handover, and the procedure text in 8.2.1.1 General chapter has already indicate the same for LTM.

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

It is then already covered by the procedure text.

Observation 1:  For use of a single UE association for multiple LTM handover request, is actually same as condition handover, and the procedure text in 8.2.1.1 General chapter has already indicate the same for LTM. Therefore no need to add more text in the procedure text.

For the purpose of subsequent LTM i.e. after a candidate gNB become the serving gNB, it will need to sync up configurations among candidate gNBs, including early sync configuration, configuration ID, and data forwarding addresses, etc. In order for the candidate gNBs to know/identify the UE context that has been prepared with old serving gNB, as agreed that “use the old target UE XnAP ID for each candidate cell(s) to the corresponding candidate gNB.”.

The next question is “how to deliver the candidate UE XnAP ID(s) to the new serving gNB”.

Considering the candidate CUs may be newly setup or release due to several different situations, i.e. it would then be easy to just transfer the candidate UE XnAP ID(s) to the new serving gNB in one shot, i.e. after each inter-CU LTM cell switch. However there can be a discussion whether feasible to use Cell Switch Notification procedure from old serving gNB to target gNB. Considering that the cell switch may be failed and the handover may fallback, it would be feasible to deliver the candidate UE XnAP ID(s) to the new serving gNB after the old serving gNB receive Handover Success from the target gNB (become new serving gNB). We propose to use LTM Configuration Update procedure to deliver the candidate UE XnAP ID(s) to the new serving gNB.

Proposal 5: For the subsequent inter-CU LTM, the old serving gNB deliver to new serving gNB the old target UE XnAP ID for each candidate cell(s) to the corresponding candidate gNB by the LTM Configuration Update procedure, after the old serving gNB has received Handover Success from the new serving gNB.

Regarding, how to let the new serving gNB know the candidate gNBs, it may be simply just to give the Global gNB-ID information.

Proposal 6: The Global gNB-ID of candidate gNBs is giving in the LTM Configuration Update procedure for the new serving gNB to identify the candidate gNBs.

Candidate gNBs List: (from 1..max)

> Global gNB ID

> old target UE XnAP ID for each candidate gNB

2.5	SP CSI-RS configuration

For the Semi Persistence CSI-RS configuration, last RAN3#127bis we agreed to introduce new procedure in F1 and Xn to activate/deactivate the CSI-RS resource. The remaining open point is the granularity of the CSI-RS resource.

Open issues for the next meeting: Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

Let see first the RAN2#129bis progress (agreement, refer to RAN2#129bis report):

Agreements on L1 event-triggered MR 1. Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. 2. The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id. …..

and the running CR on the related information i.e. the LTM-CSI-ResourceConfig

LTM-CSI-ResourceConfig The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations LTM-CSI-ResourceConfig information element -- ASN1START -- TAG-LTM-CSI-RESOURCECONFIG-START LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-SSB-ResourceSet-r18 LTM-SSB-ResourceSet-r18, ... , [[ ltm-NZP-CSI-RS-ResourceSet-r19 LTM-NZP-CSI-RS-ResourceSet-r19 OPTIONAL, -- Need R resourceType-r19 ENUMERATED {periodic, semi-persistent} OPTIONAL -- Cond CSI-RS ]] } LTM-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... } LTM-NZP-CSI-RS-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18, ... } -- TAG-LTM-CSI-RESOURCECONFIG-STOP -- ASN1STOP

Then following the RAN2 progress, we can consider two alternatives on the granularity of the CSI-RS resource/resource set to be activated/deactivated.

Alt 1: use of LTM-CSI-ResourceConfigId-r18

Alt 2: use of CSI-RS-ResourceId

The difference of Alt. 1 from Alt.2 is that, using one LTM-CSI-ResourceConfigId-r18 ID (Alt.1) can activate/deactivate the multiple CSI-RS resource at the same time, instead of using multiple CSI-RS-ResourceId (Alt.2) that needs to indicate multiple ID, therefore we would see the Alt.1 has benefit over Alt.2.

Proposal 7: For the granularity of CSI-RS resource/resource set to be activated/deactivated for the SP CSI-RS coordination, it is proposed to use LTM-CSI-ResourceConfigId-r18 as defined in RRC.

For reference: LTM-CSI-ResourceConfigId information as defined in RRC

LTM-CSI-ResourceConfigId information element -- ASN1START -- TAG-LTM-CSI-RESOURCECONFIGID-START LTM-CSI-ResourceConfigId-r18 ::= INTEGER (0..maxNrofLTM-CSI-ResourceConfigurations-1-r18) -- TAG-LTM-CSI-RESOURCECONFIGID-STOP -- ASN1STOP maxNrofLTM-CSI-ResourceConfigurations-1-r18 INTEGER ::= 111 -- Maximum number of LTM CSI resource configurations minus 1

Again following the RAN2 progress, there is another information that RAN3 need to discuss, i.e. the TCI-State ID associated with the CSI-RS resource to be activated/deactivated.  RAN3 need to decide whether the source gNB or candidate gNB to decide the TCI state associated with CSI-RS resource. In either case, the node that decide the TCI state associated with the CSI-RS resource, need to inform the other node.

Proposal 8: It is proposed to discuss and decide whether source gNB or candidate gNB to decide the TCI-Stage associated with the CSI-RS resource to be activated/deactivated. If candidate gNB to decide then need to transfer the TCI State information to the source gNB, and vice-versa.

3. Conclusion and proposals

Proposal 1: To include the R19 Set ID (ltm-NoSecurityChangeID in RRC) in the added LTM Security Information IE in the UE Context Modification Request message., from gNB-CU to gNB-DU so for the gNB-DU to judge whether to include NCC in LTM cell switch command MAC CE.

Proposal 2: It can let the source gNB-CU to be a center control so can request candidate gNBs to provide a reference configuration, then the source gNB can provide the selected reference configuration.

Proposal 3: For the inter-CU TA information transfer, only to have non-UE associated signalling procedure to transfer the TA information, with NO per UE specific, which is then per DU under the source CU.

Proposal 4: The information to be in the XNAP TA Information Transfer message, it is proposed to include also Tag ID Pointer and source gNB-DU ID which is same as what in the existing F1AP e.g. DU-CU TA Information Transfer message. The TP is provided.

Observation 1:  For use of a single UE association for multiple LTM handover request, is actually same as condition handover, and the procedure text in 8.2.1.1 General chapter has already indicate the same for LTM. Therefore no need to add more text in the procedure text.

Proposal 5: For the subsequent inter-CU LTM, the old serving gNB deliver to new serving gNB the old target UE XnAP ID for each candidate cell(s) to the corresponding candidate gNB by the LTM Configuration Update procedure, after the old serving gNB has received Handover Success from the new serving gNB.

Proposal 6: The Global gNB-ID of candidate gNBs is giving in the LTM Configuration Update procedure for the new serving gNB to identify the candidate gNBs.

Proposal 7: For the granularity of CSI-RS resource/resource set to be activated/deactivated for the SP CSI-RS coordination, it is proposed to use LTM-CSI-ResourceConfigId-r18 as defined in RRC.

Proposal 8: It is proposed to discuss and decide whether source gNB or candidate gNB to decide the TCI-Stage associated with the CSI-RS resource to be activated/deactivated. If candidate gNB to decide then need to transfer the TCI State information to the source gNB and vice-versa.

# References

[1] RP-250339 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-252320 Summary of offline discussion on MobilityEnh

[3] R3-252516 => R3-253084 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

[4] R3-251555 => R3-253085 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[5] R3-252517 => R3-253086=> R3-253184 (BL CR to 38.300) Inter-CU LTM

[6] R3-252518 => R3-253087 (BL CR to 38.401) on conditional intra-CU LTM

[7] R3-252519 => R3-253088 (BL CR to 38.420) Support for Inter-CU LTM

[8] R3-252520 => R3-253089 (BL CR to 38.423) Xn Support for Inter-CU LTM

[9] R3-252521 => R3-253091 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

[10] R3-252541 => R3-253090 (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC

[11] R3-252522 => R3-253092 => R3-253149 (BL CR to 38.473) Inter-CU LTM

[12] R3-253009 (==R2-2503012) LS on RAN2 agreements for security key handling in inter-CU LTM

[13] R3-252431 (TP for LTM BL CR for TS 38.423) – Further agreements on inter-CU LTM

[14] R3-252463 (TP for LTM BLCR for TS38.473) Update on inter-CU LTM procedure

[15] R2-2502534 Running RRC CR for inter-CU and conditional LTM

[16] R2-250xxxx (For RAN2#130) Running RRC CR for inter-CU and conditional LTM

[17] Draft_RAN2_129bis_Meeting_Report_v3

(TP to TS 38.423 on Inter-CU LTM) Inter-CU LTM on top of R3-252520

8.2.1	Handover Preparation

8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

Skip irrelevant part

If the DL LBT Failure Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the DL LBT failure information of the UE in the target cell for MRO analysis, as specified in TS 38.300 [9].

If the LTM Information Request IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node and include the LTM Information Response IE in the LTM Handover Information Response IE included in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration IE is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for generating the LTM configuration.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI Resource Configuration IE in the LTM Information Response IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM-CandidateConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Request for Reference Configuration IE is contained in the LTM Information Request IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the Reference Configuration IE contained in the LTM Information Response included in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover via the Data Collection Reporting procedure and according to clause 8.4.13.2, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node,.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.x	Procedures for L1/L2 Triggered Mobility

8.x.1	LTM Configuration Update

Editor’s note: Details need to be continued.

8.x.1.1	General

The purpose of the LTM Configuration Update procedure is to update LTM configuration data, or to inform the change of serving NG-RAN node and setup the UE associated signalling between the new serving NG-RAN node and all other candidate NG-RAN nodes. The procedure uses UE associated signalling.

8.x.1.2	Successful Operation

Figure 8.x.1.2-1: LTM Configuration Update procedure. Successful operation

The NG-RAN node1 initiates the procedure by sending the LTM CONFIGURATION UPDATE message to NG-RAN node2.

The LTM CONFIGURATION UPDATE message may contain:

-	the LTM Information Request IE;

-	the Early Sync Information Request IE;

-	the Early Sync Information IE;

-	the Early Sync Information for SUL IE;

-    the Data Forwarding Info IE.

If the LTM Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the LTM information update for the UE, include the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message and act as defined in TS 38.300 [9].

If the Reference Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take it into account for generating the LTM configuration.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, include the CSI Resource Configuration IE in the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM-CandidateConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the early synchronization information for the UE in the NG-RAN node2.

If the Early Sync Information Response IE is contained in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, store it and use it as defined in TS 38.300 [9].

If the Early Sync Information IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information to use for subsequent LTM.

If the Early Sync Information for SUL IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information for SUL to use for subsequent LTM.

If the Data Forwarding Info IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall use the information for forwarding data traffic to the NG-RAN node2.

If the Key NG-RAN Star IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall use the information for subsequent LTM.

If the Security Information for LTM (FFS on the name) IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall use the information for subsequent LTM.

If the Candidate gNB Information List IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall use the information for subsequent LTM.

8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: LTM Configuration Update procedure. Unsuccessful Operation

If the NG-RAN node2 cannot accept the update, it shall respond with the LTM CONFIGURATION UPDATE FAILURE message with an appropriate cause value.

8.y.1	Cell Switch Notification

Editor’s note: Details need to be continued.

8.y.1.1	General

The purpose of the Cell Switch Notification procedure is to enable the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source NG-RAN node to the target NG-RAN node. The procedure uses UE-associated signalling.

8.y.1.2	Successful Operation

Figure 8.y.1.2-1: Cell Switch Notification procedure. Successful operation.

The source NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message.

Upon reception of the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE where one cell of the target NG-RAN node is indicated by the included Target Cell Global ID IE.

8.z.1	TA Information Transfer

Editor’s note: Details need to be continued.

8.z.1.1	General

The purpose of the TA Information Transfer procedure is to enable the NG-RAN node1 to send the TA related information to the NG-RAN node2. The procedure uses non UE-associated signalling.

8.z.1.2	Successful Operation

Figure 8.z.1.2-1: TA Information Transfer procedure. Successful operation.

The NG-RAN node1 initiates the procedure by sending a TA INFORMATION TRANSFER message.

8.u.1	LTM Cancel

8.u.1.1	General

The purpose of the LTM Cancel procedure is to enable a target NG-RAN node to cancel an already prepared LTM handover. The procedure uses UE-associated signalling.

8.u.1.2	Successful Operation

Figure 8.z.1.2-1: LTM Cancel procedure. Successful operation.

The target NG-RAN node initiates the procedure by sending the LTM CANCEL message to source NG-RAN node.

8.w.1	CSI-RS Coordination

8.w.1.1	General

The purpose of the CSI-RS Coordination procedure is to enable coordination of CSI-RS transmission. The procedure uses UE-associated signalling.

8.w.1.2	Successful Operation

Figure 8.w.1.2-1: CSI-RS Coordination procedure, successful operation

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1	Messages for Basic Mobility Procedures

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| Skip irrelevant part |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| LTM Handover Information Request | O |  |  |  | YES | reject |
| >LTM Information Request | O |  | 9.2.1.xx1 |  | – |  |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Handover Information Response | O |  |  |  | YES | ignore |
| >LTM Information Response | O |  | 9.2.1.xx2 |  | – |  |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x2		TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Editor’s note: Details on IEs need to be continued.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node to Source NG-RAN node TA Information |  | 1 |  |  | YES | ignore |
| >LTM Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | - |  |
| >Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. | - |  |
| >Tag ID Pointer | M |  | OCTET STRING | Includes the tagId-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10]. | - |  |
| >source gNB-DU ID | M |  | 9.2.1.xx9 Early RACH Resources Requester ID |  | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | Ignore |
| Data Forwarding Info | FFS |  |  |  | YES | ignore |
| Key NG-RAN Star | O |  | BIT STRING (256) | KNG-RAN* defined in TS 33.501 [28]. | YES | ignore |
| Security Information for LTM (FFS on the name) | O |  | 9.2.1.x12 |  | YES | ignore |
| Candidate gNBs Information List | O |  | 9.2.1.xYY |  | YES | ignore |

Editor’s note: The Data Forwarding Info IE is a list, the granularity is FFS.

9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.1.x5 LTM CONFIGURATION UPDATE FAILURE

This message is sent by the NG-RAN node2 to indicate to NG-RAN node1 about LTM Configuration Update failure.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.1.x6	 LTM CANCEL

This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared LTM handover.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Cause | O |  | 9.2.3.2 |  | YES | ignore |
| LTM Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

9.1.1.x7 	CSI-RS COORDINATION REQUEST

This message is sent by  the source NG-RAN node to the target NG-RAN to coordinate the activation and deactivation of CSI-RS transmission for a UE at  target NG-RAN node.

Direction:  source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| CSI-RS Resource Request List |  | 1 |  | Cell ID list to which the request applies. | YES | reject |
| >CSI-RS Resource Request Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>LTM-CSI-Resource Configuration ID | O |  | INTEGER (0..111) | Corresponding to the LTM-CSI-ResourceConfigId-r18 IE as defined in TS 38.331 [10] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |

9.1.1.x8	CSI-RS COORDINATION RESPONSE

This message is sent by  target NG-RAN node to source NG-RAN node to coordinate the activation and deactivation of CSI-RS transmission for a UE at  target NG-RAN node.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| CSI-RS Resource Response List |  | 1 |  | Cell ID list to which the response applies. | YES | reject |
| >CSI-RS Resource Response Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Response | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>LTM-CSI-Resource Configuration ID | O |  | INTEGER (0..111) | Corresponding to the LTM-CSI-ResourceConfigId-r18 IE as defined in TS 38.331 [10] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.xx1	LTM Information Request

This IE contains a request for LTM information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Configuration | O |  | OCTET STRING | Contains the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. |
| Request for CSI Resource Configuration | FFS |  | ENUMERATED (true, …) |  |
| CSI Resource Configuration | FFS |  | 9.2.1.xx8 |  |
| LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  |
| Request for Reference Configuration | O |  | ENUMERATED (true, …) |  |

9.2.1.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Resource Configuration | O |  | 9.2.1.xx8 |  |
| LTM Candidate Configuration | M |  | OCTET STRING | Contains ltm-CandidateConfig as defined in TS 38.331 [10]. |
| Reference Configuration | O |  | OCTET STRING | Contains the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. |
| Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.xx3	Early Sync Information Request

This IE contains a request for Early synchronization information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Early Sync Configuration Request | M |  | ENUMERATED (true, …) |  |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Early RACH Resources Requester ID | M |  | 9.2.1.xx9 | Identifies the entity requesting Early RACH resources. |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

9.2.1.xx4	Early Sync Information Response

This IE contains Early synchronization related information to be provided in response to information provided in the Early Sync Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| Early UL Sync Configuration | O |  | 9.2.1.xx6 | Early UL sync configurations for the UE. |
| Early UL Sync Configuration for SUL | O |  | 9.2.1.xx6 | Early UL sync configurations for the UE for SUL carrier. |

9.2.1.xx5	LTM Configuration ID Mapping List

This IE indicates the list of LTM candidate cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  |
| >LTM Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

9.2.1.xx6	Early UL Sync Configuration

This IE indicates the Early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [10]. |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item IEs |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Early RACH Resources Requester ID | M |  | 9.2.1.xx9 | Identifies the entity requesting Early RACH resources. |
| >>Preamble Index List | O |  | 9.2.1.xx7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

9.2.1.xx7	Preamble Index List

This IE indicates the list of preamble indexes to be used for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Preamble Index Item IEs |  | 1..< maxnoofLTMCells> |  |  |
| >Preamble Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

9.2.1.xx8	CSI Resource Configuration

This IE contains the CSI resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-CSI-ResourceConfigToAddModList as defined in TS 38.331 [10]. |
| CSI Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-CSI-ResourceConfigToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [10]. |
| SSB Information | O |  | 9.2.1.x10 |  |

9.2.1.xx9	Early RACH Resources Requester ID (naming FFS)

This IE indicates the identity of the Early RACH resources requester.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Early RACH Resources Requester ID (naming FFS) | M |  | INTEGER (0 .. 236-1) | Identifies requester of the Early RACH resources. |

9.2.1.x10	SSB Information

This information element contains the SSB time/frequency information for the TRPs.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB Information List |  | 1 |  |  |
| >SSB Information Item |  | 1…<maxNoSSBs> |  |  |
| >>SSB Configuration | M |  | SSB Time/Frequency Configuration 9.2.1.x11 |  |
| >>PCI | M |  | INTEGER (0..1007) |  |

| Range bound | Explanation |
| --- | --- |
| maxNoSSBs | Maximum no of SSBs for which the configuration can be provided. Value is 255. |

9.2.1.x11	SSB Time/Frequency Configuration

This information element contains the time and frequency configuration of an SSB.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB frequency | M |  | INTEGER (0..3279165) | ARFCN |
| SSB subcarrier spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, kHz240,..., kHz480, kHz960) | The value 60kHz is not supported in this version of the specification. |
| SSB Transmit power | M |  | INTEGER (-60..50) | EPRE of SSS |
| SSB periodicity | M |  | ENUMERATED(ms5, ms10, ms20, ms40, ms80, ms160, …) |  |
| SSB half frame index | M |  | INTEGER(0..1) |  |
| SSB SFN offset | M |  | INTEGER(0..15) |  |
| SSB Positions in Burst | O |  | 9.2.2.64 |  |

9.2.1.x12	Security Information for LTM (FFS on the name)

This IE contains security related information for subsequent LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Security Capabilities | M |  |  |  |
| >NR Encryption Algorithms | M |  | BIT STRING {nea1-128(1), nea2-128(2), nea3-128(3)} (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NEA0, "second bit" – 128-NEA1, "third bit" – 128-NEA2, "fourth bit" – 128-NEA3, “fifth to eighth bit” correspond to bit 4 to bit 1 of octet 3 in the UE Security Capability IE defined in TS 24.501 [30], other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.501 [28]. |
| >NR Integrity Protection Algorithms | M |  | BIT STRING {nia1-128(1), nia2-128(2), nia3-128(3)} (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NIA0, "second bit" – 128-NIA1, "third bit" – 128-NIA2, "fourth bit" – 128-NIA3, “fifth to eighth bit” correspond to bit 4 to bit 1 of octet 4 in the UE Security Capability IE defined in TS 24.501 [30], other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.501 [28]. |
| User Plane Security Information List |  | 1 |  |  |
| >User Plane Information Item |  | 1 .. <maxnoof PDU sessions > |  |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  |
| >>Security Indication | O |  | 9.2.3.52 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256. |

9.2.1.xYY	Candidate gNB Information List

This IE indicates the list of candidate gNBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Candidate gNB Information Item IEs |  | 1..< maxnoofLgNB> |  |  |
| >Global gNB ID | M |  | 9.2.2.1 |  |
| > Old target UE XnAP ID for each candidate gNB | M |  | 9.2.3.25 | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLgNB | Maximum no. of gNBs configured for LTM towards one UE, the maximum value is xx. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

Skip unchanged part

(TP to TS 38.473 on Inter-CU LTM) Inter-CU LTM on top of R3-253149

Skip unchanged part

#### 9.3.1.XX	LTM Security Information

This IE contains the security related information for LTM candidate cell(s) to support the UE in generating the key material for AS security during an inter-CU LTM cell switch.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [Y] |
| R19 Set ID | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. |

Skip unchanged part

9.2.2.x1	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Resource Request List |  | 1 |  |  | YES | reject |
| >CSI-RS Resource Request Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Cell ID list to which the request applies. | – |  |
| >>Semi Persistent CSI-RS Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>LTM-CSI-Resource Configuration ID | O |  | INTEGER (0..111) | Corresponding to the LTM-CSI-ResourceConfigId-r18 IE as defined in TS 38.331 [10] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |

9.2.2.x2	DU-CU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-CU e.g. to inform the gNB-DU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | ignore |
| CSI-RS Resource Response List |  | 1 |  |  | YES | ignore |
| >CSI-RS Resource Response Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Cell ID list to which the response applies. | – |  |
| >>Semi Persistent CSI-RS Transmission Response | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>LTM-CSI-Resource Configuration ID | O |  | INTEGER (0..111) | Corresponding to the LTM-CSI-ResourceConfigId-r18 IE as defined in TS 38.331 [10] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |

9.2.2.y1	CU-DU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-CU e.g. to request the gNB-DU to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Resource Request List |  | 1 |  |  | YES | reject |
| >CSI-RS Resource Request Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Cell ID list to which the request applies. | – |  |
| >>Semi Persistent CSI-RS Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>LTM-CSI-Resource Configuration ID | O |  | INTEGER (0..111) | Corresponding to the LTM-CSI-ResourceConfigId-r18 IE as defined in TS 38.331 [10] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |

9.2.2.y2	CU-DU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-DU e.g. to inform the gNB-CU about the SP CSI-RS transmissions activation/deactivation result. (Detail is FFS)

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | ignore |
| CSI-RS Resource Response List |  | 1 |  |  | YES | ignore |
| >CSI-RS Resource Response Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Cell ID list to which the response applies. | – |  |
| >>Semi Persistent CSI-RS Transmission Response | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>LTM-CSI-Resource Configuration ID | O |  | INTEGER (0..111) | Corresponding to the LTM-CSI-ResourceConfigId-r18 IE as defined in TS 38.331 [10] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |