3GPP TSG-RAN WG3 #128	R3-253199

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:		13.2 Support of inter-CU LTM

Title:		Discussion Rel-19 inter-CU LTM in DC scenario

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Last RAN3#127bis meeting for the inter-CU LTM with DC configuration,

- TP to baseline CR of 38.423 (R3-252461) was agreed, and after the meeting in e-mail checking of Post_Meeting_Merging_TPs, the merging of TP into the new baseline CR of 38.423 (R3-252541). (This new baseline CR extracted from the DC part in baseline CR of 38.423 in R3-251558)

- also TP to baseline CR of 37.340 (R3-252462) was agreed, and after the meeting in e-mail checking of Post_Meeting_Merging_TPs, the merging agreed TP into the based CR of 37.340 (R3-252516).

The agreed TP was reflecting the following agreement:

RAN3#127bis: (grey font colour not directly related to LTM with DC) Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM. The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: Candidate Cell ID TA value Preamble index RA-RNTI FFS on the XnAP IDs. Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB. For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB. Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s). Include the following information in SN Change Required (Source SN to MN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for each candidate SN (FFS) CSI Resource configuration request (explicit way) Early sync information request (explicit way) Include the following information in SN Addition Request (MN to Candidate SN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for the candidate SN (explicit way) CSI Resource configuration request (explicit way) Early sync information request (explicit way) It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message. For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN. The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN. Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message. Continue to work on the solutions comparison and details…

This contribution we continue to discuss the remaining issues.

2. Discussion

2.1	Request Reference configuration from MN

Last RAN3#127bis meeting agreement with a small FFS:

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

We think that since this Rel-19 SN initiated inter-CU SCG LTM is similar to the Rel-18 Subsequent-CPAC (S-CPAC), its principle should be same as in Rel-18 SCPAC, i.e. The MN is able to request an SCG reference configuration from either the source SN or a candidate SN, then it can just simplify to have consistency handling.

Proposal 1: To follow the same in Rel-18 S-CPAC that MN is able to request SCG reference configuration, for this Rel-19 SN initiated inter-CU SCG LTM it is also able for the MN to request to the source SN to provide SCG reference configuration, in addition to the already agreed request to candidate SN(s).

TP only to remove the FFS.

| >SCG Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for SCG is requested. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

2.2	Security key updates for SCG LTM

The security information update for the inter-CU SCG LTM is for the case when the SN will be changed. The security information will be the security key and the SK-counter. The options that was left from RAN3#127bis are:

- Option 1: MN sends a list of {S-key and SK-counter} to each of candidate SN via SN addition/modification request message

- Option 2: MN sends security key and a list of sk-counters to the target SN the SN via the CELL SWITCH NOTIFICATION message

Let’s first discuss which message to use. Since all from the beginning that the for the inter-CU SCG LTM, to reuse as much as possible what has been done in Rel-18 S-CPAC, and since Rel-18 S-CPAC has introduced the forwarding of security key and SK-counter in the SN Addition Request and SN Modification Request messages, it would be good to follow this Rel-18 S-CPAC. It may have an argument that for Rel-19 Inter-CU SCG LTM that the Cell Switch Notification procedure is to be introduced, then can include the Security Key and SK-Counter in this Cell Switch Notification procedure from MN to SN. Both are workable with only difference of the timing when to send the Security Key and SK-Counter but no difference on its pros and cons.

Proposal 2: To follow the Rel-18 S-CPAC, MN to send list of Security Key and SK-Counter from MN to SN for the Rel-19 inter-CU SCG LTM, via the SN addition/modification request message.

2.3	Access Success

The stage 2 BLCR for 37.340 for the Rel-19 inter CU SCG LTM has step 23: Access Success from Target SN to Source SN after the UE successfully accessed to the target SN (Target PSCell), it was pointed out in RAN3#127bis that for the dual connectivity aspect so far there is not such handling that the target SN directly communicate with the source SN.

It may first to have a question whether need to inform the source SN that the UE has successfully accessed to the target SN i.e. to let source SN know the UE has left. What will be bad if the source SN does not know the UE has left?

- if for the SN terminated bearer, even after the UE has left from the source SN, the source SN still can hold the S1-U until it is informed by the MN to remove.

- what if the Source PScell in source SN will detect radio link failure? This may be important to avoid the ex-PSCell in the ex-source SN to initiate any failure procedure due to unnecessary detection of radio link failure, it would then be feasible to inform the source SN that the UE has left.

Observation: After the UE has been switched to target PSCell, to avoid the ex-PSCell in the ex-source SN to initiate any failure procedure due to unnecessary detection of radio link failure, it would then be feasible to inform the source SN that the UE has left.

Which procedure to use to inform the source SN that the UE has left?

- Introduce new procedure between target SN and source SN will need further complicated handling e.g. to establish new UE Association between source SN and Target SN. In order to avoid such complicated thing with no much benefit, it would be easy to just apply the procedure that the target SN inform the MN by extension of the existing Handover Success procedure, and the MN inform the source SN by the SN Modification Request procedure.

Proposal 3: It is needed to inform the source SN that the UE has left its PScell to other SN in order avoid the source SN detect any unnecessary radio failure. After the UE has successfully accessed to the target SN, the target SN inform the MN by the Handover Success procedure, then the MN inform the source SN by the SN Modification Request procedure.

2.1	Avoid simultaneous MCG and SCG LTM

RAN3 has confirmed that ““Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time”.”

For the Inter-CU LTM with DC configuration, to avoid both MCG and SCG are configured at the same time, the MN and SN need to coordinate before initiating the inter-CU MCG LTM and inter-CU-SCG LTM respectively.

One principle to draw is that, which node will be the canter management of such situation. It has been more or less understood that the MN is the node who will control for the dual connectivity configure to the UE, e.g. whether to add SCG for the UE or not, therefore it would be natural to do like:

- for the inter-CU MCG LTM, MN indicate/Notify to the SN,

- for the inter-CU SCG LTM, SN send a request to MN which may be accepted or rejected by the MN.

Proposal 4: To avoid inter-CU MCG LTM and inter-CU SCG LTM be configured at the same time, the MN is the centre control / decision so

- for the inter-CU SCG LTM the SN need to request to the MN if can initiating inter-CU SCG LTM

- for the inter-CU MCG LTM, the MN indicate/Notify to the SN;

- in case of crossing, the inter-CU MCG LTM take the precedence.

3. Conclusion and proposals

Proposal 1: To follow the same in Rel-18 S-CPAC that MN is able to request SCG reference configuration, for this Rel-19 SN initiated inter-CU SCG LTM it is also able for the MN to request to the source SN to provide SCG reference configuration, in addition to the already agreed request to candidate SN(s).

Proposal 2: To follow the Rel-18 S-CPAC, MN to send list of Security Key and SK-Counter from MN to SN for the Rel-19 inter-CU SCG LTM, via the SN addition/modification request message.

Proposal 3: It is needed to inform the source SN that the UE has left its PScell to other SN in order avoid the source SN detect any unnecessary radio failure. After the UE has successfully accessed to the target SN, the target SN inform the MN by the Handover Success procedure, then the MN inform the source SN by the SN Modification Request procedure.

Proposal 4: To avoid inter-CU MCG LTM and inter-CU SCG LTM be configured at the same time, the MN is the centre control / decision so

- for the inter-CU SCG LTM the SN need to request to the MN if can initiating inter-CU SCG LTM

- for the inter-CU MCG LTM, the MN indicate/Notify to the SN;

- in case of crossing, the inter-CU MCG LTM take the precedence.

# References

[1] RP-250339 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-252320 Summary of offline discussion on MobilityEnh

[3] R3-252516 => R3-253084 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

[4] R3-251555 => R3-253085 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[5] R3-252517 => R3-253086=> R3-253184 (BL CR to 38.300) Inter-CU LTM

[6] R3-252518 => R3-253087 (BL CR to 38.401) on conditional intra-CU LTM

[7] R3-252519 => R3-253088 (BL CR to 38.420) Support for Inter-CU LTM

[8] R3-252520 => R3-253089 (BL CR to 38.423) Xn Support for Inter-CU LTM

[9] R3-252521 => R3-253091 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

[10] R3-252541 => R3-253090 (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC

[11] R3-252522 => R3-253092 => R3-253149 (BL CR to 38.473) Inter-CU LTM

[12] R3-252461 (TP to BL CR for 38.423 DC) Inter-CU LTM in DC

[12] R3-252462 (TP for LTM BLCR for TS37.340): TP for Inter-CU LTM