3GPP TSG-RAN WG3 #128	R3-253201

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:		9.2

Title:		Discussion of L3 measurement triggered LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Last RAN3#127bis meeting the L3 measurement triggered LTM was discussed again based on the incoming LS from RANP in R3-251532. It has reached a status to take the draft CR to 38.473 in R3-252412 as the baseline, and the measurements structure to be continued.

This contribution continue to discuss a potential need to make the function work properly, to address an FFS in the SOD (R3-252265)

- Agreement 2.6: Measurement quantities will be included per beam and per candidate cell level. FFS: If there is a need for any offsets (e.g. offset better than the serving cell), but several companies said that there is no need and the frame of L1 measure is based on DU implementation.

2. Discussion

2.1	Assistance information from gNB-CU to let gNB-DU to make final decision

The baseline in the draft CR to 38.473 (R3-252412), while all are still FFS, there is a possibility for the gNB-CU to give Additional Mobility Assistance Information IE which include a list of candidate Cells with each its measurement result (RSRP, RSRQ, SINR). This would give a possibility for the gNB-DU to take a final decision of cell switching e.g. when “cell switch” is indicated by the gNB-CU.

We understand that, this L3 measurement triggered LTM is to have a handling in LTM for some UEs that do not have capability to do L1 measurement.

To have a close look, in order for the gNB-DU to choose better or suitable cell to switch to, it would need to give more information e.g. the Additional Mobility Assistance Information IE can include measurement result of the serving cell, in addition to the measurement result of candidate cells.

Alternatively, the gNB-CU can give an information that the measurement result of candidate cell is offset better than the serving cell (i.e. corresponding to Event A3 in RRC measurement result), then the offset value can be included in the Additional Mobility Assistance Information IE.

Proposal 1a: It is proposed to add in the Additional Mobility Assistance Information IE the measurement result of the serving cell, in addition to the measurement result of candidate cells.

Proposal 1b: Alternatively, it is proposed to add an offset value to show how much the measurement result of candidate cell is offset better than the serving cell.

3. Conclusion and proposals

Proposal 1a: It is proposed to add in the Additional Mobility Assistance Information IE the measurement result of the serving cell, in addition to the measurement result of candidate cells.

Proposal 1b: Alternatively, it is proposed to add an offset value to show how much the measurement result of candidate cell is offset better than the serving cell.

# References

[1] R3-252265 CB: # 4_L3MeasurementsLTM

[2] R3-252412 (draft CR to 38.473 Rel18 Cat. F) F1 support of L3 measurements-based LTM

Annex: TP to draft CR (partly from R3-252412)

9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to the gNB-DU to trigger a cell switch command and/or early synchronization for the UE.

Direction: gNB-CU  gNB-DU

FFS: Needs to further discuss the presence of IEs, the granularities of measurement quantities, and the IE levels.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Triggering Indication | FFS |  | BIT STRING (SIZE(8)) FFS | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |
| Mobility Assistance Information Cell Switch | C-ifTrigger Indication is set cell switch |  |  |  | YES | reject |
| >Candidate Cell ID | M |  |  |  |  |  |
| >others FFS |  |  |  |  |  |  |
| Mobility Assistance Information Early DL sync | C-ifTrigger Indicaion is set early DL sync |  |  |  |  |  |
| >Candidate Cell ID List |  | 1 ..<maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  |  |  |  |  |
| >>others FFS |  |  |  |  |  |  |
| Mobility Assistance Information Early UL sync |  | C-ifTrigger Indicaion is set early UL sync |  |  |  |  |
| >Candidate Cell ID List |  | 1 ..<maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  |  |  |  |  |
| >>others FFS |  |  |  |  |  |  |
| Additional Mobility Assistance Information | O |  |  |  |  |  |
| > Serving Cell ID | FFS |  |  | FFS on granularities, e.g, SSB index level, CSI-RS level |  |  |
| >>Selected Measurement Quantities | FFS |  |  |  |  |  |
| >>>RSRP | FFS |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>RSRQ | FFS |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>SINR | FFS |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |  |  |
| >Candidate Cell ID List | FFS |  |  | FFS on granularities, e.g, SSB index level, CSI-RS level |  |  |
| >>Selected Measurement Quantities | FFS |  |  |  |  |  |
| >>>RSRP | FFS |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>RSRQ | FFS |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>SINR | FFS |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |  |  |

Alternative way: (to give Offset value to show the measurement result is offset better than serving cell)

9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to the gNB-DU to trigger a cell switch command and/or early synchronization for the UE.

Direction: gNB-CU  gNB-DU

FFS: Needs to further discuss the presence of IEs, the granularities of measurement quantities, and the IE levels.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Triggering Indication | FFS |  | BIT STRING (SIZE(8)) FFS | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |
| Mobility Assistance Information Cell Switch | C-ifTrigger Indication is set cell switch |  |  |  | YES | reject |
| >Candidate Cell ID | M |  |  |  |  |  |
| >others FFS |  |  |  |  |  |  |
| Mobility Assistance Information Early DL sync | C-ifTrigger Indicaion is set early DL sync |  |  |  |  |  |
| >Candidate Cell ID List |  | 1 ..<maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  |  |  |  |  |
| >>others FFS |  |  |  |  |  |  |
| Mobility Assistance Information Early UL sync |  | C-ifTrigger Indicaion is set early UL sync |  |  |  |  |
| >Candidate Cell ID List |  | 1 ..<maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  |  |  |  |  |
| >>others FFS |  |  |  |  |  |  |
| Additional Mobility Assistance Information | O |  |  |  |  |  |
| >Candidate Cell ID List | FFS |  |  | FFS on granularities, e.g, SSB index level, CSI-RS level |  |  |
| >>Selected Measurement Quantities | FFS |  |  |  |  |  |
| >>>RSRP | FFS |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>RSRQ | FFS |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>SINR | FFS |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |  |  |
| >>> Q-OffsetRange | FFS |  | ENUMERATED {dB-24, dB-22, dB-20, dB-18, dB-16, dB-14, dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3, dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24} | Offset value better than serving cell. Indicating only once in the list. Corresponding to Q-OffsetRange IE as defined in TS 38.331 [10] |  |  |