3GPP TSG-RAN WG3 Meeting #128	R3-253203

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		11.3

Source:		NEC

Title:	Open issues for AI/ML-based CCO

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, the following agreements were achieved and TP to 38.423 [1] and TP to 38.473 [2] reflecting the agreement have been agreed.

The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s.

Legacy UE performance measurement metrics can be reused for CCO.

The following issues are still FFS:

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option1: no need

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Whether/what additional UE performance measurement metrics is needed?

Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

In this contribution, we further discuss the abovementioned open issues.

# Discussion

2.1	Recommended future CCO state

For the receiving side, after gNB-CU2 receives future coverage configuration and related Coverage Modification Cause from a neighbour gNB1 (gNB-CU1), it will indicates its own gNB-DU2 a CCO issue. Following legacy, how to generate this CCO issue can be up to implementation.

However, not as in legacy that gNB-DU2 can use received CCO issue together with its own measurement information to decide its own cell adjustment, for further CCO state adaptation, gNB-DU2 cannot decide how to adjust its cell if it only receive a CCO issue. Therefore we think, besides a CCO issue, a further CCO state is also needed to be provided from gNB-CU2 to gNB-DU2.

Regarding the further CCO state mentioned above, there are two options:

Option 1: gNB-CU2 directly forwards to gNB-DU2 the received future CCO state.

Option 2: gNB-CU2 uses the received information as input to generate a recommend future CCO state, e.g. by using AI/ML-based CCO model.

In both options, the further CCO state sent to gNB-DU2 as used only as assistance information, it is still gNB-DU2’s decision on how to adjust its own cell.

As we agreed in the SI phase, future CCO state can be generated by AI/ML-based CCO model. Considering gNB-CU2 may hold an AI/ML-based CCO model and it collects all input data for AI/ML model, it is more knowledgeable on the neighbour node status and UE performance status. Based on the received future CCO state and predicted CCO issue, also together with other information, e.g. current/predicted radio resource status, gNB-CU2 can take advantage of AI/ML tool and coordinate all gNB-DUs connected to it to generate a recommend future CCO state for gNB-DU2.

However, to achieve this, gNB-CU2 needs to configured with the alternative coverage configurations by OAM. Consider the limited time for this WI and in order to save standard effort, we suggest to adopt option1, but not to pursue option 2 in this release.

Proposal 1: For the receiving side, gNB-CU2 forwards the received future CCO state to gNB-DU2 as assistance information to derive a matching future CCO state.

2.2	UE performance feedback for CCO

RAN3 already agreed that legacy UE performance measurement metrics can be reused for CCO, and the only open issue for UE performance feedback is whether additional measurement metrics is needed.

For AI/ML based CCO, UE performance feedback is used to evaluate the AI/ML model by comparing UE performance before and after a CCO action that is triggered based on the output from AI/ML Model Inference. Since the predicted CCO issue can be avoided or mitigated by advance CCO action triggered by the AI/ML Model Inference output, UE performance should be not (much) impacted due to the coverage adjustment, especially for the UE in the cell border area. Therefore, it is reasonable that gNB needs to evaluate UE performance for UEs in the cell border area that could be affected by the CCO action. To save standard effort, we prefer this can be achieved by gNB implementation, for example, neighbour gNB can report only the UE performance of the UEs in the cell border area based on UE location information.

Observation 1: UE performance collection based on UE location information can be achieved by gNB implementation and UE location information is not needed to be reported as feedback.

Some companies also proposed to report UE radio measurements (cell level RSRP, RSRP, SINR) as UE performance feedback. However, as we discussed in SI phase, besides legacy UE performance metrics, SON Reports including RLF report, RACH report can also be used as feedback, which already can reflect the radio condition. Therefore, it is no need to report additional UE radio measurements as feedback.

Observation 2: SON report can be used to evaluate radio condition and UE radio measurements is not needed to be reported as feedback.

In summary, we think legacy UE performance measurement metrics are already enough for CCO and no additional metric is needed.

Proposal 2: No additional UE performance measurement metric is needed for CCO.

2.3	Time information over Xn

For CCO, the following two timing information were introduced:

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

In XnAP NG-RAN NODE CONFIGURATION UPDATE message, a Time for Future Coverage State IE can be included for each Future Coverage Modification Item. As explained in the semantics description, this time information indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information [1].

As for timing information for predicted CCO issue, it indicates the time when the predicted CCO issue will happen in the gNB who performed this prediction. gNB uses this timing information and the predicted CCO issue to generate the corresponding future coverage configuration and timing information for future coverage state in legacy way or using AI/ML model. If in CU/DU split, gNB-CU will send this timing information together with the predicted CCO issue to gNB-DU, which can assist gNB-DU to select corresponding future coverage configuration and time for future coverage modification.

As described above, timing information for predicted CCO issue is used to generate corresponding future coverage configuration and time for future coverage state. For neighbour nodes, this timing information for predicted CCO issue is useless. The neighbour nodes only need to know when the future coverage state(s) will be applied, which can allow the neighbour nodes to adapt to this CCO coverage change.

Proposal 3: Not support transfer timing information for predicted CCO issue over Xn.

Based on the above proposals, we also provided a TP to BLCR TS38.473 on AI/ML-based CCO in [3].

Proposal 4: To agree the TP to BLCR TS38.473 on AI/ML-based CCO in [3]

# Proposals

In this contribution, we provide our analysis on open issues for AI/ML-based CCO and made the following observations and proposals:

Proposal 1: For the receiving side, gNB-CU2 forwards the received future CCO state to gNB-DU2 as assistance information to derive a matching future CCO state.

Observation 1: UE performance collection based on UE location information can be achieved by gNB implementation and UE location information is not needed to be reported as feedback.

Observation 2: SON report can be used to evaluate radio condition and UE radio measurements is not needed to be reported as feedback.

Proposal 2: No additional UE performance measurement metric is needed for CCO.

Proposal 3: Not support transfer timing information for predicted CCO issue over Xn.

Proposal 4: To agree the TP to BLCR TS38.473 on AI/ML-based CCO in [3]

# Reference

[1] R3-252355, (TP for AI/ML BLCR to TS 38.423) Discussion on the AI/ML-based Coverage and Capacity Optimization

[2] R3-252356, (TP for AI/ML BLCR to TS 38.473) Discussion on the AI/ML-based Coverage and Capacity Optimization

[3] R3-253204, TP to BLCR TS38.473 on AI/ML-based CCO