3GPP TSG-RAN WG3 Meeting #128	R3-253204

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		11.3

Source:		NEC

Title:	TP to BLCR TS38.473 on AI/ML-based CCO

Document for:		Discussion and decision

# Introduction

In this contribution, we provide TP to BLCR TS38.473 on AI/ML-based CCO based on the proposals in R3-253203.

# TP to BLCR TS38.473

<Start of 1st change>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. This IE is only present if the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU. |
| Future Cell Coverage State | O | INTEGER (0..63, ...) |  | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. This IE is only present if the NR CGI IE contained in the Predicted Affected Cells and Beams IE is not served by the gNB-DU. |

<Start of 2nd change>

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection	Predicted-CCO-issue-detection	OPTIONAL,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

timeforPredictedCCOIssue		TimeforPredictedCCOIssue,	OPTIONAL,

futurecellCoverageState			FutureCellCoverageState,	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

<End of change>