3GPP TSG-RAN WG3 Meeting #128	R3-253207

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		11.4

Source:		NEC

Title:	NR-DC mobility optimization

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, the following agreement was achieved and TP to 38.423 [1] for adding termination conditions of UE performance collection in NR-DC case has been agreed.

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following issues are still FFS:

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In this contribution, we further discuss the open issues for NR-DC case.

# Discussion

## 2.1	UE performance

For NR-DC scenario, if a Data Collection ID is included in S-NODE ADDITION REQUEST message from MN to SN, the target SN needs to collect measured UE performance after SN addition and MN-initiated SN change and reports to MN in DATA COLLECTION UPDATE message.

Regarding UE Throughput DL/UL, one LS to SA5 [2] was sent out to ask feedback on whether the granularity of UE performance metrics specified in TS 28.558 is per-UE or per-DRB, and SA5 has already replied the LS in [3]. In the reply LS, SA5 clarifies that the measurement inputs of UE Throughput DL/UL are taken per DRB, but they are aggregated to provide throughput metrics at the UE level.

Regarding Packet Dealy and Packet Loss, the same situation applies, i.e. the measurements can be taken per DRB and UE level performance is derived as the average value of DRB level measurements.

In summary, it can be confirmed that UE level performance is derived as the average value of per DRB level measurements of the UE. And the granularity of UE performance metrics in NR-DC case should be no difference from legacy ones.

Proposal 1: Legacy UE level performance feedback can be reused for NR-DC case.

## 2.2	Predicted PSCell ID from MN to SN

Another open issue is whether MN can predict PSCell and send the predict PSCell ID to SN. Our understanding is that when selecting which candidate PSCell is relevant for a UE, SN needs to take into account various factors, including radio link quality, load condition and other implementation-specific considerations. Therefore, MN may not have enough input to perform PSCell prediction.

Proposal 2: Not support transfer predicted PSCell ID from MN to SN.

## 2.3	PSCell ID for UE Performance

The UE performance collected in SN is used by MN to evaluate whether its inference about the SN is optimal or not. As explained above, PSCell selection is performed by SN based on information from MN and also its own measurements and configuration, therefore MN does not need to know which PSCell is used by the UE and thus there is no need to report PSCell ID together with UE Performance IE in the DATA COLLECTION UPDATE message.

Proposal 3: Not support report PSCell ID in the DATA COLLECTION UPDATE message.

# Proposals

In this contribution, we provided our consideration on UE performance feedback for NR-DC case and the following proposals are made.

Proposal 1: Legacy UE level performance feedback can be reused for NR-DC case.

Proposal 2: Not support transfer predicted PSCell ID from MN to SN.

Proposal 3: Not support report PSCell ID in the DATA COLLECTION UPDATE message.

# Reference

[1] R3-252448, (TP for AI/ML BLCR to TS 38.423) Discussion on AI/ML-based Mobility Optimization in NR-DC

[2] R3-247775, LS on per-UE UE performance metrics

[3] S5-252056, Reply LS on per-UE UE performance metrics