3GPP TSG-RAN WG3 Meeting #128	R3-253208

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		11.4

Source:		NEC

Title:	(TP to BLCR TS38.473) Split architecture support

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, the following agreements were achieved and TP to 37.480 [1] and TP to 37.483 [2] for the new procedure over E1 interface have been agreed.

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

The following issues are still FFS:

Defer the discussion on packet loss rate in the split architecture in the next meeting.

For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

In this contribution, we provide our considerations on the procedure used for AI/ML data collection over F1.

# Discussion

## 2.1	Measured EC over F1

As agreed, Energy Cost EC is measured in gNB-DU and reported to gNB-CU, and then gNB-CU calculates the sum of the EC of its gNB-DUs to generate the EC of gNB.

According to the already achieved agreements, only measured EC needs to be transferred from gNB-DU to gNB-CU. Therefore, in order to save standard effort, we support to reuse the legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

Proposal 1: To change the following WA into agreement:

Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

Below we analyse the detailed changes in the existing resource status initiation procedure and resource status report procedure.

In the RESOURCE STATUS REQUEST message, one bit for measured EC needs to be added in Report Characteristics IE. All the other IEs, e.g. Registration Request, Reporting Periodicity, can be reused for AI/ML data collection.

Since EC is a node level measurement, in the RESOURCE STATUS UPDATE message, a Node Measurement Result IE needs to be added for EC reporting. The IE value of Energy Cost IE should be the same as in TS38.423, i.e. INTEGER (0..10000,…).

TP to TS38.473, mirroring the analysis above, are available in Annex.

Proposal 2: To agree the TP to BLCR TS38.473 on measured EC transfer from gNB-DU to gNB-CU in Annex.

# Proposals

In this contribution, we provide our consideration on split architecture support and the following proposals are made:

Proposal 1: To change the following WA into agreement:

Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

Proposal 2: To agree the TP to BLCR TS38.473 on measured EC transfer from gNB-DU to gNB-CU in Annex.

# Annex TP to BLCR TS38.473

<Start of 1st change>

### 8.2.10	Resource Status Reporting Initiation

#### 8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-DU shall perform measurements on. For each cell, gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE; If the cell for which Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell may include the MIMO PRB usage Information IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List IE for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value IE. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE providing the SSB area capacity with respect to the Cell Capacity Class Value. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

-	the Hardware Load Indicator IE, if the fourth bit, " HW LoadInd Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Number of Active UEs IE, if the fifth bit, "Number of Active UEs Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the NR-U Channel List IE, if the sixth bit, " NR-U Channel List Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

-	the Energy Cost IE, if the seventh bit, "Energy Cost" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

<Start of 2nd change>

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic, Seventh Bit = Energy Cost. Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) |  | - |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  | 1..< maxnoofBPLMNsNR> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

<Start of 3rd change>

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel Occupancy Time Percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.3.1.295 | Indicates the radio resource status per NR-U channel. | YES | ignore |
| Node Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost |  |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

<Start of 4th change>

### 9.4.4	PDU Definitions

<Unchanged part omitted>

NetworkControlledRepeaterAuthorized,

NodeMeasurementResultList,

NRCGI,

<Unchanged part omitted>

id-PreconfiguredSRSInformation,

id-NodeMeasurementResultList,

maxCellingNBDU,

<Unchanged part omitted>

-- **************************************************************

--

#### -- Resource Status Reporting ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Update

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ResourceStatusUpdateIEs}},

...

}

ResourceStatusUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID			CRITICALITY reject	TYPE GNBCUMeasurementID				PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID			CRITICALITY ignore	TYPE GNBDUMeasurementID			PRESENCE mandatory	}|

{ ID id-HardwareLoadIndicator			CRITICALITY ignore	TYPE HardwareLoadIndicator			PRESENCE optional	}|

{ ID id-TNLCapacityIndicator			CRITICALITY ignore	TYPE TNLCapacityIndicator		PRESENCE optional	}|

{ ID id-CellMeasurementResultList		CRITICALITY ignore	TYPE CellMeasurementResultList	PRESENCE optional	}|

{ ID id-NodeMeasurementResultList		CRITICALITY ignore	TYPE NodeMeasurementResultList	PRESENCE optional	},

...

}

<Start of 5th change>

### 9.4.5	Information Element Definitions

<Unchanged part omitted>

Endpoint-IP-address-and-port-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-portNumber	CRITICALITY reject	EXTENSION PortNumber		PRESENCE optional},

...

}

EnergyCost ::= INTEGER (0..10000,...)

EnergyDetectionThreshold ::= INTEGER (-100..-50, ...)

<Unchanged part omitted>

NID ::= BIT STRING (SIZE(44))

NodeMeasurementResultList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF EnergyCost

NonF1terminatingTopologyIndicator ::= ENUMERATED {

true,

...

}

<Start of 6th change>

### 9.4.7	Constant Definitions

<Unchanged part omitted>

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-NodeMeasurementResultList 						ProcedureCode ::= xxx

<End of change>