3GPP TSG-RAN WG3 Meeting #128	R3-253212

St Julian's, Malta, May 19th – 23rd, 2025

Agenda item:	20.2

Title:	[TP for BL CR to TS 38.305 & TS 38.455] Discussion on AI/ML for positioning

Source:	NEC

Document for:	Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #107 finally with the removal of 2nd priority cases. The details concerning the positioning use case after RAN #107 have been finalized as follows [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Based on the previous progress, this contribution addresses the remaining issues of Case 3a and Case 3b.

- Discussion

  - Case 3a

2.1.1 Ground truth label provided by LMF to gNB

Regarding the label provided from LMF to gNB in case 3a for training data collection purpose, RAN1#120bis discussed following proposal during online session.

Proposal 4.1.5-2 For training data collection of Part B in AI/ML based positioning Case 3a, study the following options for providing label and quality indicator of label, when timing information is provided by Case 3a. - Option A. the corresponding label is provided by LMF to gNB in the format of UE location, which is provided using one of the geographic shapes defined in TS 23.032. - The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label. - Option B. the corresponding label is provided by LMF to gNB in the format of timing measurement. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. - Option B-1: the timing measurement corresponds to the distance between UE and TRP. - Option B-2: the timing measurement is UL-RTOA Note: It is assumed that user data privacy of non-PRU UE is preserved.

Eventually, following agreement was made based on Option B (with modifications). Main reason to not have Option A is that UE privacy information (i.e., UE location) is directly exposed to gNB in option A, which is not acceptable to many companies. Please note that according to existing positioning procedure, the LMF and UE never disclose target UE’s location to gNB.

Agreement (RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved.

However, "4. NRPPa POSITIONING DATA COLLECTION REPORT (Information related to UE location, Correlation Information)" and "The message includes information related to UE location and correlation information" in current TS 38.305 BL CR [2] seems to allow LMF to provide UE location to gNB, which is not aligned with the cited RAN1 discussion and agreement.

To avoid the misunderstanding, we propose to change "information related to UE location" to " timing information" based on above agreement from RAN1.

In TS 38.305, change "information related to UE location" to "timing information".

A TP for the AI/ML BLCR to TS 38.305 is provided in Annex 1.

2.1.2 Correlation information provided by LMF to gNB

Agreement (RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement (RAN1#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

In terms of correlate ("pair" used in RAN1) Part A (e.g., UL measurements at gNB) and Part B (e.g., timing information provided from LMF to gNB), RAN1 made the above agreements that the pairing needs the time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted).

Regarding other information (e.g., LMF Measurement ID and RAN Measurement ID), RAN1 has not achieved any consensus.

Therefore, again in "4. NRPPa POSITIONING DATA COLLECTION REPORT (Information related to UE location, Correlation Information)" and "The message includes information related to UE location and correlation information", we propose to change "correlation information" to "time stamp of the timing information".

In TS 38.305, change "correlation information" to "time stamp of the timing information".

A TP for the AI/ML BLCR to TS 38.305 is provided in Annex 1.

2.1.3 Signalling on the indicator of model output

During RAN3#126 meeting, there triggered an issue about whether LMF should be aware of that the reported measurements are generated by using AI/ML model or not, as well as the potential impact on further specification effect if this information should be known at LMF side.

- Option 1: dedicated IEs (e.g., AI/ML UL-RTOA) in the Measurement request and response message. - Option 2: new flag, AI/ML Positioning measurement ENUMERATED (true,…) in the TRP Measurement Result IE.

The similar issue was also discussed in RAN1. During RAN1 #120 meeting, there triggered a discussion on whether LMF can distinguish that the reported timing measurement is pre Rel-19 legacy timing measurement(i.e., generated by legacy way) or Rel-19 timing measurement(i.e., generated by AI/ML). We also think it is benefit for LMF to distinguish the reported measurement is generated by AI/ML or by legacy way. However, how to distinguish should be further studied.

In RAN2 #129b meeting, RAN2 agreed that, for Case 1, LMF is responsible for functionality management, and also agreed that an AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. The detail agreement is:

Agreements for positioning case 1 1 LMF is responsible for functionality management 2 UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control. 3 Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation". 4 An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. 5 For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

According to this agreement, at least for Case 1 that the AI/ML model is deployed at UE side, it is feasible for LMF to know whether the AI/ML functionality is activated or not without any other additional indicator, and further able to distinguish the reported location is derived from AI/ML model or from legacy positioning method without any other additional indicator also. For example, if the AI/ML positioning functionality is activated by LMF, the LMF can know the reported positionings are generated by deployed AI/ML model, until the LMF deactivate all the AI/ML positioning functionalities in UE side.

As for Case 3a, the same principle can be reused, that is, if LMF is responsible for functionality management for Case 3a, the LMF can distinguish the reported timing information is generated by AI/ML or by legacy way. However, there lacks the related agreement from RAN2(no any related agreement about LCM for Case 3a). Therefore, RAN3 can wait for more discussion on LCM for Case 3a, and then to determine what is the specification impact on distinguishing the legacy measurement and AI/ML based measurement.

At least for positioning Case 1, the LMF can distinguish the reported location is derived from AI/ML model or from legacy positioning method without the dedicated IE or new flag, as the LMF is responsible for functionality management according to RAN2’s agreement.

For Case 3a, if LMF knows whether AI/ML positioning is activated, LMF is also aware of that whether the reported measurement is generated by AI/ML.

For Case 3a, postpone the discussion of whether the specification impact is needed to distinguish whether the reported measurement is generated by AI/ML or legacy way until more discussions on LCM operation in RAN2.

  - Case 3b

2.2.1 Signalling on unavailable measurement

Regarding the measurement for case 3b, two alternatives, i.e., sample-based measurement and path-based measurement were discussed in RAN1. In RAN1 #119, it agreed that the enhanced measurement (sample-based measurement) in additional to path-based measurement can also be supported at least for case 3b in Rel-19. During RAN1#120 meeting, a discussion regarding whether the gNB can report the measurements different with what LMF required. The final conclusion is [3]:

Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

From RAN3 perspective, a problem that needs to be solved is how to signal the details when the gNB cannot provide measurement with the requested measurement type indicated by LMF. In current NRPPa procedure, if the gNB cannot provide any of the requested measurement for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, this gNB responds with a MEASUREMENT FAILURE message with an appropriate cause value to LMF. The same behavior should be reused when the gNB cannot provide the required measurement type from LMF. That is, the gNB also responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

If the gNB cannot provide measurement with the requested measurement type required by LMF, the gNB responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

A TP for the AI/ML BLCR to TS 38.455 is provided in Annex 2.

2.2.2 Entity for performing model performance monitoring for Case 3b

Regarding model performance monitoring of Case 3b, RAN1 reached the following agreement:

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

For Case 3b, the model is deployed at LMF side. In model inference stage, the gNB generates the timing information or paired timing information and power information (and phase information maybe) and reports it to LMF to further extract input data. In model performance monitoring stage, LMF can use gNB reported measurement to obtain the model output, and contrasts this output with ground truth label(i.e., UE location) to evaluate the model performance. Whether to keep this model or deactivate this model depend on the contrast result, for example, if this model output deviates the ground truth a lot, the LMF think this model is inappropriate, otherwise the LMF still keep this model at least until the enforcement of next model performance monitoring. Therefore, the model performance monitoring for Case 3b can be performed at LMF fully.

For Case 3a, LMF is the only entity to perform model performance monitoring.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observations and proposals are summarized as following:

Observation 1:	At least for positioning Case 1, the LMF can distinguish the reported location is derived from AI/ML model or from legacy positioning method without the dedicated IE or new flag, as the LMF is responsible for functionality management according to RAN2’s agreement.

Observation 2:	For Case 3a, if LMF knows whether AI/ML positioning is activated, LMF is also aware of that whether the reported measurement is generated by AI/ML.

Proposal 1:	In TS 38.305, change "information related to UE location" to "timing information".

Proposal 2:	In TS 38.305, change "correlation information" to "time stamp of the timing information".

Proposal 3:	For Case 3a, postpone the discussion of whether the specification impact is needed to distinguish whether the reported measurement is generated by AI/ML or legacy way until more discussions on LCM operation in RAN2.

Proposal 4:	If the gNB cannot provide measurement with the requested measurement type required by LMF, the gNB responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

Proposal 5:	For Case 3a, LMF is the only entity to perform model performance monitoring.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #107.

- R3-252500 (BL CR to 38.305) Introduction of AIML air

- RAN1 Chairman’s notes, 3GPP RAN1 #120

# Annex 1 – TP for the AI/ML BLCR to TS 38.305

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

4.	The LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes timing information and time stamp of the timing information that enables the gNB to correlate the timing information with information related to UL measurements.

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>

# Annex 2 – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

## 9.2.1	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ..., Serving NG-RAN node changed, Requested Item not Supported on Time, Requested Measurement Type not Supported, Requested Measurement Type Temporarily not Available ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message), ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |
| Serving NG-RAN node changed | The UE has moved to another serving NG-RAN node. |
| Requested Item not Supported on Time | The NG-RAN node is unable to provide the measurement results on time. |
| Requested Measurement Type not Supported | The NG-RAN node does not support the requested measurement tpye. |
| Requested Measurement Type Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement type. |

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>