3GPP TSG-RAN WG3 Meeting #128	R3-253213

St Julian's, Malta, May 19th – 23rd, 2025

Agenda item:	20.2

Title:	Discussion on dataset and model parameter sharing

Source:	NEC

Document for:	Discussion and Decision

- Introduction

After RAN1#119 meeting in Orlando, RAN1 triggered an LS to RAN2 and asked RAN2 for feedback on the feasibility of standardized signalling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options:

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

RAN2 provided the response and requested RAN3/SA2/SA5 to validate RAN2’s assumption on non-OTA solutions.

In this contribution, we provide our interpretation of non-OTA solutions implemented via OAM.

- Discussion

In the RAN2#129b meeting, RAN2 defined two standardized approaches for network-side sharing of datasets and model parameters transmission:

- Alternative 1(non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)

- Alternative 2(OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

Regarding Alternative 1(non-OTA approach), RAN2 identified the following candidate solutions:

- Option 1: OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity

- Option 2: CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity

- Option 3: gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity

RAN2 in receiving LS (R3-253010) asks RAN3/SA2/SA5 to confirm its assumption on non-OTA solutions “From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption.”. Since Option 1 and Option 2 have no direct impact on RAN3 specifications, RAN3 focuses solely on analysing Option 3.

For Option 3, we analysed on the CN part. The OAM part can let SA5 to analyse as there is no OAM specification defined in RAN3.

From RAN3 perspective, the transfer of datasets or model parameters from gNB to CN (via AMF) can go through NGAP protocol, we may need to define a new NGAP procedure for such purpose. The AMF, as a CN entity, can act as a relay node for such data transferring. Additionally, RAN3 has also previously discussed data transfer from AMF to gNB to support RAN-selected UE in AI/ML positioning use cases.

Regarding the transfer from CN to the UE-side training entity (MNO or OTT server), the current NAS transport procedure with gNB-transparent messaging can be utilized to enable dataset or model parameter sharing between AMF and UE. As for how the UE transfers these datasets/model parameters to its servers, this falls beyond 3GPP’s scope.

Therefore, at least the procedure of gNB -> CN -> UE-side training entity in Option 3 using existing interfaces (NGAP and NAS) can be confirmed.

- Observation: From RAN3’s perspective, RAN3 can confirm that RAN2’s assumption on non-OTA solutions of gNB -> CN -> UE-side training entity can be supported within existing architecture framework.

The AMF’s transfer of datasets/model parameters to the UE-side training entity is transparent to the gNB, as per RAN3’s understanding.

Proposal: it is proposed to reply to RAN2 to answer as, RAN3 confirm that the assumption on the procedure of gNB -> CN -> UE-side training entity (a server inside MNO or an OTT server) can be supported within Rel-19 existing architecture framework

A draft LS reply is attached to this contribution.

- Conclusion

In this contribution, we discussed the issues of dataset and model parameter sharing. Observation and proposal are summarized as following:

- Observation: From RAN3’s perspective, RAN3 can confirm that RAN2’s assumption on non-OTA solutions of gNB -> CN -> UE-side training entity can be supported within existing architecture framework.

The AMF’s transfer of datasets/model parameters to the UE-side training entity is transparent to the gNB, as per RAN3’s understanding.

Proposal: it is proposed to reply to RAN2 to answer as, RAN3 confirm that the assumption on the procedure of gNB -> CN -> UE-side training entity (a server inside MNO or an OTT server) can be supported within Rel-19 existing architecture framework

3GPP TSG-RAN WG3 Meeting #128	R3-xxxxx

St Julian’s, Malta, May 19th-23rd, 2025

Title:	[draft] LS reply on signalling feasibility of dataset and parameter sharing

Response to:	R3-253010(R2-2503169)

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	NEC (to be RAN3)

To:	RAN2

Cc:	RAN1, SA2, SA3, SA5

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN3 thanks RAN2 for the LS on signalling feasibility of dataset and parameter sharing. RAN3 discussed the feasibility on the data collection procedure on gNB -> CN -> UE-side training entity. RAN3 confirm that the assumption on the procedure of gNB -> CN -> UE-side training entity (a server inside MNO or an OTT server) can be supported within Rel-19 existing architecture framework.

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

# 2. Action2:

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above information into account.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #129		25th Aug – 29th Aug 2025			Bangalore, India

TSG RAN WG3 Meeting #129bis		13th Oct – 17th Oct 2025			Prague, Czech