3GPP TSG-RAN WG3 #128	R3-253217

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		16.2

Title:		Discussion on the New A-IoT Area

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The AIoT area was discussed in the previous two meetings, and RAN3 has made the following progress:

RAN3#127: AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list. AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN. RAN3#127bis: Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

This contribution will discuss the open issues of the AIoT area.

# 2. Discussion

2.1	Discussion on the New A-IoT area

In the last meeting, RAN3 has agreed that the AIOTF becomes aware of gNB-supported A-IoT service areas through OAM configuration. However, the AIOTF should also maintain knowledge of the A-IoT areas supported by readers. Without this mapping information, when the AF provides a target A-IoT service area, the AIOTF can only identify the A-IoT enabled gNBs rather than the specific readers serving that area. Therefore, we propose that the AIOTF should be aware of the A-IoT area(s) supported by each reader by means of OAM.

Proposal 1: The AIoTF should be aware of the A-IoT area(s) supported by each reader by means of OAM.

A LS S2-2504288 from SA2 wants to check the definition of the “supported area” included in the A-IoT RAN information.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information. [SA2 reply]: - SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area. - SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

In the last meeting, it was agreed that for the supported area, a new A-IoT area should be adopted while reusing the Tracking Area should be excluded.

However, regarding the new A-IoT Area, RAN3 needs to choose from the following options:

- Option 1: The new A-IoT Area is encoded as a new A-IoT Area ID, which is defined as a TAC-like ID.

- Option 2: The new A-IoT Area is encoded as GNSS.

The new A-IoT Area should be used both in the A-IoT RAN information reporting from OAM to AIOTF and in the Request Service Area in the Inventory request message.

Considering Option 1 (new A-IoT Area ID), the relationship between the reader serving area and the new A-IoT Area should be similar to that of NR cells and Tracking Areas. Therefore, the reader serving area can be part of the new A-IoT Area (refer to Figure 1), and/or the reader serving area can support multiple new A-IoT Areas (refer to Figure 2), and/or a reader serving area could be the same as a new A-IoT Area, which depending on the implementation (refer to Figure 3). Thus, the supported area(s) of gNB and reader can be described by new A-IoT Area ID(s) in the A-IoT RAN information reporting via OAM. So that the A-IoT CN can know the mapping between new A-IoT Area ID(s) and reader serving areas, which enables the A-IoT CN can send Inventory Requests to the correct reader(s).

Figure 1: Reader serving area is part of the new A-IoT Area

Figure 2: The reader serving area supports multiple new A-IoT Areas

Figure 3: A reader serving area is the same as a new A-IoT Area

Observation 1: If the new A-IoT area ID is defined as a TAC-like ID, the reader serving area can be part of the new A-IoT area, and/or the reader serving area can support multiple the new A-IoT Area, and/or a reader serving area could be the same as a new A-IoT Area, which depending on the implementation.

Observation 2: For option 1, the reader serving area can be described by new A-IoT area ID(s), which enables the A-IoT CN can know the mapping between the new A-IoT area and readers serving area.

​​Regarding Option 2 (GNSS Encoding), while GNSS-based description offers greater precise in defining service areas and more accuracy geographical description of both Request Serving Areas and A-IoT-enabled gNB supported areas However, it provides no clear advantages over Option 1 (TAC-like ID), and it would introduce significant overhead in Inventory Request messages and A-IoT RAN information reporting.

The A-IoT Area ID provides more flexibility than the GNSS method for A-IoT service use cases. For example, different rooms or different floors in a building can correspond to different A-IoT areas. It is easy to assign distinct A-IoT Area IDs to specific rooms/floors of a building, but difficult to describe these same locations precisely using GNSS coordinates.

Observation 3: The A-IoT Area ID offers greater flexibility than the GNSS method and represents a more future-proof solution. Additionally, encoding the new A-IoT Area using GNSS would generate significant signaling overhead without providing clear advantages compared to encoding as A-IoT Area ID.

After evaluating both options, it is proposed to implement the new A-IoT Area using a TAC-like ID approach.

Proposal 2: The new A-IoT Area should be encoded as the new A-IoT Area ID, and the new A-IoT area ID is defined as a TAC-like ID.

Proposal 3: A draft LS R3-253218 to SA2 is provided based on the proposals.

# 3. Conclusion

Observation 1: If the new A-IoT area ID is defined as a TAC-like ID, the reader serving area can be part of the new A-IoT area, and/or the reader serving area can support multiple the new A-IoT Area, and/or a reader serving area could be the same as a new A-IoT Area, which depending on the implementation.

Observation 2: For option 1, the reader serving area can be described by new A-IoT area ID(s), which enables the A-IoT CN can know the mapping between the new A-IoT area and readers serving area.

Observation 3: The A-IoT Area ID offers greater flexibility than the GNSS method and represents a more future-proof solution. Additionally, encoding the new A-IoT Area using GNSS would generate significant signaling overhead without providing clear advantages compared to encoding as A-IoT Area ID.

Proposal 1: The AIOTF should be aware of the A-IoT area(s) supported by each reader by means of OAM.

Proposal 2: The new A-IoT Area should be encoded as the new A-IoT Area ID, and the new A-IoT area ID is defined as a TAC-like ID.

Proposal 3: A draft LS R3-253218 to SA2 is provided based on the proposals.

# References