3GPP TSG-RAN WG3 #128	R3-253219

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		16.2

Title:		Discussion on A-IoT Leftover Issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The following progress was made during the last RAN3 meeting:

Turn the WA to agreement, i.e., Including AIOTF information containers in NGAP. Enhancing the existing NGAP interface management procedures. Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed. Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers. General - Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements. - Define the Reader Index as INTEGER (1..65536, …). - Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. - It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID. About Inventory - WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - The Inventory Request Transfer IE, also includes the following: - Device Identification for Paging (to enable paging for single device, a group of devices, all devices) - Requested Service Area Information (A-IoT Area list and/or Reader list) - Inventory Assistance Information (Approximates Number of Target A-IoT Devices). - It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size” About Command after inventory - Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory. - For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure. - WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. - WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages. - Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages. - The Command Request Transfer IE, also includes the following: - An A-IoT NAS PDU - Command Assistance Information: (Estimate of Expected D2R Message Size) - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…) - Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device. - The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE. It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF. FFS on the Release/Cancel/End related procedure(s)/IE(s) for the A-IoT service/inventory/command. FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

This contribution will discuss the open issues of Ambient IoT.

# 2. Discussion

2.1	 Discussion on Parallel Command Request Procedures for Different Devices

In the previous meeting, an FFS was left open regarding whether to introduce Device Association or a Transaction ID (outside of Transfer IEs) to enable parallel Command Request procedures for different devices between gNB and AMF. This discussion raises two key questions:

- Should parallel Command Request procedures be allowed for different devices between gNB and A-IoT CN?

- If yes, should Device Association or Transaction ID be used to differentiate these parallel procedures?

We believe the supporting parallel Command Request procedures is necessary because the sequential execution of command procedures would result in relative long latency for the A-IoT service, so the target devices would experience longer waiting periods, which would negatively impact A-IoT device power consumption.

Proposal 1: RAN3 should agree to support parallel Command Request procedures for different devices between gNB and AMF.

If parallel procedures are adopted, either Device Association or Transaction ID could be used to correlate Command Request and Response messages for each A-IoT device. Before selecting between Device Association and Transaction ID, RAN3 should determine whether to support parallel Command procedures for a single device within an A-IoT session:

- If supported: Device Association is unavailable since one Device Association might related to multiple Command procedures for the single A-IoT device.

- If not supported: Both Device Association and Transaction ID are available.

Observation 1: The selection between Device Association and Transaction ID depends on supporting parallel procedures for single devices.

We think parallel procedures for a single device are unnecessary because, the A-IoT CN can encapsulate multiple commands in a single Request. Alternatively, the A-IoT CN can encapsulate single command data per Command Request and sending them in sequence. Both approaches have less specification impact than supporting parallel procedures.

Proposal 2: Parallel Command procedures for a single device should not be supported.

For differentiating parallel procedures across devices, the Device Association was already defined as "RAN NGAP Device ID", which provides simple and straightforward differentiation. Regarding to the Transaction ID, RAN3 never defined it before in the interface between RAN and CN, and it may have more impact on RAN3, so it is not preferred. In addition, the Device Association should exist outside Transfer IEs to enable AMF differentiation of parallel procedures.

Proposal 3:

- Prefer Device Association ("RAN NGAP Device ID") for differentiating parallel Command procedures

- Device Association should exist outside Transfer IEs.

2.2 Discussion on A-IoT RAN Information Reporting

During the previous meeting, the following options were proposed for reporting reader location as part of A-IoT RAN information to the AIOTF:

- Option 1: Geographical Coordinates IE specified in TS 38.455

- Option 2: NG-RAN Access Point Position IE defined in TS 38.455

- Option 3: NG-RAN High Accuracy Access Point Position IE defined in TS 38.455

- Option 4: Civic Address IE defined in TS 29.572

For the Option 1, the Geographical Coordinates IE supports two location accuracy levels that the normal accuracy, which refers to the NG-RAN Access Point position, and the high accuracy, which refers to the NG-RAN High Accuracy Access Point Position. So, the Option 1 including both the Option 2 and Option 3.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  | - |  |
| >Direct |  |  |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  | - |  |
| >>>normal accuracy |  |  |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. | - |  |
| >>>high accuracy |  |  |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.49 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. | - |  |

Regarding to the Option 4 (Civic Address IE), it provides another aspect to describe the reader location (in granularity of Room, floor, or Building), which may also available to the A-IoT CN.

Proposal 4: Reader location should be reported to AIOTF using both, Geographical Coordinates IE (for geospatial positioning) and Civic Address IE (for physical location context).

In terms of the “FFS on the NGAP signaling support to provide the A-IoT RAN Information to the AIOTF”, from technique point of view, the OAM-based solution has fully capable of handling the A-IoT RAN information exchange. While, no substantive technical advantages have been identified for implementing a signaling-based solution, so the signaling-based approach would introduce unnecessary complexity.

Proposal 5: RAN3 should not pursue NGAP signaling support for A-IoT RAN information transfer to AIOTF.

2.3 NG Interface Release for Direct Connection

For the direct A-IoT NG interface between gNB and AIOTF, the interface is only utilized when AIOTF triggers A-IoT services. Therefore, RAN3 needs to determine the appropriate management approach for this interface. Two options are under our consideration:

- Option 1: The A-IoT NG interface is established upon A-IoT service initiation from AIOTF and released when no subsequent services are required.

- Option 2: The A-IoT NG interface is established when TNL association becomes operational (legacy approach) and maintained continuously.

The legacy NG interface between gNB and AMF requires persistent maintenance because the RAN and AMF cannot predict downlink and uplink transmissions between UE and CN and real-time downlink and uplink data delivery must be guaranteed.

However, the A-IoT NG interface is different because the communication between A-IoT device and AIOTF only occurs during A-IoT service sessions, and there is no no strict latency requirements for the A-IoT UL/DL data transmission. Also, continuous interface maintenance consumes unnecessary network resources and generates more signaling overhead, we

Given these factors, we recommend that the A-IoT NG interface should be established when AIOTF initiates a service and released when no further services are configured.

Proposal 6: The A-IoT NG interface establishment shall be triggered by AIoT service initiation from AIOTF and released when no new services are required.

2.4 Discussion on Assistance Information

In the last meeting, it was agreed to include the Approximate Number of Target A-IoT Devices as assistance information in the Inventory Request message, while the Estimate of Expected D2R Message Size remains under discussion.

The Estimate of Expected D2R Message Size has quit straightforward benefit to resource scheduling, otherwise the reader would need to blindly allocate radio resources to the PDRCH.

Additionally, SA2 has agreed to include the Time Interval (defined in 23.700-13 section 8.1) as A-IoT assistance information to facilitate A-IoT device response aggregation. We believe this Time Interval parameter will also be beneficial for A-IoT device response reporting to the AIOTF.

The AIOTF may provide the AIoT aggregation assistance information to the AIoT RAN. Aggregation assistance information is used to assist in the aggregation of AIoT responses received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy, considering information provided from the AF, which consists of: - Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF.

Proposal 7: Both the Estimate of Expected D2R Message Size and Time Interval should be included as assistance information in the Inventory Request message.

# 3. Conclusion

Observation 1: The selection between Device Association and Transaction ID depends on supporting parallel procedures for single devices.

Proposal 1: RAN3 should agree to support parallel Command Request procedures for different devices between gNB and AMF.

Proposal 2: Parallel Command procedures for a single device should not be supported.

Proposal 3:

- Prefer Device Association ("RAN NGAP Device ID") for differentiating parallel Command procedures

- Device Association should exist outside Transfer IEs.

Proposal 4: Reader location should be reported to AIOTF using both, Geographical Coordinates IE (for geospatial positioning) and Civic Address IE (for physical location context).

Proposal 5: RAN3 should not pursue NGAP signaling support for A-IoT RAN information transfer to AIOTF.

Proposal 6: The A-IoT NG interface establishment shall be triggered by A-IoT service initiation from AIOTF and released when no new services are required.

Proposal 7: Both the Estimate of Expected D2R Message Size and Time Interval should be included as assistance information in the Inventory Request message.

# References