3GPP TSG-RAN WG3 #128	R3-253220

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		14.3

Title:		Discussion on NR NTN leftover Issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the previous meeting, RAN3 made the following progress on NR NTN regenerative payload enhancement:

Support RRC_INACTIVE UE in NTN by implementation in R19. No enhancements on location based CHO for NTN in R19. In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. No new NGAP suspend/resume procedures in R19. Work on the stage2/3 details…

There are some open issues left in RAN3. And this paper will discuss them.

# 2. Discussion

2.1 Discussion on the NG interruption of the hard feeder link switchover

In the last meeting, it was agreed to implement NG interface suspension and resume indications in the Downlink NG Transmission Control IE from the onboard gNB to AMF. This mechanism suspends downlink NG transmission during hard feeder link switchover that cause NG interface interruptions.

While, it should be noted that uplink NG transmission is equally affected by the hard feeder link switchover. The onboard gNB is aware of the NG interface interruption timing, which can implement uplink signaling buffering and transmit the buffered signaling once the NG interface resumes. Therefore, for the UL transmission, stage 2 statement is need and no further enhancement on stage 3. The corresponding TP is provided in the Annex A.

Observation 1: The Uplink NG transmission is equally affected by the hard feeder link switchover.

Proposal 1: For the UL NG transmission, stage 2 statement is need and no further enhancement on stage 3. The corresponding TP is provided in the Annex A.

However, the NG interface suspension mechanism may potentially affect handover procedures, particularly impacting the timer TNGRELOCprep and TNGRELOCoverall that defined in TS 38.413 that

9.6 Timers TNGRELOCprep - Specifies the maximum time for the Handover Preparation procedure in the source NG-RAN node. TNGRELOCoverall - Specifies the maximum time for the protection of the overall handover procedure in the source NG-RAN node.

Example 1: When the source NG-RAN node sends the Handover Required message to the AMF, it shall start timer TNGRELOCprep. If the NG interface is suspended (due to hard feeder link switchover) before the NG-RAN node receives the Handover Command message, TNGRELOCprep may expire before the NG interface resumes. Consequently, the source NG-RAN node will consider the handover preparation procedure failed and initiate the Handover Cancel procedure to the AMF with the cause value "TNGRELOCprep expiry".

Example 2: Timer TNGRELOCoverall starts upon the source NG-RAN's reception of the Handover Command message from the AMF, and stops upon its reception of the UE Context Release Command message from the AMF. If the NG interface is suspended before TNGRELOCoverall stops, the timer may expire due to the NG interface suspension. As a result, the source NG-RAN node will determine the handover procedure has failed and shall initiate the UE Context Release procedure to the AMF with the cause value "TNGRELOCoverall expiry".

Observation 2: NG interface suspension may cause timers TNGRELOCprep and TNGRELOCoverall to expire, leading to handover procedure failure.

The following proposals are presented to address the issues mentioned above:

- Option 1: Maintain current implementation. It has no Stage 2/3 specification changes, and accepting the fact that some potential handover failures during hard feeder link switchovers

- Option 2: Timer Suspension. Pausing TNGRELOCoverall/TNGRELOCprep timers during NG interface suspension to prevent unnecessary handover failures during switchover events

- Option 3: New cause value introduction. Adding a new cause value “NG interface suspension” for the handover failure. The handover procedure is failed because of NG interface suspension instead of the TNGRELOCprep configured an un-appropriate small value

Therefore, RAN3 should evaluate the three options above. And we slightly prefer the option 2 because it avoids some handover failure and some network and radio resource waste.

Proposal 2: RAN3 should consider the following solutions:

- Option 1: Maintain current implementation.

- Option 2: Timer Suspension.

- Option 3: New cause value introduction.

And our recommended approach is Option 2.

Proposal 3: The corresponding TP is provided in the Annex B.

# 3. Conclusion

Observation 1: The Uplink NG transmission is equally affected by the hard feeder link switchover.

Observation 2: NG interface suspension may cause timers TNGRELOCprep and TNGRELOCoverall to expire, leading to handover procedure failure.

Proposal 1: For the UL NG transmission, stage 2 statement is need and no further enhancement on stage 3. The corresponding TP is provided in the Annex A.

Proposal 2: RAN3 should consider the following solutions:

- Option 1: Maintain current implementation.

- Option 2: Timer Suspension.

- Option 3: New cause value introduction.

And our recommended approach is Option 2.

Proposal 3: The corresponding TP is provided in the Annex B.

# References

# Annex A: TP to BLCR for TS 38.300

--------------------------------------------------------------Start of the change-----------------------------------------------------------

## 16.14	Non-Terrestrial Networks

/******* Unchanged part skipped ******/

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links. For regenerative payload, the NG link interruption cause data and signaling procedure over NG interface to suspend.

#### 16.14.4.3	Procedures

he NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

# Annex B: TP to BLCR for TS 38.413

/******* Unchanged part skipped ******/

## 9.6	Timers

TNGRELOCprep

-	Specifies the maximum time for the Handover Preparation procedure in the source NG-RAN node.

TNGRELOCoverall

-	Specifies the maximum time for the protection of the overall handover procedure in the source NG-RAN node.

TXnRELOCOverall

-	Specified in TS 38.423 [24].

NOTE: The timers TNGRELOCprep and TNGRELOCoverall may be suspended during NG interface interruption caused by a hard feeder link switchover.

--------------------------------------------------------------End of the change----------------------------------------------------------