3GPP TSG-RAN WG3 Meeting #128	R3-253239

Malta, MT, 19th - 23th May 2025

Agenda Item:	15.2

Source:	ZTE Corporation

Title:	(TPs to BL CR for 36.300 and 36.413) Remaining issues for IoT NTN

Document for:	Discussion and Approval

1  Introduction

In this contribution, we continue on the issues of Store&Forward mechanism and provide our views on the support of full eNB as regenerative payload for IoT NTN.

2  Discussion

2.1 Support of S&F satellite operation

Based the WID [1], the impacted specs consist of TS 36.401 which is for Architecture description in LTE. Since our RAN3 is responsible for this spec, we can introduce the stage 2 description to support the Store and Forward satellite operation.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.211 | LTE; Physical channels and modulation | RAN#108 | Core part |
| 36.212 | LTE ; Multiplexing and channel coding | RAN#108 | Core part |
| 36.213 | LTE; Physical layer procedures for control | RAN#108 | Core part |
| 36.214 | LTE; Physical layer; Measurements | RAN#108 | Core part |
| 36.300 | LTE; Overall description; Stage-2: | RAN#109 | Core part |
| 36.304 | LTE; User Equipment (UE) procedures in idle mode and in RRC Inactive state | RAN#109 | Core part |
| 36.306 | LTE; User Equipment (UE) radio access capabilities | RAN#109 | Core part |
| 36.314 | LTE ; Layer 2 - Measurements | RAN#109 | Core part |
| 36.321 | LTE; Medium Access Control (MAC) protocol specification: | RAN#109 | Core part |
| 36.322 | LTE; Radio Link Control (RLC) protocol specification | RAN#109 | Core part |
| 36.331 | LTE; Radio Resource Control (RRC); Protocol specification | RAN#109 | Core part |
| 36.401 | LTE ; Architecture description | RAN#109 | Core part |
| 36.413 | Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 application protocol (S1AP) | RAN#109 | Core part |
| 36.423 | Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol (X2AP) | RAN#109 | Core part |
| 36.133 | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management | RAN#111 | Performance part |
| 36.108 | Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception | RAN#111 | Performance part |

Proposal 1: Introduce the stage 2 description of Store and Forward satellite operation in TS 36.401.

2.2 Support of full eNB as regenerative payload

In NR NTN, as given in TS38.412[4], since the multiple SCTP associations has already been supported over NG, the gNB IP address changes due to soft feeder link switch can be supported by existing NGAP procedures.

NG-RAN node and AMF shall support a configuration with a single SCTP association per NG-RAN node/AMF pair. Configurations with multiple SCTP endpoints per NG-RAN node/AMF pair should be supported. When configurations with multiple SCTP associations are supported, the AMF may request to dynamically add/remove SCTP associations between the NG-RAN node/AMF pair. Within the set of SCTP associations established between one AMF and NG-RAN node pair, the AMF may request the NG-RAN node to restrict the usage of SCTP association for certain types of NG-C signalling. If no restriction information is provided for an SCTP association, any type of NG-C signalling is allowed via the SCTP association. Selection of the SCTP association by the NG-RAN node and the AMF is specified in TS 23.501 [3] and TS 23.502 [4]. The NG-RAN node shall establish the SCTP association. The SCTP Destination Port number value to be used by NGAP is assigned by IANA in [11]. When the AMF requests to dynamically add additional SCTP associations between the NG-RAN node/AMF pair, the SCTP Destination Port number value may be the one assigned by IANA in [11], or any dynamic port value (IETF RFC 6335 [9]). When the configuration with multiple SCTP endpoints per NG-RAN node is supported and the NG-RAN node wants to add additional SCTP endpoints, the RAN configuration update procedure shall be the first NGAP procedure triggered on an additional TNLA of an already setup NG-C interface instance after the TNL association has become operational, and the AMF shall associate the TNLA to the NG-C interface instance using the included Global RAN node ID.

Currently, the multiple SCTP associations has not been supported over S1 by checking the TS 36.412 [5], which means the case of eNB IP address changes due to soft feeder link switch cannot be achieved by the existing S1AP procedure.

However, due to feature of service delay-tolerant, the soft feeder link switch is not necessarily supported in IoT NTN. And the hard soft feeder link switch is workable without any enhancement over S1. During the feeder link switch for IoT NTN, the MME can request to remove the existing SCTP association, and the eNB is able to establish a new SCTP association later.

Proposal 2: Considering the feature of service delay-tolerant, the multiple SCTP association is not necessarily supported in IoT NTN.

In last RAN3 meeting[2], a new indicator to suspend the NG connection has been introduced to solve the NG connection interruption issue due to hard FLSO for NR NTN. And this indicator could also been introduced in IoT NTN. In addition, the clarification on TNL management and other correction on the FLSO should be applied in IoT NTN

Proposal 3: The progress of FLSO in NR NTN should be introduced in IoT NTN .

3  Conclusion

Support of Store and Forward mechanism

Proposal 1: Introduce the stage 2 description of Store and Forward satellite operation in TS 36.401.

Support of full eNB as regenerative payload

Proposal 2: Considering the feature of service delay-tolerant, the multiple SCTP association is not necessarily supported in IoT NTN.

Proposal 3: The progress of FLSO in NR NTN should be introduced in IoT NTN .

The stage 2 CR for TS36.401 is given in [3].

The stage 2 and stage 3 CR on support of FLSO are given in the Annex

References

- RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

- RAN3#127bis Chairman Notes

- R3-253240, Support of Store and Forward Satellite operation, ZTE Corporation, Samsung, CMCC, NEC, CATT, Xiaomi

- TS 38.412 v18.1.0

- TS 36.412 v18.1.0

- TR 23.700-29 v19.0.0

Annex

TP for BL CR 36.300 on support of FLSO

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

23.21.5	Switchover

23.21.5.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption and/or S1 link interruption may occur during the transition between the feeder links.

23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs at feeder link switchover is performed by means of handover or TNL functionality, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, E-RAB Modification Indication procedure) may be used.

For regenerative payload, an indication to suspend or resume S1 connection may be sent to MME from eNB in the eNB Configuration Update procedure.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

TP for BL CR 36.413 on support of FLSO

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.7.4	eNB Configuration Update

8.7.4.1	General

The purpose of the eNB Configuration Update procedure is to update application level configuration data needed for the eNB and the MME to interoperate correctly on the S1 interface. This procedure does not affect existing UE-related contexts, if any.

8.7.4.2	Successful Operation

Figure 8.7.4.2-1: ENB Configuration Update procedure: Successful Operation.

>>> Unchanged part is skipped <<<

If the Connected en-gNB to be Removed List IE is included in the ENB CONFIGURATION UPDATE message, the MME shall remove any stored information for the concerned en-gNBs.

If the S1 Connection Control IE is included in the ENB CONFIGURATION UPDATE message, the MME shall, if supported, suspend or resume the S1 connection accordingly.

The updated configuration data shall be stored in both the eNB and the MME and used for the duration of the TNL association or until any further update is triggered by the eNB.

The eNB may initiate a further eNB Configuration Update procedure only after a previous eNB Configuration Update procedure has been completed.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.8.7	ENB CONFIGURATION UPDATE

This message is sent by the eNB to transfer updated information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 0..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Default Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |
| Connected en-gNB To Be Added List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the added en-gNB. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Connected en-gNB To Be Removed List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the en-gNB to be removed. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| S1 Connection Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the S1 connection. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |
| maxnoofConnecteden-gNBs | Maximum no. of en-gNBs connected to the eNB. Value is 256. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>