3GPP TSG-RAN WG3 Meeting #128	R3-253242

Malta, MT, 19th May– 23th May 2025

Agenda Item:	17.3

Source:	ZTE Corporation

Title:		(TP to BL CR for 38.423) Further discussion on on-demand SIB1

Document for:	Discussions & Approval

1	Introduction

During previous RAN3 meetings, a new XnAP Class 1 procedure, UL WUS Configuration Provision, was introduced to transfer the UL WUS configuration from the NES cell gNB to Cell A gNB. In addition, a new XnAP Class 2 procedure is to be introduced to allow the Cell A gNB to signal to the NES cell gNB the UL WUS configuration provision status.

In RAN3#127bis meeting, the following agreements were reached.

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

- One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

- This paper provides further discussion on on-demand SIB1, and provide the XnAP TP.

2	Discussion

2.1 XnAP procedure

In the current XnAP, a NG-RAN node can support up to 16384 cells.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

Considering that there may be multiple NES cells in a NG-RAN node, it is proposed that the UL WUS Configuration Provision message include a list of UL WUS configurations, with each configuration corresponding to one or more NES cells. Otherwise, if the message supports only a single UL WUS configuration, the NG-RAN node would need to trigger the provision procedure multiple times to transfer multiple configurations, leading to increased signaling load over XnAP.

Observation 1: a NG-RAN node can support up to 16384 cells. if the UL WUS Configuration Provision message supports only a single UL WUS configuration, the NG-RAN node would need to trigger the provision procedure multiple times to transfer multiple configurations, leading to increased signaling load over XnAP.

Proposal 1: For the "Start with UL WUS Configuration" operation, the UL WUS Configuration Provision Request message shall include a list of UL WUS configuration.

A NES Cell can be configured with one UL WUS configuration, The NES Cell ID associated with a UL WUS configuration is represented by a combination of the Physical Cell ID (PCI) and ARFCN. At RAN2#129 meeting, RAN2 agreed that "WUS configuration can be associated with a list of cells if the whole WUS configuration is same", as below:

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

To reflect the above highlighted agreement, in the latest RAN2 38.331 running CR [1] following the RAN2#129bis meeting, a list of PCIs sharing the same ARFCN(FFS if list of cells is ARFCN&PCI or only PCI) is associated with a UL WUS configuration (od-SIB1-Config-r19) in SIBxx, as shown below:

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

FFS: if list of cells is ARFCN&PCI or only PCI

Observation 2: In SIBxx, a WUS configuration can be associated with an area comprising a list of NES cells. This UL WUS Area Cell List is represented by a list of PCIs sharing the same ARFCN.

Based on above discussion, we give the following proposal.

Proposal 2: For the "Start with UL WUS Configuration" operation, for each UL WUS configuration in the list, the UL WUS Configuration Provision message includes :

a UL WUS Area Cell List, indicating the NES cells associated with same UL WUS configuration;

a UL WUS Assisting Cell List, indicating the assisting cells at Cell A gNB responsible for broadcasting this UL WUS configuration.

Consider "stop" choice in the UL WUS Configuration Provision message, the meaning is there are some previous OD-SIB1 NES cells no longer requiring assistance to broadcast UL WUS configuration, e.g., when on-demand SIB1 is disabled and periodic SIB1 transmission is resumed in these cells. Therefore, we give the following proposal.

Proposal 3: For the "stop" operation, the UL WUS Configuration Provision message includes a OD-SIB1 NES cell list no longer requiring assistance to broadcast its UL WUS configuration.

As discussed above, in SIBxx, a WUS configuration can be associated with an area comprising a list of NES cells. For the "stop" operation, the NES cell gNB may request to stop a subset of the previous indicated UL WUS Area Cell List. In this case, Cell A gNB shall re-encode the SIBxx and omit/not broadcast these OD-SIB1 Cell IDs(PCIs) for previous associated UL WUS configuration in SIBxx.

For example, a UL WUS area may consist of multiple NES cells with the same UL WUS configuration operating in OD-SIB1 NES mode at night for energy savings. However, due to varying daytime traffic patterns, certain sub-areas, such as those inside factory buildings, may require higher availability and thus exit OD-SIB1 mode to resume periodic SIB1 broadcasting. In contrast, outdoor or sparsely populated sub-areas may continue in OD-SIB1 mode due to low access demand. Consequently, the UL WUS provision may need to be dynamically start/stop in some sub-areas.

Proposal 4: For the "stop" operation, the NES cell gNB may request to stop UL WUS provision within a sub-area of the previously configured UL WUS Area Cell List. Subsequently, the NES cell gNB may request to re-start UL WUS provision for this sub-area without the need to re-provide the associated UL WUS configuration.

If not all OD-SIB1 cells associated with the same UL WUS configuration are stopped, Cell A gNB shall continue broadcasting the UL WUS configuration. Only when all associated cells have been stopped, Cell A gNB shall stop broadcasting and delete the UL WUS configuration.

Proposal 5: For the "stop" operation,if not all OD-SIB1 NES cells associated with the same UL WUS configuration are stopped, Cell A gNB shall continue broadcasting this UL WUS configuration together with available physical cell IDs. Only when all associated cells have been stopped, Cell A gNB shall stop broadcasting and delete the UL WUS configuration.

In previous RAN3 meeting, RAN3 agreed the following "Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). ".   A Cell A may broadcast more than one UL WUS configuration. To simplify provision response procedure, from our perspective, if Cell A is capable of assisting with UL WUS provision, it should accept all required UL WUS configurations; otherwise, it should reject all of them.

Proposal 6: Upon receiving the UL WUS Configuration Provision message with the "start" choice, if Cell A is capable of assisting with UL WUS provision, it shall accept all required UL WUS configurations; otherwise, it shall reject all of them.

Proposal 7: Upon receiving the UL WUS Configuration Provision message with the "start" choice, the Cell A gNB shall respond with a list of assisting cells that accepted the provision.

Based on the above discussion, we provide corresponding TP for TS38.423 in annex.

Proposal 8: RAN3 discuss the corresponding TP for TS38.423 in annex.

2.1 F1AP procedure

In the current F1AP BL CR, in order to enable UL WUS configuration transfer from the DU to the CU, RAN3 agreed to add the UL WUS Configuration Information to the Served Cell Information IE. However, there is no conclusion on the F1AP signaling to support DU broadcasting the UL WUS configuration of neighboring NES cell.

In previous RAN3 meetings, some companies proposed that the UL WUS configurations could be encoded in SIBxx by the CU and included in the CU System Information sent to the DU. We support this proposal, as it reuses the existing procedure and minimizes the impact on the F1 interface.

Proposal 9: Considering the F1AP signaling to support DU broadcasting of UL WUS configurations, it is proposed that the UL WUS configurations be encoded in SIBxx by the CU and included in the CU System Information provided to the DU.

Another issue is whether the CU can provide the DU with a list of cells that are allowed to enable OD-SIB1. We would like to highlight that a similar discussion in the OD-SSB topic. In that discussion, some companies have proposed that the CU could provide a list of allowed cells for OD-SSB. Given the similarity between these two cases, we believe the conclusions in both should be aligned for consistency.

Proposal 10: The conclusion on whether the CU can provide the DU with a list of cells allowed to enable OD-SIB1 should be aligned with the conclusion on the similar issue regarding OD-SSB.

3 Conclusion

For XnAP procedure:

Observation 1: a NG-RAN node can support up to 16384 cells. if the UL WUS Configuration Provision message supports only a single UL WUS configuration, the NG-RAN node would need to trigger the provision procedure multiple times to transfer multiple configurations, leading to increased signaling load over XnAP.

Proposal 1: For the "Start with UL WUS Configuration" operation, the UL WUS Configuration Provision Request message shall include a list of UL WUS configuration.

Observation 2: In SIBxx, a WUS configuration can be associated with an area comprising a list of NES cells. This UL WUS Area Cell List is represented by a list of PCIs sharing the same ARFCN.

Proposal 2: For the "Start with UL WUS Configuration" operation, for each UL WUS configuration in the list, the UL WUS Configuration Provision message includes :

a UL WUS Area Cell List, indicating the NES cells associated with same UL WUS configuration;

a UL WUS Assisting Cell List, indicating the assisting cells at Cell A gNB responsible for broadcasting this UL WUS configuration.

Proposal 3: For the "stop" operation, the UL WUS Configuration Provision message includes a OD-SIB1 NES cell list no longer requiring assistance to broadcast its UL WUS configuration.

Proposal 4: For the "stop" operation, the NES cell gNB may request to stop UL WUS provision within a sub-area of the previously configured UL WUS Area Cell List. Subsequently, the NES cell gNB may request to re-start UL WUS provision for this sub-area without the need to re-provide the associated UL WUS configuration.

Proposal 5: For the "stop" operation,if not all OD-SIB1 NES cells associated with the same UL WUS configuration are stopped, Cell A gNB shall continue broadcasting this UL WUS configuration together with available physical cell IDs. Only when all associated cells have been stopped, Cell A gNB shall stop broadcasting and delete the UL WUS configuration.

Proposal 6: Upon receiving the UL WUS Configuration Provision message with the "start" choice, if Cell A is capable of assisting with UL WUS provision, it shall accept all required UL WUS configurations; otherwise, it shall reject all of them.

Proposal 7: Upon receiving the UL WUS Configuration Provision message with the "start" choice, the Cell A gNB shall respond with a list of assisting cells that accepted the provision.

Proposal 8: RAN3 discuss the corresponding TP for TS38.423 in annex.

For F1AP procedure:

Proposal 9: Considering the F1AP signaling to support DU broadcasting of UL WUS configurations, it is proposed that the UL WUS configurations be encoded in SIBxx by the CU and included in the CU System Information provided to the DU.

Proposal 10: The conclusion on whether the CU can provide the DU with a list of cells allowed to enable OD-SIB1 should be aligned with the conclusion on the similar issue regarding OD-SSB.

4 Reference

- Running RRC CR NES Post129bis, RAN2

Annex: TP to BL CR for TS38.423

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

8.x.x	UL WUS Configuration Provision (FFS)

8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide a list of UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit the list of UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to provide a cell list to request NG-RAN node2 to stop transmission of UL WUS configuration associated with these cells.  (FFS)”

The procedure uses non UE-associated signaling.

8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2 to start, or stop the provision of UL WUS configuration .Upon receipt of the message, the NG-RAN node2 :

-	If the CHOICE Request Type IE in message is set to "start", the NG-RAN node2 shall broadcast the UL WUS configurations included in the UL WUS Configuration List IE, in the SIBxx of cells indicated by the UL WUS Assisting Cell List IE. The NG-RAN node2 shall respond with a list of assisting cells that accepted the provision in the UL WUS CONFIGURATION PROVISION RESPONSE message; or

-	If the CHOICE Request Type IE in message is set to "stop", the NG-RAN node2 shall not broadcast the physical cell IDs indicated in Cell List to Stop IE in SIBxx. If all associated cells for a UL WUS configuration have been stopped, NG-RAN node2 remove the UL WUS configuration.

Note: For the "stop" operation, the NG-RAN node2 may request to stop a sub-area of the previously configured UL WUS Area Cell List IE. Subsequently, the NES cell gNB may request to re-start UL WUS provision for this sub-area without the need to re-provide the associated UL WUS configuration.

8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the Cell A gNB (Naming FFS) cannot broadcast UL WUS Configuration in SIBx (Naming FFS), or a failure occurs during UL WUS Configuration Provision (Naming FFS), the Cell A gNB (Naming FFS) shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value.

8.x.x.4	Abnormal Conditions

Void.

8.x.y	UL WUS Configuration Transmission Status Update (FFS)

8.x.y.1	General

The purpose of the UL WUS Configuration transmission Status Update procedure is to enable an NG-RAN node2 to inform NG-RAN node1, the ongoing broadcasting of the SIBxx has stopped in one or more assisting cells.

The procedure uses non UE-associated signalling.

8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message to NG-RAN node2.

8.x.y.3	Unsuccessful Operation

Not applicable.

8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >> UL WUS Configuration List |  | 1..<maxnoofCellsinNG-RAN node> |  |  |  |  |
| >>> UL WUS Configuration | M |  | OCTET STRING | Includes the OD-SIB1-Config-r19 message as defined in subclause 6.3.1 of TS 38.331 [10]. |  |  |
| >>>UL WUS Area Cell List | M |  | 9.2.2.xxxx | List of cells associated with the same UL WUS configuration. |  |  |
| >>>UL WUS Assisting Cell List |  | 1..<maxnoofCellsinNG-RAN node> |  | List of assisting cells at the NG-RAN node2 for broadcasting the indicated UL WUS configuration. |  |  |
| >>>>NR CGI |  |  | 9.2.2.7 |  |  |  |
| >Stop |  |  |  |  |  |  |
| >> Cell List to Stop |  |  | 9.2.2.yyyy OD-SIB1 NES Cell List | List of OD-SIB1 cells no longer requiring assistance to broadcast UL WUS configuration, e.g., when on-demand SIB1 is disabled and periodic SIB1 transmission is resumed. |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.x.x.a	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by a NG-RAN node2 to a NG-RAN node1 to acknowledge the request of UL WUS configuration provision.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| UL WUS Assisting Cell List |  | 1..<maxnoofCellsinNG-RAN node> |  | List of assisting cells at the NG-RAN node2 that have successfully accepted the UL WUS configuration(s) for broadcast. | YES | ignore |
| >NR CGI |  |  | 9.2.2.7 |  |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

9.x.x.b	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to an NG-RAN node1 to indicate that the UL WUS configuration provision failure.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

9.x.x.c	UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE

This message is sent by an NG-RAN node2 to an NG-RAN node1 to indicate that the ongoing broadcasting of the SIBxx has stopped in one or more assisting cells in NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| UL WUS Assisting Cell List |  | 1..<maxnoofCellsinNG-RAN node> |  | List of assisting cells at the NG-RAN node2 that no longer broadcast SIBxx. | YES | ignore |
| >NR CGI |  |  | 9.2.2.7 |  |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

*************** Next change ***************

9.2.2.xxxx	UL WUS Area Cell List

This IE indicates the list of cells associated with the same UL WUS configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) |  |
| UL WUS Associated Cell Item |  | 1..<maxnoofULWUSAssociatedCells> |  |  |
| >NR PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |

| Range bound | Explanation |
| --- | --- |
| maxnoofULWUSAssociatedCells | Maximum no. cells that can be configure with the same UL WUS Congiruation. |

FFS: if list of cells is ARFCN&PCI or only PCI

*************** Next change ***************

9.2.2.yyyy	OD-SIB1 NES Cell List

This IE indicates the list of on-demand SIB1 NES cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NES Cell ID list |  | 1..<maxnoofCellsinNG-RANnode> |  |  |
| >NR PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |
| >NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

*************** End changes ***************