# 3GPP TSG-RAN WG3 Meeting #128	R3-253248

# Malta, MT, 19th – 23th May 2025

Title: 	(TP to BL CR 38.300, 38.410) New A-IoT procedures

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

The TPs are based on the following proposals.

Proposal 1: Introduce A-IoT service cancel procedure.

Proposal 2: Introduce A-IoT reader Status Reporting procedure.

The detail discussion is in the paper R3-253247

- TP to TS38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

16.xx.xa	A-IoT service cancellation procedure

Figure 16.xx.xa-1 depicts the basic communication between the gNB and the A-IoT CN node for the A-IoT service cancellation procedure.

Editor’s Note 5:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.xa-1: A-IoT service cancellation procedure

1.	The A-IoT CN node initiates the A-IoT service cancellation procedure over NG-C by sending the A-IoT service Cancel message to the gNB.

3.	The gNB releases the usage of A-IoT radio resources and performs the A-IoT service cancellation procedure over the A-IoT radio interface.

4.	The gNB replies to the A-IoT service cancel with the A-IoT service cancel acknowledge message.

16.xx.xb	A-IoT reader Status reporting procedure

Figure 16.xx.xb-1 depicts the basic communication between the gNB and the A-IoT CN node for the A-IoT reader Status reporting procedure.

Editor’s Note x:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.xb-1: A-IoT reader Status reporting procedure

1.	The A-IoT CN node initiates the A-IoT reader Status reporting procedure over NG-C by sending the A-IoT reader Status Request message to the gNB.

Editor’s Note 4:	Specification of further parameterisation of the A-IoT reader Status Request message needs further work.

2.	The gNB confirms the request from the A-IoT CN node by replying with the A-IoT reader Status Response message.

NOTE 2:	If the gNB is not able to perform the A-IoT reader Status reporting procedure, it rejects the request and sends an A-IoT reader Status Failure message to the A-IoT CN node.

3./4.	The gNB sends the A-IoT reader Status reporting message to the A-IoT CN node. If the A-IoT reader Status reporting procedure concerns e.g., reporting periodicity, multiple A-IoT reader Status Reports may be sent to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- TP to TS38.410

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.xx1	A-IoT procedures

The following procedures are used for A-IoT services:

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT service Cancellation;

-	A-IoT reader Status;

-	A-IoT reader Status Report.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>