3GPP TSG-RAN WG3 Meeting #128	R3-253255

Malta, MT, 19 – 23 May, 2025

Title: 	(TP to BLCR for TS 38.413, TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

# 1. Introduction

RAN2-129bis meeting sent a LS on the LP-WUS subgrouping with the following contents in [1].

# 1. Overall Description: RAN2 has discussed LP-WUS subgrouping and made the following agreements: LP-WUS UE-ID based subgrouping formula (RAN2#129): For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. UE-ID calculation (RAN2#129bis): - Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X. - X is based on 32 subgrouping number. Details can be discussed in the running CR. LP-WUS with eDRX case (RAN2#129bis): - LP-WUS is supported with eDRX, FFS on exact impact if any RAN2 assumes that the above information is necessary for RAN3 specification work (e.g., providing additional bits for the UE identity index value) and would like to request RAN3 to take them into account. # 2. Actions: # To RAN WG3: # RAN2 respectfully requests RAN3 to take the above information into account during the future work, and provide feedback, if any.

In this contribution, we continue to discuss the RAN3 related issues on the procedure and configuration for RRC_IDLE/INACTIVE UEs.

# 2. Discussion

## 2.1 LP-WUS Subgrouping

- During recent RAN1 meetings it was confirmed that the maximum number of subgroups in one PO for LP-WUS will be 31.

Agreement at RAN1#120 Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31. Agreement at RAN1#120bis For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is - 7 for the case where 4 POs are mapped to one LO - 15 for the case where 2 POs are mapped to one LO

With the above agreements, RAN3-127bis meeting agreed that the maximum number of LP-WUS CN-based subgrouping is 31.

- The maximum number of subgroups is decided to 31.

Regarding the exact range of LP-WUS CN Subgroup ID (either [0, 30] or [1, 31]), from the formula of the subgrouping ID indicated in [1], it can be deduced that the subgroup ID shall always start from 0 if there is no CN subgroup ID assigned to the UE. Consequently, it is more appropriate to define the ​​LP-WUS CN Subgroup ID​​ range as ​​0 to 30​​ in the LP-WUSPS Assistance Information via NGAP/XnAP/F1AP.

RAN3 to set the range of LP-WUS CN Subgroup ID from 0 to 30 in LP-WUSPS Assistance Information in NGAP/XnAP/F1AP.

In the current specification, the gNB will be provided by the core network or other gNB with an IE of Extended UE Identity Index Value carried in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, or in the XnAP RAN Paging message.

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

The Extended UE Identity Index Value IE comprises 16 least significant bits of 5G-S-TMSI (i.e., UE ID), in which up to 12 bits are used to determine the PF and PO (if configured with eDRX) and up to 15 bits (if configured with eDRX) are used for PEI subgrouping as specified in TS 38.304 [2]. According to the latest RAN2 meeting indicated in [1], it was agreed that the UE ID shall be determined by 5G-S-TMSI in the formula of UE_ID based subgrouping also considering the subgroupsNumForUEID for PEI. From the TS 38.331, the maximum number of the subgroupNumForUEID for PEI is eight. Hence in such case, at least 20 bits are used in UE ID for PO/PF determination, PEI subgrouping and LP-WUS subgrouping, the current Extended UE Identity Index Value IE with 16 bits is definitely insufficient. Therefore, RAN3 should introduce a new IE with at least a size of 20 bits for LP-WUS UE based subgrouping.

Observation 1: The current size of Extended UE Identity Index Value IE is not sufficient if both PEI and LP-WUS are supported by UE and gNB.

RAN3 to introduce a new Extended UE Identity Index Value IE with a size of 20 bits to fully carry the bit information of the 5G-S-TMSI. The corresponding TPs can be found in the contributions for this meeting.

## 2.2 The configuration of LP-WUS

In the latest RAN2 meeting, it was agreed that the LP-WUS related configurations are carried in SIB1 but except the RRM measurement configuration, i.e., the configuration for RRM measurement relaxation and serving cell measurement offloading [3].

Agreement at RAN2#129bis - All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. - Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

From RAN2 discussion, the LP-WUS related RRM measurement configuration mainly includes the following:

- Thresholds of the entry condition for RRM measurement relaxation

- Thresholds of the exit condition for RRM measurement relaxation

- Thresholds of the entry condition for RRM serving cell measurement offloading

- Thresholds of the exit condition for RRM serving cell measurement offloading

- (FFS) highPriorityMeasRelax to indicate whether UE can perform relaxed measurement on the neighbour cell with higher priority.

Currently RAN2 is still discussing whether to include the LP-WUS related RRM measurement configuration in SIB1 or in SIB3/4 (similar to Rel-16 RRM measurement relaxation criteria). For the CU/DU split case, if the RRM measurement configuration for LP-WUS is provided in SIB1, naturally it is up to gNB-DU configuration. Whereas if the related thresholds are configured in SIB3/4, then the gNB-CU shall be responsible to generate the configuration. RAN3 can further revisit the following agreement made during RAN3 previous meetings when RAN2 makes final decision. But it is expected there is no RAN3 impact no matter either way is taken.

- It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

RAN3 to further revisit the agreements on the measurement configurations generated by gNB-DU when RAN2 makes final conclusion.

## 2.3 LP-WUS and eDRX

As indicated in [1], the LP-WUS could be supported together with eDRX. From RAN3 perspective, the existing procedures could apply. Specifically,

- for CN paging, the CN can provide both the LP-WUSPS Assistance Information IE and NR Paging eDRX Information IE in the NG Paging message;

- for RAN paging, the CN can provide both LP-WUSPS Assistance Information IE and NR Paging eDRX Information IE in the Core Network Assistance Information for RRC INACTIVE IE, then NG-RAN node can decide and propagate the RAN eDRX accordingly, and perform LP-WUS paging during the RAN eDRX cycle and PTW window (if the eDRX cycle is greater than 10.24seconds).

RAN3 can consider to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

RAN3 can consider to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.  No further RAN3 signalling impact is foreseen.

## 2.4 Last used cell only

The agreed XnAP BLCR has editor notes whether to introduce the LP-WUS subgrouping capability related information into the XnAP RAN paging message.

For Rel-17 PEI, each cell may broadcast the lastUsedCellOnly IE in SIB1, to indicate that the PEI can only be used in the last used cell. Typically, for an inactive UE moved from the anchor cell to the new cell, if the new cell broadcasts lastUsedCellOnly IE, the UE will not monitor the PEI, and the new gNB will not send PEI for this UE when it receives Xn RAN paging message from the anchor cell. On the contrary, if the new cell does not broadcast the lastUsedCellOnly IE, the UE should monitor the PEI instead.

Due to the fact that the anchor cell does not know whether the neighbor cells broadcast this lastUsedCellOnly IE or not, it should always include the PEIPS Assistance Information in the RAN paging message. Then it is the receiving cell to decide to use PEI or not depending whether the lastUsedCellOnly IE is broadcast or not. Hence for LP-WUS, it seems a very same procedure would be adopted as PEI, but this needs to be by RAN2 first.

RAN3 to keep the editor’s note in the XnAP BLCR pending RAN2 progress.

## 2.5 Paging capability loss issue

The RAN2-129bis meeting sent a LS on the paging capability loss with the following contents in [4].

1. Overall Description: RAN2 further discussed the issue of UE radio paging capability loss mentioned in the LS S2-2412876 and acknowledged that based on the legacy signalling, a gNB implementing Release X may not be able to forward the radio paging capabilities for a feature introduced after Release X, which results in UE missing the paging message. For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”. 2. Actions: To SA2, RAN3, CT1 RAN2 respectfully asks SA2, RAN3, CT1 to take the updated conclusions and agreements above into account.

It can be observed for pre-R19 features, e.g., (e)RedCap, PEI etc, RAN2 will not change legacy UE behaviours, and it is up to network implementation to avoid the paging capability loss. While for R19 features, RAN2 will introduce a new R19 container, and would work only for gNBs that support this R19 container. From RAN3 perspective, on top of RAN2 decision, it can be confirmed that the paging capability loss issue is left to the network implementation (i.e., homogeneous support configured by the OAM) for pre-R19 features, and the new R19 features including LP-WUS.

RAN3 to confirm that the paging capability loss issue are left to the network implementation (e.g., OAM) for pre-R19 features, and the R19 LP-WUS.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- LP-WUS Subgrouping

- RAN3 to set the range of LP-WUS CN Subgroup ID from 0 to 30 in LP-WUSPS Assistance Information in NGAP/XnAP/F1AP.

Observation 1: The current size of Extended UE Identity Index Value IE is not sufficient if both PEI and LP-WUS are supported by UE and gNB.

- RAN3 to introduce a new Extended UE Identity Index Value IE with a size of 20 bits to fully carry the bit information of the 5G-S-TMSI. The corresponding TPs can be found in the contributions for this meeting.

- The configuration of LP-WUS

- RAN3 to further revisit the agreements on the measurement configurations generated by gNB-DU when RAN2 makes final conclusion.

- LP-WUS and eDRX

RAN3 can consider to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.  No further RAN3 signalling impact is foreseen.

- Last used cell only

RAN3 to keep the editor’s note in the XnAP BLCR pending RAN2 progress.

- Paging capability loss issue

RAN3 to confirm that the paging capability loss issue are left to the network implementation (e.g., OAM) for pre-R19 features, and the R19 LP-WUS.

The TPs to NGAP, XnAP and stage2 are provided in the Annex to capture the above agreements. And the F1AP CR is provided in [5].

# 4. Reference

- R2-2503182 LS on LP-WUS subgrouping progress, RAN2

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- RAN2#129bis chairman notes.

- R2-2503172 Reply LS on paging capability loss issue, RAN2

- R3-253256 (TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR, Huawei

# 5. TP to BLCR for TS 38.413 against R3-251572

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..30, …) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LoggedMDTTrigger ::= CHOICE{

periodical			NULL,

eventTrigger			EventTrigger,

choice-Extensions		ProtocolIE-SingleContainer { {LoggedMDTTrigger-ExtIEs} }

}

LoggedMDTTrigger-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LPWUSPSAssistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {LPWUSPSAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP to BLCR for TS 38.423 against R3-251571

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..30, …) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LPWUSPSassistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions				ProtocolExtensionContainer { { LPWUSPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 9.2.10	Extended DRX for RRC_IDLE and RRC_INACTIVE

When extended DRX (eDRX) is used, the following applies:

-	For RRC_INACTIVE, eDRX configuration for RAN paging is decided and configured by NG-RAN. In RRC_INACTIVE the UE monitors both RAN and CN paging;

-	For RRC_IDLE, eDRX for CN paging is configured by upper layers. In RRC_IDLE the UE monitors only CN paging;

-	Information on whether eDRX for CN paging and RAN paging is allowed on the cell is provided separately in system information;

-	The maximum value of the eDRX cycle is 10485.76 seconds (2.91 hours) while the minimum value of the eDRX cycle is 2.56 seconds;

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging according to DRX during a Paging Time Window (PTW). The PH and PTW are determined based on a formula (see TS 38.304 [10]) that is known by the AMF, UE and NG-RAN;

-	H-SFN, PH and PTW are used if the eDRX cycle is greater than 10.24 seconds;

-	When the RRC_IDLE eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before resuming/establishing an RRC connection;

-	The eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>