3GPP TSG-RAN WG3 Meeting #128	R3-253258

Malta, MT, 19 – 23 May, 2025

Title: 	(TP to BLCR for TS 38.473 and TS 38.470) Discussion on on-demand SSB SCell operation

Source: 	Huawei

Agenda item:	17.2

Document Type:	 Other

# 1. Introduction

The previous RAN3#126 meeting discussed the on-demand SSB SCell operation, with the following agreement.

- For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

In this contribution, we provide our considerations on on-demand SSB SCell operation, based on RAN1 and RAN2 progress.

# 2. Discussion

## 2.1 Allowed Cell list

The last RAN3-127bis meeting discussed whether and how to introduce the allowed cell list to allow on-demand SSB SCell with the following contents in [1].

It is proposed to take the following as baseline and to be discussed at the next meeting (no need to capture into the Chairman notes). Take the following texts as baseline: If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

There are two main issues discussed online as follows.

- Issue 1: Whether there is any NBC issue to reuse the legacy IE for both the SSB beam deactivation, and the OD-SSB SCell?

- Issue 2: Whether the DU should strictly follow the allowed Cell list, or make decisions for those cells not included in the Cell list?

For the first issue, we consider there is no NBC issue. The Cells Allowed to be Deactivated List IE was introduced in R18 WI. Based on the information, the DU can decide to deactivate some beams itself and then to notify CU the deactivated SSB beams. Since the intension is to deactivate some SSB beams, it is also possible that all SSB beams are allowed to be deactivated. That means, the existing IE already covers the scenario when the whole Cell could be switched off.

- Observation 1: The existing IE (Cells Allowed to be Deactivated List IE) already covers the scenario when the whole Cell could be switched off.

Then for the case 1 no always on SSB case, this legacy Cells allowed list IE can be reused to indicate these Cells which are allowed to be totally switch-off when the OD-SSB is not broadcast. For case 2 always on SSB case, the legacy IE can be reused to indicate these cells where the on-demand SSB could be activated or deactivated dynamically.

- There is no NBC issue for the CU to reuse the existing IE (Cells Allowed to be Deactivated List IE) to also indicate the allowed cells for on-demand SSB operation to the DU.

For the second issue, we consider that the logic could be the same as the Rel-18, where the DU behaviour is specified as follows. The DU would just consider those indicated Cells to deactivate the SSB beams. Then for the OD-SSB SCell, the same principles as well. Hence the corresponding procedure texts are provided as follows.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose. The gNB-DU shall, if supported also consider that it is allowed to enable the on-demand SSB SCell operation within the indicated Cells for network energy saving purpose as specified in TS 38.300 [xx].

Based on the above analysis, the existing IE could be reused to indicate the on-demand SSB SCell operation. With the cells allowed list, it is up to the DU to decide to only deactivate some SSB beams, or all SSB beams, or enable on-demand SSB SCell operation.

- The same R18 logic to use the Cells Allowed to be Deactivated List IE applies for R19, and RAN3 to agree the above stage3 procedure texts.

## 2.2 Other issues for Case 1 – No Always-on SSB Transmission

The operation of on-demand SSB SCell involves three potential triggering methods as detailed in [1]:

- Using an existing signal or channel for UE uplink wake-up signals,

- Cell on/off status communicated via backhaul,

- SCell activation/deactivation signaling.

For Case #1, the SSB is not continuously transmitted. When the SSB is not transmitted, this Cell can be regarded as deactivated cell due to the energy saving by the neighbouring nodes. According to the current XnAP specification, the procedure for such cell activation/deactivation via backhaul is already supported, allowing an NG-RAN node to request the activation of a cell by a neighbouring NG-RAN node. For the case discussed, the existing cell activation message can be re-used without changes to the specification.

With regards to the SMTC in case the SCell SSB broadcast status changes, RAN3 can be pending to RAN2 further progress, then decide if there is any impact.

For Case #1, with no always-on SSB transmission, RAN3 to agree no additional modifications to the Xn specification for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

For the CU/DU split architecture, the general on-demand SSB procedure over F1 is illustrated in Figure 1 below.

Figure 1: The general on-demand SSB procedure

For the RRC and MAC CE based signaling to indicate on-demand SSB transmission, below provides the latest progress in RAN1 and RAN2. Also, previous RAN3 meeting agreed that it is up to the DU to decide the activation/deactivation state of OD-SSB configuration for RRC based signalling.

Agreement@RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 FFS: Whether the value of N can be implicitly determined using a timer Agreement at RAN1#120bis For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions Agreement at RAN2#127bis meeting 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Agreement at RAN2#129bis meeting 1. For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). 2. A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. 3. For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. 4. OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. 5. RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS. 6. Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

It can be observed that multiple OD-SSB parameters (e.g., OD-SSB periodicity, od-ssb-PositionsInBurst, od-ssb-nrofBurst) can be configured in the RRC. The MAC CE can be used to indicate one configuration index for each SCell activating OD-SSB. The deactivation can be realized by explicit and implicit indication.

For MAC CE based signalling, the DU can send the MAC CE to the UE. Since such signalling is dynamic to trigger the activation/deactivation of OD-SBB transmission, it could be the DU to decide the activation/deactivation of OD-SSB transmission.

For MAC CE based signalling, it is the DU to decide to send the On-demand SSB activation (even the deactivation) MAC CE to the UE.

Due to the fact that the OD-SSB transmission could be used for synchronization, cell measurement, SCell activation etc in different scenarios. The notification to CU should be discussed by considering different scenarios’ requirement. For example, if the OD-SSB transmission is used for L3 measurement, there is a need to notify the CU so that the CU may be anticipated to receive L3 measurement results from the UE. RAN3 can discuss whether to reuse the existing Service Status IE including in-service, out-of service, or introduce a new IE.

When the DU sends on-demand SSB MAC CE to the UE and begins the SSB broadcast, the CU should be notified by the DU (e.g., CU may be anticipated to receive L3 measurement results from the UE).

- In addition, when the DU sends the on-demand SSB activation MAC CE to the UE, RAN3 can discuss how the DU decides and sends the SCell activation MAC CE to the UE. One possible way is to rely on the L1 measurement based on on-demand SSB, as agreed in RAN1 to use periodic, semi-persistent, and aperiodic L1 measurement reports by using existing CSI framework. Another way is to rely on the L3 measurement result. In this case, the DU may need the assistance from the CU to decide whether to activate the SCell or not.

RAN3 to discuss how the DU decides the SCell activation based on the L1 measurement and/or the L3 measurement result after sending the on-demand SSB activation MAC CE.

## 2.2 Other issues for Case 2 - Always-on SSB is periodically transmitted on the cell

For Case #2, the always-on SSB is periodically transmitted on the SCell. From RAN3 perspective, this cell can be considered as a normal working cell. For example, this SCell can be already activated for one UE while being deactivated for another. Also, RAN1 agreed that the always-on SSB could be multiplexed with on-demand SSB as follows.

Agreement at RAN1-118bis meeting For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain Agreement at RAN1#119 Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement at RAN1#119 Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Over Xn interface, neighbouring NG-RAN nodes can treat this cell as a normal served cell since there is always on SSB transmission, i.e., without deactivation indication. As a result, there is no need to activate such cell. The difference of AO-SSB and OD-SSB mainly are the time/frequency locations, for which RAN1 continues its discussion.

Similarly, over F1 interface, the CU can treat this cell as an activated and “in service” cell. Similar to case 1, RAN3 can wait for RAN2 further progress of the SMTC when the SSB periodicity is changed.  For the exact F1 impact, it is suggested that RAN3 can focus on the case1 first, then decide if any agreements can be applied to case 2 as well.

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress. The rest of F1 impact can be considered after case 1.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Allowed Cell list

- Observation 1: The existing IE (Cells Allowed to be Deactivated List IE) already covers the scenario when the whole Cell could be switched off.

- There is no NBC issue for the CU to reuse the existing IE (Cells Allowed to be Deactivated List IE) to also indicate the allowed cells for on-demand SSB operation to the DU.

- The same R18 logic to use the Cells Allowed to be Deactivated List IE applies for R19, and RAN3 to agree the above stage3 procedure texts.

Case#1: No always-on SSB transmission

For Case #1, with no always-on SSB transmission, RAN3 to agree no additional modifications to the Xn specification for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

For MAC CE based signalling, it is the DU to decide to send the On-demand SSB activation (even the deactivation) MAC CE to the UE.

When the DU sends on-demand SSB MAC CE to the UE and begins the SSB broadcast, the CU should be notified by the DU (e.g., CU may be anticipated to receive L3 measurement results from the UE).

RAN3 to discuss how the DU decides the SCell activation based on the L1 measurement and/or the L3 measurement result after sending the on-demand SSB activation MAC CE.

- Case#2: Always-on SSB is periodically transmitted on the cell

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress. The rest of F1 impact can be considered after case 1.

- The TPs for TS 38.473 and TS 38.470 to implement the Proposal 1 and 2 are provided in Annex.

# 3. Reference

- R3-252357 Summary of Offline Discussion on on-demand SSB SCell operation, Huawei (Moderator)

- Chair notes RAN1#117 eom4.

# 5. TP to BLCR for TS 38.473

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose. The gNB-DU shall, if supported also consider that it is allowed to enable the on-demand SSB SCell operation within the indicated Cells for network energy saving purpose as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP for TS 38.470

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU. The F1 setup function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams or to enable the on-demand SSB operation. With the gNB-CU Configuration Update function, energy saving with cell activation and cell deactivation can be supported as defined in TS 38.300 [8]. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>