3GPP TSG-RAN WG3 Meeting #128	R3-253260

Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0172 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of gNB-CU-UP ID IE in the GNB-CU-UP CONFIGURATION UPDATE message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With respect to the gNB-CU-UP ID IE handling in the GNB-CU-UP CONFIGURATION UPDATE message, the following procedure text is specified. - If the gNB-CU-UP ID IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall associate the TNLA to the E1 interface instance using the gNB-CU-UP ID. However, the procedural text is unnecessary as presence of gNB-CU-UP ID IE is mandatory. Further, it is incorrectly described as optional in Tabular. To align with the ASN.1, both the above procedural text “if … included”, and presence in Tabular are not correct. There are two alternatives for the procedural text: - Remove the procedural text (since we typically don’t have procedural text for mandatory IEs). - Change the “If xxx is included in the yyy message” to “Upon receiving xxx in the yyy message”. In this CR, it is proposed to use alternative a) to fix this issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the procedure texts regarding handling of gNB-CU-UP ID IE when included in the GNB-CU-UP CONFIGURATION UPDATE message. - Update the presence of the gNB-CU-UP ID in the Tabular to algin with ASN.1. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the gNB-CU-UP ID IE included in the GNB-CU-UP CONFIGURATION UPDATE message by removing the incorrect procedure texts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct descriptions and handllings of the gNB-CU-UP ID IE in the GNB-CU-UP CONFIGURATION UPDATE message and misalignment between ASN.1 and Tabular. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.2, 9.2.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

### 8.2.5	gNB-CU-UP Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU-UP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-UP and the gNB-CU-CP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU-UP Configuration Update procedure: Successful Operation.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall take this IE into account.

If the gNB-CU-UP Name IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU-UP. If the Extended gNB-CU-UP Name IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU-UP and shall ignore the gNB-CU-UP Name IE if also included.

If the GNB-CU-UP CONFIGURATION UPDATE message includes gNB-CU-UP TNLA To Remove List IE, the gNB-CU-CP shall, if supported, initiate removal of the TNL association(s) indicated by gNB-CU-CP TNL endpoint(s) and gNB-CU-UP TNL endpoint(s) if the TNLA Transport Layer Address gNB-CU-CP IE is present, or the TNL association(s) indicated by gNB-CU-UP TNL endpoint(s) if the TNLA Transport Layer Address gNB-CU-CP IE is absent:

- 	if the received TNLA Transport Layer Address IE includes the Port Number IE, the gNB-CU-UP TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU-UP TNL endpoints correspond to all gNB-CU-UP TNL endpoints identified by the Endpoint IP Address IE and any Port Number(s).

-	if the received TNLA Transport Layer Address gNB-CU-CP IE includes the Port Number IE, the gNB-CU-CP TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU-CP TNL endpoints correspond to all gNB-CU-CP TNL endpoints identified by the Endpoint IP Address IE and any Port Number(s).

If the GNB-CU-UP CONFIGURATION UPDATE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - |  |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - |  |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells for gNB CP-UP separation. | - |  |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - |  |
| >NPN Support Information | O |  | 9.3.1.83 | NOTE: This IE is not applicable to eNB-CP/eNB-UP and ng-eNB-CU-CP/ng-eNB-CU-UP | YES | reject |
| >Extended Slice Support List | O |  | 9.3.1.94 | Additional Supported S-NSSAIs per PLMN. | YES | reject |
| >Extended NR CGI Support List | O |  | 9.3.1.97 | Additional supported cells per PLMN. | YES | ignore |
| >ECGI Support List | O |  | 9.3.1.100 | Supported cells for eNB or ng-eNB CP-UP separation. | YES | ignore |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - |  |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - |  |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-UP Name | O |  | 9.3.1.96 |  | YES | ignore |
| gNB-CU-UP MBS Support Information | O |  | 9.3.1.110 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< For Information Only >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- GNB-CU-UP Configuration Update

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory }|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE optional	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-GNB-CU-UP-TNLA-To-Remove-List		CRITICALITY reject	TYPE GNB-CU-UP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-UP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-UP-Name		PRESENCE optional	}|

{ ID id-gNB-CU-UP-MBS-Support-Info			CRITICALITY reject	TYPE GNB-CU-UP-MBS-Support-Info	PRESENCE optional	},

...

}

GNB-CU-UP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-UP-TNLA-To-Remove-Item

Change Ends