3GPP TSG-RAN WG3 Meeting #128														            R3-253261

Malta, MT, May 19th – 23rd, 2025

Source			: 	      CEWiT

Agenda item	:    	20.2

Title 		:       Further Discussions on Case 3a

Document for: 	    Discussion and Decision

# 1.	Introduction

The Work Item on artificial intelligence/machine learning for NR air interface has the following objectives on AI/ML positioning [1]:

Provide specification support for the following aspects: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

The positioning case 3a with potential RAN3 specification impact is discussed in this document.

For the network-based positioning (Case 3a/3b), RAN3 made the following general agreements:

RAN3#125bis

The existing LCS framework is re-used for AI/ML positioning.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

Case 3a:

The AI/ML model (inference and training functions) is at the gNB.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

RAN3#126

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

RAN3#127

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement.

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

RAN3#127bis

Case 3a data collection:

An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned.

FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. Pending feedback from SA2.

# 2.	Case 3a

In RAN3#127bis meeting, the following call flow was agreed for the opportunistic method as part of the AI/ML model training. The RAN1#118bis meeting agreement is also added for reference.

Figure 1: Positioning Information collection for UEs under positioning

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

4.	The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

If the LMF is unable to provide the positioning information, the LMF sends a positioning failure indication to the gNB using the (new) NRPPa POSITIONING DATA COLLECTION FAILURE message.

RAN1#118bis Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

• (Mandatory) timing information;

• (Optional) Quality of the timing information;

◦ Existing IE “Timing Measurement Quality” can be reused.

• (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

The stage 3 impact for case 3a training phase is captured below.

9.1.4.2		MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node → LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Data Collection Indication | O |  | ENUMERATED(true, ...) |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

9.2.37										       TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

The highlighted parameters are the ones impacted. However, according to the RAN1 agreement mentioned above, the TRP Measurement Result information element already includes the required parameters. Hence, no specification impact is expected in RAN3.

For the opportunistic case agreed in RAN3, the data collection requirement needs to be conveyed by the NG-RAN during the ongoing positioning of a UE. This parameter needs to be captured in the NRPPa MEASUREMENT RESPONSE information element, as highlighted above.

A new NRPPa message POSITIONING DATA COLLECTION REPORT needs to be defined to send the information related to the UE location and the corresponding correlation information from LMF to gNB.

9.1.1.X		POSITIONING DATA COLLECTION REPORT

This message is sent by the LMF to report the information related to the UE location and the corresponding correlation information.

Direction: LMF → NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Location Information |  |  | 9.2.X |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

9.2.X		Location Information

This information element contains the information related to the UE location and the corresponding correlation information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Location Information (FFS) |  |  |  |  |  |  |

9.1.1.X		POSITIONING DATA COLLECTION FAILURE

This message is sent by the LMF to report the information related to the UE location and the corresponding correlation information.

Direction: LMF → NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

Proposal 1: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement response from gNB to LMF is as in legacy NRPPa, that is:

1. Timing information: mandatory IE

2. Quality of the timing information: optional IE

3. Time stamp: mandatory IE.

Proposal 2: Data Collection Indication parameter shall be added in the measurement response from the gNB to the LMF.

Proposal 3: A new NRPPa message Positioning Data Collection Report shall be defined to send the information related to the UE location and the corresponding correlation information from the LMF to the gNB.

Proposal 4: A new NRPPa message Positioning Data Collection Failure shall be defined for the LMF to indicate to the gNB that positioning information for data collection cannot be provided.

# 2.	Conclusion

The following proposals are made in this document.

Proposal 1: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement response from gNB to LMF is as in legacy NRPPa, that is:

1. Timing information: mandatory IE

2. Quality of the timing information: optional IE

3. Time stamp: mandatory IE.

Proposal 2: Data Collection Indication parameter shall be added in the measurement response from the gNB to the LMF.

Proposal 3: A new NRPPa message Positioning Data Collection Report shall be defined to send the information related to the UE location and the corresponding correlation information from the LMF to the gNB.

Proposal 4: A new NRPPa message Positioning Data Collection Failure shall be defined for the LMF to indicate to the gNB that positioning information for data collection cannot be provided.

# References

[1]	RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2]	3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2”.

[3]	R3-252487, “LS on AI/ML Positioning Case 3a”