3GPP TSG-RAN WG3 Meeting #128	R3-253284

Malta, MT, 19-23, May, 2025

Agenda item:	14.3

Source: 	Samsung

Title: 	Further discussion on support of regenerative payload for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of regenerative payload for NR NTN, and the following agreements and common understandings are achieved,

Support RRC_INACTIVE UE in NTN by implementation in R19.

No enhancements on location based CHO for NTN in R19.

Whether to support NG suspend/Resume?

Proposal 2, in case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used.

Proposal 3bis, Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

No new NGAP suspend/resume procedures in R19.

In this contribution, we further discuss the support of regenerative payload.

Discussion

Last several RAN3 meetings continued to discuss the NG interface management. And the following open issue is captured,

Whether to support NG suspend/Resume?

Proposal 3bis, Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

In addition, companies agreed to capture an indicator in NGAP RAN CONFIGURATION UPDATE message in the latest BL CR to 38.413 and 38.300 in R3-252525 and R3-252523 separately as follows,

BL CR to 38.413 in RAN CONFIGURATION UPDATE message:

| Downlink NG Transmission Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the downlink NG Transmission | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Editor’s Note: FFS whether this IE is needed.

BL CR to 38.300:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

Editor’s Note: FFS whether this text is needed.

It can be observed that, the BL CRs capture the suspend/resume indicator mainly for the purpose of avoiding DL transmission loss in case of hard feeder link switch over, which in our understanding is a valid case to introduce the suspend/resume indicator in RAN CONFIGURATION UPDATE message.

So this meeting, firstly we’d like to propose to confirm to introduce the suspend/resume indicator in RAN CONFIGURATION UPDATE message, at least to avoid DL packet loss in case of hard feeder link switch over.

Moreover, it has been discussed for several meetings on whether and how to support suspend/resume functionality from interface management perspective. And it should be noted that the companies also proposed a similar solution, i.e. introduce a suspend/resume indicator in RAN CONFIGURATION UPDATE message to support suspend/resume functionality for interface management. As a result, it is worth discussing whether we can use the newly introduced suspend/resume indicator for multiple purposes.

According to the discussion during last several RAN3 meetings, the proponents of NG suspend/resume mentioned that the introduction of NG suspend/resume will save transmission energy when recovering the connection between gNB and AMF since less information is expected to be transmitted compared to NG Setup procedure.

On the other hand, the introduction of NG suspend/resume will require much more storage on NG interface management related information for gNB on board, especially after several orbits, the gNB on board may need to store the NG interface related information with every AMF along the orbits. In addition, there’s also a concern that whether the original suspended information is still valid when the gNB on board connects back to a specific AMF.

In our understanding, the main extra information that is needed to be stored at the gNB onboard is the TNL association related information, which actually does not require too much storage space, even if the gNB onboard needs to maintain the interface related information with every AMF along the orbits.

In addition, it has already been agreed that the supported TA list of an on-board gNB could also be provided to AMF by OAM. And we do not see why the TNL association related information will be updated after the satellite travels back to provide the coverage for a specific location after one or several orbits. As a consequence, for now we do not identify real issue on why the suspend/resume indicator cannot be introduced for interface management purpose.

And the benefit of introducing suspend/resume indicator is clear: The TA supported list in NG SETUP REQUEST message is mandatory while optional in RAN CONFIGURATION UPDATE message. Even if the supported TA list of an on-board gNB has already been provided to AMF by OAM, the NG SETUP REQUEST message still has to send TA supported list; under such case, we will save many signalling overhead on TA supported list if the gNB sends RAN CONFIGURATION UPDATE message with one resume indicator.

As a result, from saving signalling overhead point of view, it is beneficial to introduce one suspend/resume indicator in RAN CONFIGURATION UPDATE message.

Proposal 1: Agree to introduce a suspend/resume indicator in RAN CONFIGURATION UPDATE message, for the following purposes,

- NG Suspend/Resume

- Avoid DL packet loss in case of hard FLSO.

If we want to adopt such indicator for multiple purposes, the IE name should be general enough.

Conclusion

In this paper, we provide discussion on support of regenerative payload for NR NTN. The following proposals are provided,

Proposal 1: Agree to introduce a suspend/resume indicator in RAN CONFIGURATION UPDATE message, for the following purposes,

- NG Suspend/Resume

- Avoid DL packet loss in case of hard FLSO.

References