3GPP TSG-RAN WG3 #128                                            R3-253293

Malta, MT, 19th – 23rd May, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.413 and TS38.300) Consideration on A-IoT area

Agenda Item:	16.2

Document for:	Decision

Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the Reader selection solution for AIoT Topology 1, corresponding agreements and working assumptions are listed below:

- Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

- FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

In this contribution, we will further discuss the Reader selection and signaling impact on support of Ambient IoT. Base on the discussion, we will provide the proposals accordingly.

Discussion

2.1. On providing A-IoT RAN Information to CN

AIOTF selects A-IoT RAN and optionally, a list of readers based on area information from AF and A-IoT RAN Information. The following open issue is about how to provide A-IoT RAN Information to CN.

FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

If AIOTF could know the A-IoT RAN related information, the selection of RAN node by AIOTF can be supported. If reader information including reader id, reader location, and probably the coverage information (service area) of each reader is known byAIOTF, RAN node selection and reader selection performed by AIOTF can be supported.

Besides the reader related information, the supported TAI list included in the NG Setup Request could also be considered for RAN node selection. We do not see any need to define and report the Supported A-IoT Areas to the AIoTF. Then, AIOTF could trigger A-IoT service(s) towards the selected RAN node and may also provide reader list.

Proposal 1: Supported TAI list and/or Reader list of the A-IoT RAN node could be used for A-IoT RAN selection and reader selection, no need to introduce a new “Supported A-IoT Areas”.

On providing A-IoT RAN related information to AIOTF, there are also two solutions:

- Signaling based solution.

- OAM based solution.

For the same reason as providing reader related information to AIOTF, OAM based solution is sufficient to provide the A-IoT RAN Information to AIOTF.

Proposal 2: OAM based solution is sufficient to provide necessary info for AIoTF to perform the RAN node selection and reader selection.

2.2. On providing Service Area Information to RAN

In last RAN3 meeting, A-IoT reader selection in AIOTF and AIoT RAN has been discussed, and there is an open issue on how to define new A-IoT Area.

It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID

First of all, we see it’s not necessary to define another Area ID to indicate the areas, no big difference with using the reader id (each reader has its own location, its own service area).

We understand that A-IoT Area is a kind of intended service area, more like GNSS description. Intended Service Area Information IE has been defined for MBS broadcasting in NR NTN, which represents for one or more circles or polygons, such kind of definition could be reused to indicate the A-IoT service area. Corresponding TP is provided in the Annex, RAN3 is requested to discuss and agree the TP.

Proposal 3: In the Inventory Request message, A-IoT Area could be encoded as Intended Service Area Information IE as defined for NR NTN MBS broadcasting.

If AIoTF includes only A-IoT Area in the Inventory Request, gNB should select proper reader(s) which serves the A-IoT Area to perform the Inventory procedure.

But if both the A-IoT area and the reader list are received in the Inventory Request, the expected behavior of the gNB is not clear, especially when the A-IoT area and the reader list are overlapped.

In our understanding, A-IoTF should either use A-IoT area to describe the intended GNSS area of inventory, or select proper readers (base on the intended service area, and the reader related information). It is not reasonable to include both area and the reader list in Inventory Request simultaneously.

If AIOTF could take the responsible for reader list selection, no need to provide A-IoT Area to gNB. Or else, it could provide A-IoT area directly, gNB is responsible for reader selection on behalf of the A-IoT area. Provision of both A-IoT area and reader list in the Inventory request may bring confusion, or making the reader selection more complex.

Proposal 4: AIoT area and Reader list should not be used simultaneously in an Inventory Request message.

Conclusions

In this contribution, we mainly discussed the issue on UE reader selection. Based on the discussion above, the following proposals are provided:

Proposal 1: Supported TAI list and/or Reader list of the A-IoT RAN node could be used for A-IoT RAN selection and reader selection, no need to introduce a new “Supported A-IoT Areas”.

Proposal 2: OAM based solution is sufficient to provide necessary info for AIoTF to perform the RAN node selection and reader selection.

Proposal 3: In the Inventory Request message, A-IoT Area could be encoded as Intended Service Area Information IE as defined for NR NTN MBS broadcasting.

Proposal 4: AIoT area and Reader list should not be present at the same time.

Reference

- TR 38.769 TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

TP to BL CR for TS38.413

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

- 9.3.3.xx3	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area | M |  | OCTET STRING | Encoded in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331 [18] |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP to BL CR for TS38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

16.xx	Support of Ambient IoT

- 16.xx.x1	Introduction

- 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE1:	It is not expected a deployment will deploy both direct connectivityand indirect connectivity.

-	In this version of the specification A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>