3GPP TSG-RAN WG3 #128                                           R3-253295

Malta, MT, 19th – 23rd May, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.413 and TS38.300) A-IoT command procedures

Agenda Item:	16.2

Document for:	Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the inventory and command procedure for AIoT Topology 1, corresponding agreements and working assumptions are listed below:

About Inventory

Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers.

- WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- The Inventory Request Transfer IE, also includes the following:

  - Device Identification for Paging (to enable paging for single device, a group of devices, all devices)

  - Requested Service Area Information (A-IoT Area list and/or Reader list)

  - Inventory Assistance Information (Approximates Number of Target A-IoT Devices).

  - It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

About Command after inventory

- Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory.

- For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure.

- WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

- Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages.

- The Command Request Transfer IE, also includes the following:

  - An A-IoT NAS PDU

  - Command Assistance Information: (Estimate of Expected D2R Message Size)

  - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

- In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

- The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE.

It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF..

In this contribution, we will further discuss the command procedures and signaling impact on support of Ambient IoT. Base on the discussion, we will provide the proposals accordingly.

- Discussion

For signalling message for A-IoT command procedure in Topology 1, there are command request and command response captured in candidate procedures.

For command related message, the following FFS is captured:

WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.

In our opinion, Command Report message is not necessary.

In case of writing device, the result of action “write” is included in command response message.

In case of reading device, the “data” as the result of read operation could also be include in command response message.

So, it is unnecessary to introduce Command Report procedure.

Proposal 1: No need to introduce Command Report procedure.

For the granularity of command procedure, we support per single device command procedure for the following reasons:

1. The AIoT command information is encapsulated as NAS PDU. For NAS PDU, it is per device design due to security consideration. So, it is not reasonable to send the same NAS PDU to multiple devices.

2. For command after inventory procedure, after receiving per device inventory report message, per device command procedure could be triggered at once. This solution is simple and quick.

Proposal 2: It is proposed to only introduce per device command request procedure, not support a single command for a group of devices, or multiple devices.

To correlate inventory and the following command procedure, Correlation Identifier could also be used as the agreement below:

For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure.

But in case of multiple devices inventory and the following per device triggered command procedure, only Correlation Identifier is not sufficient to correlate per device command procedure. The following FFS is captured in last RAN3 meeting:

It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

Correlation Identifier could be used to correlate inventory and command procedure. Within command procedure, device association may be used as the following agreements.

Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE.

In inventory report message, besides Correlation Identifier, RAN NGAP Device ID as device association is sent to AIOTF. In the following command request message, RAN NGAP Device ID is included to indicate the same device. With this information, parallel Command Request procedures after inventory could be triggered between NB and AIOTF.

Proposal 3: It is proposed to introduce device association inside of the Transfer IEs to support parallel Command Request procedures for command after inventory.

As for CN NGAP Device ID, whether to include it in Command Request after inventory was also disused in last RAN3 meeting. In our understanding, it is unnecessary for CN to allocate CN NGAP Device ID again because only RAN NGAP Device ID is enough to indicate the per-session per-device Device Association.

Proposal 4: No need to introduce CN NGAP Device ID for device context.

There is an FFS on whether introducing Release/Cancel/End related procedure for Command after inventory as below:

FFS on the Release/Cancel/End related procedure(s)/IE(s) for the A-IoT service/inventory/command.

In our understanding, there may be two cases needs Release/Cancel/End related procedure from CN to RAN.

Case1: AIOTF triggers inventory to multiple devices, after inventory report is sent to CN, only partial devices would be selected to perform command procedure. While for the other parts of devices, RAN NGAP Device ID as device context would still be saved in RAN. In order to release device context, Release/Cancel/End related procedure(s)/IE(s) for these A-IoT devices may be needed.

Case2: There may be multiple command messages after inventory for a single device. Then RAN do not know which command message is the last one and whether it is needed to save device context for the following command message. For this case, from RAN2 perspective, Release/Cancel/End related procedure(s)/IE(s) may be used to indicate the end of command procedure, i.e. release device context in RAN.

In our opinion, device context may be released by RAN implementation, for example, based on timer, if no more command message received during a period of time, RAN would release device context. But considering releasing device context as soon as possible, signalling method may be also necessary.

Proposal 5: Beneficial to clearly indicates gNB the end of the Command procedures.

On how to indicate gNB end of command procedures, the possible solutions are provided below:

- Solution 1: Introducing a new Release/Cancel/End related procedure(s) for A-IoT command after inventory.

- Solution 2: Add a new indicator in Command Request message to indicate end of the command procedure.

- Solution 3: Add a new indicator in Command Request message to indicate there’s follow-up command PDU is still to be transferred.

For the solution 1, new Release/Cancel/End procedure could be used to indicate the end of a command procedure for a transaction, correlation or a device (RAN NGAP Device ID). When receiving this message, RAN should release related device context and related resources.

For the solution 2, the new indicator could be used to indicate the end of a command procedure for a transaction, correlation or RAN NGAP Device ID. When receiving this indication, RAN should release related device context and related resources, and then send command response message to CN.

For the solution 3, with this follow-up indication, RAN should keep device context and related resource for subsequent command message, otherwise, RAN should release device context and related resources after the command procedure is completed.

We slightly prefer solution2 and 3 for simplicity.

Proposal 6: It is proposed for RAN3 to discuss above solutions to indicate end of command procedures. We slightly prefer to add an indicator in the Command Request (solution 2 or 3) for simplicity.

As for command type, the FFS is captured below:

It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

For RAN, whether command type is needed should be decided by RAN2, but currently it seems unnecessary.

For SA2, Command Type is not included in the assistance information provided to NG-RAN node according to TS23.369 [3].

Proposal 7: No need to include command Type in assistance Information provided to NG-RAN node.

- Conclusions

In this contribution, we mainly discussed A-IoT command Procedures and Information exchange between A-IoT CN and A-IoT RAN. Based on the discussion above, the following proposals are provided:

Proposal 1: No need to introduce Command Report procedure.

Proposal 2: It is proposed to only introduce per device command request procedure, not support a single command for a group of devices, or multiple devices.

Proposal 3: It is proposed to introduce device association inside of the Transfer IEs to support parallel Command Request procedures for command after inventory.

Proposal 4: No need to introduce CN NGAP Device ID for device context.

Proposal 5: Beneficial to clearly indicates gNB the end of the Command procedures.

Proposal 6: It is proposed for RAN3 to discuss above solutions to indicate end of command procedures. We slightly prefer to add an indicator in the Command Request (solution 2 or 3) for simplicity.

Proposal 7: No need to include command Type in assistance Information provided to NG-RAN node.

- Reference

[1]TR 38.769Study on solutions for ambient IoT (Internet of Things)

[2]RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] TS23.369 V0.3.0(2025-04)“Ambient power-enabled Internet of Things”

- TP to BL CR for TS38.413

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

8.xx.3	Command Request

8.xx.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command to the A-IoT device over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

8.xx.3.2	Successful Operation

Figure 8.xx.3.2-1: Command Request, successful operation.

The A-IoT CN node triggers the procedure by sending a COMMAND REQUEST message to the NG-RAN node. In case of successful operation, the NG-RAN node responds with the COMMAND RESPONSE message.

If the Command Assistance Information IE is included in the COMMAND REQUEST message, the NG-RAN node, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

If the End Indicator IE is set to “true” in the COMMAND REQUEST message, the NG-RAN node should release device context and related resource after complete this Command procedure.

[Alternative solution: If the Following-up Indicator IE is set to “true” in the COMMAND REQUEST message, the NG-RAN node should keep the device context and related resource for subsequent Command procedures. Or else, NG-RAN node should release the device context and related resource after complete this Command procedure.]

Editor’s Note: FFS on which indicator is better, End Indicator or Following-up Indicator?

8.xx.3.3	Unsuccessful Operation

Figure 8.xx.3.3-1: Command Request, unsuccessful operation.

If the NG-RAN node is not able to transmit the received command, it shall send the COMMAND FAILURE message with an appropriate cause value.

8.xx.3.4		Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<< Next changes >>>>>>>>>>>>>>>>>>>>>>>

9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| End Indicator(or Following-up Indicator) | O |  | ENUMERATED (true, …) | FFS on which indicator is feasible, End Indicator or Following-up Indicator? FFS on the granularity, a correlation or a device? | YES | reject |

Editor’s note: FFS whether the Command Request also contains outside of the Command Request Transfer container, the RAN A-IoT device NGAP ID (FFS for CN NGAP device ID) which was received in the Inventory Report message or some additional transaction ID.

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- TP to BL CR for TS38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned deviceandpotentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

TheCommand Request also includestheRAN A-IoT device NGAP ID and the A-IoT NAS PDU targeting the A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier and the Correlation ID received in the Command Request and the RAN A-IoT device NGAP ID.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

NOTE 6:	AIoTF may indicate gNB the end of a Command procedure by include an indicator in the COMMAND REQUEST message.

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>