3GPP TSG-RAN WG3 Meeting #128                           	  R3-253297

Malta, MT, 19th – 23rd May, 2025

Source:	CATT, ZTE Corporation

Title:	(TP to BL CR for TS38.305) RAN3 inputs for UE-based AI positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

Introduction

For the UE-based AI/ML positioning (case 1), it’s mainly discussed in RAN2. Currently, RAN2 defined a new AI/ML positioning method, and start to capture it in the TS 38.305 (RAN2 running CR in [2]).

However, it seems necessary to involve RAN3 to provide some inputs to complete the UE-based AI/ML Positioning method as defined by RAN2, e.g. on the Information that may be transferred from the gNB to LMF, and Procedures between LMF and gNB. Refer to the RAN2 Running CR as below:

## 8.X UE-based AI/ML positioning ### 8.X.1 General In the AI/ML positioning method, the UE position is predicted by AI/ML model. How AI/ML model is used for inference of UE position is up to implementation and beyond the scope of this specification. …… #### 8.X.2.2 Information that may be transferred from the UE to LMF Editor's note: FFS what information may need to be transferred from the UE to LMF, e.g., the UE location and time stamp, which depends on RAN1 parameter list or RAN2 further discussion. #### 8.X.2.3 Information that may be transferred from the gNB to LMF Editor's note: Information that may be transferred from the gNB to LMF depends on RAN1 and RAN3 input. …… ### 8.X.3 AI/ML Positioning Procedures #### 8.X.3.0 General The procedures described in this clause support UE-based AI/ML positioning. #### 8.X.3.1 Procedures between LMF and UE …. #### 8.X.3.2 Procedures between LMF and gNB Editor's note: depending on further RAN1/RAN3 input.

In this contribution, we will try to provide some input from RAN3 perspective to make the UE-based AI positioning method complete.

Discussion

As for case 1, RAN2 defined a new Positioning method for UE-based AI Positioning on top of DL-TDOA. RAN2 expects RAN3 provides input for Information that may be transferred from the gNB to LMF and Procedures between LMF and gNB.

Observation 1: A new positioning method is defined for UE-based AI/ML positioning, RAN3 inputs are expected.

Editor's note: Information that may be transferred from the gNB to LMF depends on RAN1 and RAN3 input.

For the new defined, the positioning parameters and basic procedures of DL-TDOA are reused. On support of DL-TDOA method, assistance data transferred from gNB to the LMF includes TRP related information, DL-PRS configuration. As there’s no extra requirement to make support of UE-based AI positioning method as defined by RAN2, the information exchange between NG-RAN and LMF could be same as DL-TDOA, as the parameters listed in Table 8.12.2.3.0-1 of TS38.305.

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.12.2.3.0-1.

Table 8.12.2.3.0-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, ARFCN, and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation

TRP Tx TEG association information

Mobile TRP Location Information

Mobile IAB-MT UE ID NOTE 1

NOTE 1: If TRP Type is Mobile TRP.

Proposal 1: For the UE UE-based AI positioning, the Assistance data that may be transferred from gNB to the LMF are same as the one for DL-TDOA.

8.X.3.2	Procedures between LMF and gNB

Editor's note: depending on further RAN1/RAN3 input.

From RAN3 perspective, no new procedures between LMF and gNB need to be defined for UE-based AI Positioning, the TRP Information Exchange related procedure could be reused, as below:

#### 8.12.3.2	Procedures between LMF and gNB

##### 8.12.3.2.1	Assistance Data Delivery procedure

###### 8.12.3.2.1.0	General

The purpose of this procedure is to enable the gNB to provide assistance data to the LMF, for subsequent delivery to the UE using the procedures of clause 8.12.3.1.2 or for use in the calculation of positioning estimates at the LMF.

###### 8.12.3.2.1.1	LMF-initiated assistance data delivery to the LMF

Figure 8.12.3.2.1.1-1 shows the TRP Information Exchange operation from the gNB to the LMF for the DL-TDOA positioning method.

Figure 8.12.3.2.1.1-1: LMF-initiated TRP Information Exchange Procedure

(1)	The LMF determines that certain TRP configuration information is desired (e.g., as part of a periodic update or as triggered by OAM) and sends an NRPPa TRP INFORMATION REQUEST message to the gNB. This request includes an indication of which specific TRP configuration information is requested.

(2)	The gNB provides the requested TRP information in an NRPPa TRP INFORMATION RESPONSE message, if available at the gNB. If the gNB is not able to provide any information, it returns an TRP INFORMATION FAILURE message indicating the cause of the failure.

Proposal 2: LMF-initiated TRP Information Exchange Procedure used for DL-TDOA could also be applied to UE-based AI Positioning.

Base on the discussion above, we provide the corresponding TP to BL CR for TS38.305 to complete the UE-based AI Positioning method. Compared to simply add the reference to the legacy sections of DL-TDOA, we prefer to make a copy of the tabular and the procedures from DL-TDOA to the new positioning method, to make it future proof (e.g. the design of this new method is updated later but the DL-TDOA stays as it is).

Proposal 3: Discuss and agree the TP to BL CR for TS38.305 to provide the RAN3 inputs for UE-based AI Positioning method.

Conclusion

Observation 1: A new positioning method is defined for UE-based AI/ML positioning, RAN3 inputs are expected.

Proposal 1: For the UE UE-based AI positioning, the Assistance data that may be transferred from gNB to the LMF are same as the one for DL-TDOA.

Proposal 2: LMF-initiated TRP Information Exchange Procedure used for DL-TDOA could also be applied to UE-based AI Positioning.

Proposal 3: Discuss and agree the TP to BL CR for TS38.305 to provide the RAN3 inputs for UE-based AI Positioning method.

Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R2-2503587, 38.305 running CR for AIML Positioning, CATT

- Annex A. TP to BL CR for TS38.305

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 8.X.2.3	Information that may be transferred from the gNB to LMF

##### 8.x.2.3.0	General

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.x.2.3.0-1.

Table 8.12.2.3.0-1: Assistance data that may be transferred from gNB to the LMF

- Information

PCI, GCI, ARFCN, and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation

TRP Tx TEG association information

Mobile TRP Location Information

Mobile IAB-MT UE ID NOTE 1

NOTE 1: If TRP Type is Mobile TRP.

Editor’s Note: The content may need to be refined pending to other WGs.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 8.X.3.2	Procedures between LMF and gNB

##### 8.x.3.2.1 Assistance Data Delivery procedure

###### 8.x.3.2.1.0 General

The purpose of this procedure is to enable the gNB to provide assistance data to the LMF, for subsequent delivery to the UE using the procedures of clause 8.x.3.1.2 or for use in the calculation of positioning estimates at the LMF.

###### 8.x.3.2.1.1 LMF-initiated assistance data delivery to the LMF

Figure 8.x.3.2.1.1-1 shows the TRP Information Exchange operation from the gNB to the LMF for the UE-based AI/ML positioning method.

Figure 8.x.3.2.1.1-1: LMF-initiated TRP Information Exchange Procedure

(1)	The LMF determines that certain TRP configuration information is desired (e.g., as part of a periodic update or as triggered by OAM) and sends an NRPPa TRP INFORMATION REQUEST message to the gNB. This request includes an indication of which specific TRP configuration information is requested.

(2)	The gNB provides the requested TRP information in an NRPPa TRP INFORMATION RESPONSE message, if available at the gNB. If the gNB is not able to provide any information, it returns an TRP INFORMATION FAILURE message indicating the cause of the failure.

Editor’s Note: The content may need to be refined pending to other WGs.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>