3GPP TSG-RAN WG3 Meeting #128                           	  R3-253298

Malta, MT, 19th – 23rd May, 2025

Source:	CATT

Title:	(TP to BL CRs) Support of gNB-based AI positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

1 Introduction

In the previous RAN3 meetings, we discussed the potential RAN3 impact to support AI/ML assisted Positioning, i.e. the Case 3a. According to the discussion [1], some agreements and open issues as below:

Case 3a: The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. RAN3#127bis: Case 3a data collection: An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned. FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. Pending feedback from SA2. Check progress in RAN1 and solution discussion…

In this contribution, we will discuss the potential RAN3 impact to support AI/ML assisted positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals.

2 Discussion

2.2 Training data collection

On training data collection for case 3a, we discussed “opportunistic” solutions and “proactive” solutions in the previous meetings. And we agreed and capture an “opportunistic” solution in our BL CR, which only taking the ongoing positioning measurement into account for data collection, the call flow in the BL CR is copied below for information.

Figure 7.x.2-1: Positioning Data Collection Procedure

On the stage 3 details, we can introduce a new indicator in NRPPa Measurement Response to indicate Positioning Data Collection Needed.

Proposal 1: Introduce a new IE Positioning Data Collection Needed in MEASUREMENT RESPONSE.

When the requested Positioning data(Part B) is available in the LMF, LMF provides it to the gNB via the new class 2 message POSITIONING DATA COLLECTION REPORT.

Proposal 2: Define a new class 2 message POSITIONING DATA COLLECTION REPORT to provide the data collection information from LMF to gNB.

To associate with the previous Measurement procedure, LMF Measurement ID and RAN Measurement ID could be used for association.

Proposal 3: Include LMF Measurement ID and RAN Measurement ID in POSITIONING DATA COLLECTION REPORT to associate the Part A and Part B.

On the definition of Part B, RAN1 agreements could be referred as below:

Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

As above, the Part B information includes “Timing Measurement Quality” and “LOS/NLOS Information”. According to the existing Positioning measurement procedures, the above info should be per TRP level, thus we should provide the “Timing Measurement Quality” and “LOS/NLOS Information” info per TRP in the POSITIONING DATA COLLECTION REPORT message.

Proposal 4: Include “Timing Measurement Quality” and “LOS/NLOS Information” per TRP level in the POSITIONING DATA COLLECTION REPORT message.

As above, the corresponding TP to BL CR for TS38.413 is provided in the Annex 1, the TP to BL CR for TS38.455 is provided in Annex 2. RAN3 is requested to discuss and agree the stage 3 TPs to support the opportunistic solution.

Proposal 5: RAN3 is requested to discuss and agree the stage 3 TPs to support the opportunistic solution.

On whether and how to support of Proactive solution:

FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. Pending feedback from SA2.

First of all, we should discuss and decide whether to support Proactive data collection. According to previous discussion, the benefit is gNB can obtain a large amount of user data in a short period to satisfy the requirement of model training. However, some companies believed that the data collection for model training is not a urgent task need to be completed within a few seconds, maybe a long term collection is good enough. If yes, the “opportunistic” data collection by reusing the on-going positioning procedures as we agreed and captured in the stage 2 BL CR is sufficient.

Proposal 6: Discuss and decide whether the “opportunistic” solution is sufficient, whether need to support “Proactive” data collection.

On support of Proactive solution, a key issue is which node is responsible for label UE selection.

From gNB perspective, it knows the served UEs, but it does not know the UEs served by its neighbour gNBs.

Observation 1: If gNB is responsible to select UEs for data collection, it could only select its served UEs, not able to select the UEs from the other gNBs (e.g. neighbour gNBs).

In the TS 38.305, it’s clearly specified that NG-RAN node functions as an LCS client is not supported, as below:

The case that the NG-RAN node functions as an LCS client is not supported in this version of the specification.

And we assume we should not break this principle for AI/ML assisted Positioning.

Observation 2: In current positioning policy, NG-RAN node functions as an LCS client is not supported.

If allow gNB to select UE(s) and request for data collection, which means gNB could trigger Positioning procedure towards a dedicated UE, which breaks the principle of positioning.

If the UE selection is performed in CN, gNB is not aware of the UE in the Positioning measurement procedures for the selected UE.

As above, it’s not proper to select UEs for data collection in the gNB, it’s more appreciate to select UE labels in 5GC.

Proposal 7: to support proactive solution, UE label selection should be performed by CN.

For the procedures of proactive solution, two possible options:

- Option 1: NGAP based procedure

- Option 2: NRPPa based procedure

For the option 1, the call flow is shown as below:

Proactive data collection – NG based solution

As discussed above, it’s not proper for gNB to select and provide NGAP ID(s) to AMF to trigger data collection. Then the data collection should be done by AMF, and AMF triggers positioning request towards the LMF for the selected UEs.

In this solution, Part B(s) will be provided to gNB over NGAP in step 6, using NGAP ID to distinguish UEs. If Part A is generated by the gNB at step 4, when performing positioning procedures in step 4 for the selected UE(s), indicator maybe needed to indicate this measurement is for a selected UE. Furthermore, some kind of UE identity (e.g. NGAP ID) need to be added to the Measurement Request, to mark the Part A, to be associate with the Part B received in step 6.

For NG-based solution, potential specification impact:

- Define new NGAP signallings to request for data, and provision of data.

- existing NRPPa measurement Request need to be enhanced, add new indicator to indicate gNB the measurement is for a selected UE, gNB should generate Part A.

- UE identity, e.g. NGAP ID may need to be introduced in Measurement Request, to mark the Part A, for association with Part B when receive in step 6 in NGAP message.

For the NRPPa based solution, we could refer to the call flow in the SoD the last RAN3 meeting, as below:

Proactive data collection – NRPPa based solution

For the NRPPa based solution, step 1 is used to initiate data collection transaction, then gNB could trigger the data collection request (a new non-UE associated class 2 message) with the same transaction id towards LMF when needed.

LMF performs UE selection and corresponding positioning procedures with some coordination with AMF, similar design with the data collection for case 3b in 5GC, no big impact to core network is foreseen. From RAN3 perspective, we do not need to care about which UEs are selected and how positioning procedures are triggered in 5GC.

When performing Measurement procedures for the selected UEs, LMF should indicate gNB the Measurement is selected for data collection, gNB could generate Part A internally, and gNB indicates LMF the Part B is needed.

LMF provides the requested training data (i.e. Part B) to the gNB via a new non-UE associated class 2 message (e.g. Data Collection Update). gNB associates Part A and Part B with Measurement ID and use them as the collected data for AI/ML model training.

Compared the 2 solutions above, it’s easier to use NRPPa based solution with the assumption that UE selection should be performed in 5GC, corresponding specification impact includes:

- LMF sends a non-UE associated class 2 message to gNB to establish an NRPPa transaction for data collection.

- gNB sends a non-UE associated class 2 message to LMF to request for data collection.

- A new indicator should be added to the Measurement Request to indicate that this measurement (UE) could be used for data collection.

- A new indicator should be added to the measurement Response to indicate that Part B associated to this measurement is requested. (aligned with the opportunistic solution as agreed)

- Introduce a new non-UE associated class 2 message to transfer the Part B from LMF to gNB, Measurement ID is used for association of Part A and Part B. (aligned with the opportunistic solution as agreed)

Proposal 8: To support “Proactive” data collection, NRPPa based solution is preferred with no NGAP impact and minimize the potential impact to 5GC.

For Case 3a, the AI/ML model is deployed at gNB side and the channel measurement entity is also gNB, gNB may request for different information for training, thus, some assistance info should be provided to LMF when request for the training data, e.g. TRP IDs, expected amount of labels, expected measurement types (e.g. UL-RTOA, Multi-RTT).

Moreover, since the AI/ML model is deployed at gNB side, gNB may have its preferred UL SRS configuration for training data collection. However, currently, only LMF has the capability of requesting preferred SRS transmission characteristics, e.g., the density and periodicity of SRS resources. During the request process, LMF does not have prior knowledge about gNB-side model, i.e., LMF does not know the SRS configuration most suitable for gNB-side models, no matter from consistency or measurement perspective. To tackle this issue, gNB can provide its preferred SRS configuration to LMF in advance. In this way, LMF can better negotiate with different TRPs to determine the SRS configuration most suitable for gNB side models. Specifically, the report can be performed in explicit (request specific SRS transmission characteristics) or implicit manner (via associated ID).

Proposal 9: For Proactive data collection, gNB should provide some assistance information to LMF when request for training data, e.g. TRP IDs, expected amount of labels, expected measurement types (e.g. UL-RTOA, Multi-RTT), the preferred UL SRS characteristics/configuration.

2.3 Inference

For Model Input/Report for inference, there’s an open issue that:

- FFS if LMF should be aware that the reported measurements are AI/ML generated.

Not sure about the purpose of such kind of indication, and what’s the expected behaviors of LMF if reported measurements are AI/ML generated or not. As we’ve already agreed that the performance monitoring for the case 3a is performed in the gNB, gNB may start or stop using the AI model for positioning measurement, not necessary to inform the LMF the reported measurements are AI/ML generated or not.

Proposal 10: For Case 3a, no need to indicate LMF the reported measurements are AI/ML generated or not.

3. Conclusion

In this contribution, discussed the RAN3 impact to support AI/ML assisted positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following proposals are provided:

On support of “opportunistic” solution:

Proposal 1: Introduce a new IE Positioning Data Collection Needed in MEASUREMENT RESPONSE.

Proposal 2: Define a new class 2 message POSITIONING DATA COLLECTION REPORT to provide the data collection information from LMF to gNB.

Proposal 3: Include LMF Measurement ID and RAN Measurement ID in POSITIONING DATA COLLECTION REPORT to associate the Part A and Part B.

Proposal 4: Include “Timing Measurement Quality” and “LOS/NLOS Information” per TRP level in the POSITIONING DATA COLLECTION REPORT message.

Proposal 5: RAN3 is requested to discuss and agree the stage 3 TPs to support the opportunistic solution.

On support of “Proactive” Solution:

Proposal 6: Discuss and decide whether the “opportunistic” solution is sufficient, whether need to support “Proactive” data collection.

Observation 1: If gNB is responsible to select UEs for data collection, it could only select its served UEs, not able to select the UEs from the other gNBs (e.g. neighbour gNBs).

Observation 2: In current positioning policy, NG-RAN node functions as an LCS client is not supported.

Proposal 7: to support proactive solution, UE label selection should be performed by CN.

Proposal 8: To support “Proactive” data collection, NRPPa based solution is preferred with no NGAP impact and minimize the potential impact to 5GC.

Proposal 9: For Proactive data collection, gNB should provide some assistance information to LMF when request for training data, e.g. TRP IDs, expected amount of labels, expected measurement types (e.g. UL-RTOA, Multi-RTT), the preferred UL SRS characteristics/configuration.

Inference:

Proposal 10: For Case 3a, no need to indicate LMF the reported measurements are AI/ML generated or not.

4. Reference

- RAN3#127bis Chair Notes

- RAN1#116/#116bis/#117/#118/118bis/#119/120 Chair’s Notes

Annex 1. (TP to BL CR for TS38.413) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.10	NRPPa Transport Procedures

### 8.10.1	General

The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface.

The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling.

The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, Area-specific Information Transfer, Positioning Data Collection Information Transfer and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Annex 2. (TP to BL CR for TS38.455) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

- Positioning Data Collection Information Transfer. This function allows the LMF to provide positioning data information for gNB-based AI Positioning.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Positioning Data Collection Information Transfer | Positioning Data Collection Report |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8	NRPPa procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Positioning Data Collection Report | POSITIONING DATA COLLECTION REPORT |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.X	Positioning Data Collection Information Transfer

### 8.X.1	Positioning Data Collection Report

#### 8.X.1.1	General

The Positioning Data Collection Report procedure allows the LMF to provide the NG-RAN node that the information related to the UE location for gNB-based AI positioning.

#### 8.X.1.2	Successful Operation

Figure 8.x.1.2.1: Positioning Data Collection Report procedure. Successful operation.

The LMF initiates the procedure by sending a POSITIONING DATA COLLECTION REPORT message to the NG-RAN node. The POSITIONING DATA COLLECTION REPORT contains the positioning information for data collection at NG-RAN for gNB-based AI Positioning.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.1.x	Messages for Measurement Information Transfer Procedures

9.1.x.1	POSITIONING DATA COLLECTION REPORT

This message is sent by the NG-RAN node to report information related to UE location.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Information Related to UE Location | M |  | 9.2.x |  | YES | reject |

### 9.2.x	Information Related to UE Location

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP Measurement Related To UE Location List |  | 1 |  |  | YES | reject |
| >TRP Measurement Related To UE Location Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >>Timing Measurement Quality | O |  |  | Corresponds to information provided in NR-TimingQuality IE as defined in TS 37.355 [14] | YES | ignore |
| >>>Measurement Quality | M |  | INTEGER(0..31) |  | - |  |
| >>>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NRPPA-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode,

NRPPATransactionID

FROM NRPPA-CommonDataTypes

ErrorIndication,

PrivateMessage,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

OTDOAInformationRequest,

OTDOAInformationResponse,

OTDOAInformationFailure,

AssistanceInformationControl,

AssistanceInformationFeedback,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningInformationUpdate,

MeasurementRequest,

MeasurementResponse,

MeasurementFailure,

MeasurementReport,

MeasurementUpdate,

MeasurementAbort,

MeasurementFailureIndication,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

SRSInformationReservationNotification,

PositioningDataCollectionReport

FROM NRPPA-PDU-Contents

id-errorIndication,

id-privateMessage,

id-e-CIDMeasurementInitiation,

id-e-CIDMeasurementFailureIndication,

id-e-CIDMeasurementReport,

id-e-CIDMeasurementTermination,

id-oTDOAInformationExchange,

id-assistanceInformationControl,

id-assistanceInformationFeedback,

id-positioningInformationExchange,

id-positioningInformationUpdate,

id-Measurement,

id-MeasurementReport,

id-MeasurementUpdate,

id-MeasurementAbort,

id-MeasurementFailureIndication,

id-tRPInformationExchange,

id-positioningActivation,

id-positioningDeactivation,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-sRSInformationReservationNotification,

id-PositioningDataCollectionReport

<<<<< non-changed texts omitted >>>>>>>>>>

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

e-CIDMeasurementInitiation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-e-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-e-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-e-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-e-CIDMeasurementTermination

CRITICALITY				reject

}

oTDOAInformationExchange NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OTDOAInformationRequest

SUCCESSFUL OUTCOME		OTDOAInformationResponse

UNSUCCESSFUL OUTCOME	OTDOAInformationFailure

PROCEDURE CODE			id-oTDOAInformationExchange

CRITICALITY				reject

}

assistanceInformationControl NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationControl

PROCEDURE CODE			id-assistanceInformationControl

CRITICALITY				reject

}

assistanceInformationFeedback NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationFeedback

PROCEDURE CODE			id-assistanceInformationFeedback

CRITICALITY				reject

}

errorIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY			ignore

}

privateMessage			NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY			ignore

}

positioningDataCollectionReport NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDataCollectionReport

PROCEDURE CODE			id-PositioningDataCollectionReport

CRITICALITY			ignore

}

positioningInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-positioningInformationExchange

CRITICALITY				reject

}

<<<<< non-changed texts omitted >>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

UE-Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

Cell-Portion-ID,

OtherRATMeasurementQuantities,

OtherRATMeasurementResult,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList,

SRSConfiguration,

TRPMeasurementQuantities,

TrpMeasurementResult,

TRP-ID,

TRPInformationTypeListTRPReq,

TRPInformationListTRPResp,

TRP-MeasurementRequestList,

TRP-MeasurementResponseList,

TRP-MeasurementUpdateList,

MeasurementBeamInfoRequest,

PositioningBroadcastCells,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

TRPList,

AbortTransmission,

SystemFrameNumber,

SlotNumber,

RelativeTime1900,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

UEReportingInformation,

UETxTEGAssociationList,

TRP-PRS-Information-List,

PRS-Measurements-Info-List,

UE-TEG-Info-Request,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

PRSConfigRequestType,

MeasurementAmount,

PreconfigurationResult,

RequestType,

UE-TEG-ReportingPeriodicity,

MeasurementPeriodicityNR-AoA,

SRSTransmissionStatus,

CGI-NR,

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

PosValidityAreaCellList,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

PosSRSResourceSet-Aggregation-List,

Remote-UE-Indication-Request,

Remote-UE-Indication,

Remote-UE-Status,

InformationRelatedToUELocation

FROM NRPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-Single-Container{},

NRPPA-PRIVATE-IES,

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-LMF-Measurement-ID,

id-LMF-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-RAN-Measurement-ID,

id-RAN-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-Cell-Portion-ID,

id-OtherRATMeasurementQuantities,

id-OtherRATMeasurementResult,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList,

id-SRSConfiguration,

id-TRPMeasurementQuantities,

id-MeasurementResult,

id-TRP-ID,

id-TRPInformationTypeListTRPReq,

id-TRPInformationListTRPResp,

id-TRP-MeasurementRequestList,

id-TRP-MeasurementResponseList,

id-TRP-MeasurementReportList,

id-TRP-MeasurementUpdateList,

id-MeasurementBeamInfoRequest,

id-PositioningBroadcastCells,

id-SRSType,

id-ActivationTime,

id-SRSResourceSetID,

id-TRPList,

id-SRSSpatialRelation,

id-AbortTransmission,

id-SystemFrameNumber,

id-SlotNumber,

id-SRSResourceTrigger,

id-SFNInitialisationTime,

id-SRSSpatialRelationPerSRSResource,

id-MeasurementPeriodicityExtended,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-UEReportingInformation,

id-UETxTEGAssociationList,

id-TRP-PRS-Information-List,

id-PRS-Measurements-Info-List,

id-UE-TEG-Info-Request,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-PRSConfigRequestType,

id-MeasurementAmount,

id-PreconfigurationResult,

id-RequestType,

id-UE-TEG-ReportingPeriodicity,

id-MeasurementPeriodicityNR-AoA,

id-SRSTransmissionStatus,

id-NewNRCGI,

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-PosValidityAreaCellList,

id-SRSReservationType,

id-NewCellIdentity,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-PosSRSResourceSet-Aggregation-List,

id-PreconfiguredSRSInformation,

id-Remote-UE-Indication-Request,

id-Remote-UE-Indication,

id-Remote-UE-Status,

id-InformationRelatedToUELocation

<<<<< non-changed texts omitted >>>>>>>>>>

-- **************************************************************

--

#### -- POSITIONING DATA COLLECTION REPORT

--

-- **************************************************************

PositioningDataCollectionReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PositioningDataCollectionReport-IEs}},

...

}

PositioningDataCollectionReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-InfoRelatedToUELocation		CRITICALITY reject	TYPE InformationRelatedToUELocation	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- POSITIONING INFORMATION REQUEST

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationRequest-IEs}},

...

}

PositioningInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional	}|

{ ID id-UEReportingInformation					CRITICALITY ignore	TYPE UEReportingInformation							PRESENCE optional	}|

{ ID id-UE-TEG-Info-Request						CRITICALITY ignore	TYPE UE-TEG-Info-Request								PRESENCE optional	}|

{ ID id-UE-TEG-ReportingPeriodicity				CRITICALITY reject	TYPE UE-TEG-ReportingPeriodicity					PRESENCE conditional	}|

-- This IE shall be present if the UE TEG Information Request IE is set to the value "periodic".

{ ID id-TimeWindowInformation-SRS-List			CRITICALITY ignore	TYPE TimeWindowInformation-SRS-List				PRESENCE optional	}|

{ ID id-RequestedSRSPreconfigurationCharacteristics-List	CRITICALITY ignore	TYPE RequestedSRSPreconfigurationCharacteristics-List				PRESENCE optional	}|

{ ID id-Remote-UE-Indication-Request			CRITICALITY ignore	TYPE Remote-UE-Indication-Request				PRESENCE optional	},

...

}

-- **************************************************************

--

-- POSITIONING INFORMATION RESPONSE

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationResponse-IEs}},

...

}

PositioningInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration				PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-UETxTEGAssociationList		CRITICALITY ignore	TYPE UETxTEGAssociationList		PRESENCE optional}|

{ ID id-NewNRCGI					CRITICALITY ignore	TYPE CGI-NR							PRESENCE optional}|

{ ID id-PosValidityAreaCellList		CRITICALITY ignore	TYPE PosValidityAreaCellList	PRESENCE optional}|

{ ID id-SRSPreconfiguration-List	CRITICALITY ignore	TYPE SRSPreconfiguration-List	PRESENCE optional}|

{ ID id-Remote-UE-Indication		CRITICALITY ignore	TYPE Remote-UE-Indication			PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

<<<<< non-changed texts omitted >>>>>>>>>>

-- I

InformationRelatedToUELocation	::= SEQUENCE {

trpMeasurementRelatedtoUeLocation 		TrpMeasurementRelatedtoUeLocation,

iE-Extensions		ProtocolExtensionContainer { { InformationRelatedToUELocation-ExtIEs} } OPTIONAL,

...

}

InformationRelatedToUELocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- T

TrpMeasurementRelatedtoUeLocation	::= SEQUENCE (SIZE (1.. maxnoofMeasTRPs)) OF TrpMeasurementRelatedtoUeLocationItem,

TrpMeasurementRelatedtoUeLocationItem::= SEQUENCE {

tRP-ID							TRP-ID,

loS-NLoSInformation 			LoS-NLoSInformation				OPTIONAL,

timingMeasQuality				TrpMeasurementTimingQuality 		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{ TrpMeasurementRelatedtoUeLocationItem-ExtIEs}}		OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-assistanceInformationControl									ProcedureCode ::= 7

id-assistanceInformationFeedback								ProcedureCode ::= 8

id-positioningInformationExchange								ProcedureCode ::= 9

id-positioningInformationUpdate									ProcedureCode ::= 10

id-Measurement													ProcedureCode ::= 11

id-MeasurementReport											ProcedureCode ::= 12

id-MeasurementUpdate											ProcedureCode ::= 13

id-MeasurementAbort												ProcedureCode ::= 14

id-MeasurementFailureIndication									ProcedureCode ::= 15

id-tRPInformationExchange										ProcedureCode ::= 16

id-positioningActivation										ProcedureCode ::= 17

id-positioningDeactivation										ProcedureCode ::= 18

id-pRSConfigurationExchange										ProcedureCode ::= 19

id-measurementPreconfiguration									ProcedureCode ::= 20

id-measurementActivation										ProcedureCode ::= 21

id-sRSInformationReservationNotification						ProcedureCode ::= 22

id-PositioningDataCollectionReport								ProcedureCode ::= xx

<<<<< non-changed texts omitted >>>>>>>>>>

ritykminus4AdditionalPath					ProtocolIE-ID ::= 150

id-ReportingGranularitykminus5AdditionalPath					ProtocolIE-ID ::= 151

id-ReportingGranularitykminus6AdditionalPath					ProtocolIE-ID ::= 152

id-MeasuredResultsAssociatedInfoList							ProtocolIE-ID ::= 153

id-PointA														ProtocolIE-ID ::= 154

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-Remote-UE-Indication-Request									ProtocolIE-ID ::= 159

id-Remote-UE-Indication											ProtocolIE-ID ::= 160

id-Remote-UE-Status												ProtocolIE-ID ::= 161

id-LocalOrigin													ProtocolIE-ID ::= 162

id-PreconfiguredSRSInformation									ProtocolIE-ID ::= 163

id-DL-reference-signal-UERxTx-TD								ProtocolIE-ID ::= 164

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-InformationRelatedToUELocation								ProtocolIE-ID ::=	yyy

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>