3GPP TSG-RAN WG3 Meeting #128                                                                   R3-253300

Malta, MT, May 19th – 23rd, 2025

Source:	CATT, China Telecom, CMCC, NTT Docomo

Title:	Discussion on L3-report based fast SCell activation

Agenda Item:	9.2

Document for:	Discussion and Approval

1 Introduction

In the Rel-18 NR RRM enhancement topic, RAN4 had some conclusions on unknown FR2 SCell activation enhancement with L3 reporting. RAN4 sent a LS[1] to RAN2 in RAN4#108 meeting (August 2023) for designing the corresponding signaling to support the enhancements.

Following details are captured in the LS to RAN2:

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation

Enhancement for multiple FR2 unknown SCells’ activation Agreements: - Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band. - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized TCI activation command transmitted is based on L3 measurement results or L1 measurement results? Agreements: - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

RAN4 introduced an enhancement to support L3-report based TCI activation during FR2 unknown SCell activation. Later RAN4 extended the same framework for both FR1 and FR2 Unknown SCells.

When SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported for the unknown SCell, the L3 measurement report containing SCell measurements will be sent to gNB. In this case, the gNB will receive the L3 SCell measurement report quickly within few milli seconds (note that UE already performed these L3 deactivated SCell measurements at low periodicity but were not reported to gNB before Aperiodic Scell activation) after sending SCell activation MAC-CE command, the gNB can activate TCI state (i.e., to activate a specific beam) for UE’s SCell based on the L3 measurement report so that it can significantly reduce the latency of SCell activation. Without this enhancement, UE will start legacy L1 measurements after receiving the SCell activation command and report L1 measurements, but result in significant delay for SCell TCI state activation, CSI reporting and subsequent scheduling from gNB.

To support this enhancement, RAN2 has agreed to introduce a new report type “reportOnScellActivation”. According to the CR[2] to RRC spec and CR[3] to MAC spec of RAN2, for the unknown SCell, L3 measurement results are reported upon UE receiving the SCell activation command MAC CE in case the new report type is configured to UE.

2 Discussion

According to RAN4, if the L3 report is triggered upon UE receiving the SCell activation command for the unknown SCell, gNB can activate the TCI state based on the L3 report instead of waiting for L1 report, in this way the activation of unknown SCell can be significantly accelerated.

If both L3 and L1 measurement reports are received by gNB then it is left to implementation which measurements to be used for Scell TCI state activation. However, in split gNB architecture, the node receiving the L3 measurement report is not that transmitting the MAC-CE command of TCI activation, it is CU to receive the L3 measurement report while DU MAC is responsible for MAC CE based SCell activation and MAC CE based TCI state activation to UE. So, the L3-report based SCell activation in split architecture is not supported at present.

To support this enhancement, coordination between CU and DU is needed, and the function split between CU and DU should be discussed. Note that it is important to support this fast SCell activation enhancement, otherwise enhancements to Unknown Scell activation (for both FR1 and FR2) does not work in split gNB architecture and RAN4/RAN2 work will become useless. Without this enhancement, it will take few seconds to activate SCell and start scheduling. RAN3 is proposed to study how to support it.

Proposal 1: RAN3 to discuss how to support L3 measurement report based SCell activation in split gNB architecture.

In general, gNB-CU configure the report type of reportOnScellActivation to UE and receives the corresponding L3 report from UE with this report type, so it knows the L3 measurements report is reported due to SCell activation command based on report config. Thus, CU can determine when to initiate the L3-report based TCI activation.

Proposal 2: CU can initiate the L3-report based TCI activation for fast SCell activation, at reception of relative L3 report from UE.

On how to handle the L3 report, there would be several potential options.

Alt. 1: CU selects the beam for SCell using L3 report

Alt. 2: CU delivers the L3 report to DU

Alt. 3: CU sends the assistant information and the L3 report for DU selecting the beam of SCell

Alt. 1: CU select the beam for SCell using L3 report

When gNB-CU receives the L3 report from UE with trigger type of reportOnScellActivation, it uses the L3 report received from UE to determine the SCell beam, then indicates the selected beam to gNB-DU. Since UE reports all serving cells measurements only if there is one SCell activation triggered, the gNB-CU does not know which SCell activation triggered L3 measurements, therefore it would do the beam selection for all SCells of UE blindly. As CU is also not aware of the information of TCI configuration, it can only carry out SSB level beam selection. Thus, DU shall probably send SSB level TCI activation command.

Alt. 2: CU delivers the L3 report to DU

gNB-CU simply deliver the L3 report to gNB-DU when it receives L3 report with the trigger type reportOnScellActivation. Then gNB-DU determines the beam of SCell based on the L3 report. We understand option 2 with these characteristics:

- F1AP should be enhanced for delivering the L3 report of SCell.

- DU can send TCI activation command based on:

- L3 measurement report, if no L1-RSRP report is received by DU before transmitting the TCI activation command.

- L3 measurement report or L1-RSRP report (up to DU schedular implementation), if L1-RSRP report is also received by DU before transmitting the TCI activation command.

- DU can carry out both SSB level and CSI-RS based beam selection for TCI activation. It will allow DU to retain its MAC schedular functionality.

Alt. 3: CU sends the assistant information for DU selecting beam for SCell

The alternative combining Alt.1 and Alt.2. For example, CU sends DU the recommended SSB(s) together with the L3 report. CU can use the L3 report from UE to select some candidate SSBs for SCell TCI activation, then indicates the candidate beams to gNB-DU. Then DU selects the final beam within the candidate beams which are recommended by CU. Similar to Alt.1, CU would send the recommended information for all the SCells blindly, but DU would be able to do CSI-RS based beam selection.

As we know, up to Rel-18, the functions with respect to beam or L1 report handling are located in DU, while CU is involved when cell is changed. However, note that the function for gNB-DU selecting the beam based on L3 report is already supported in Rel-18. In Rel-18 RACH-less HO, the RRC handover command may contain a beam identifier for the target cell. The beam is determined based on L3 beam level measurement results for the target cell reported by UE. In split architecture, the RRC handover command is generated by gNB-DU and gNB-DU can select the beam used in target cell based on the L3 report transferred via the Handover Preparation Information IE from CU to DU. On this point, L3 report handling is not a new function to DU.

Definition of RACH-less HO command in TS 38.331[4]:

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

On the other hand, as the beam determination in this issue is related to pure SCell operation without cell switching and CU is not involved in SCell operation in legacy, it is assumed that the impact to CU for this issue would be as little as possible. The alternatives with CU handling L3 report would not be necessary for this issue.

Observation 1: Considering the L3 report handing in gNB-DU is already supported in Rel-18 and this is purely SCell related operation without cell switching, CU being involved in L3-report handling would not be necessary.

As discussed above, CU does not know to which SCell(s) the DU needs to send TCI activation, as the L3 report trigger by SCell activation command MAC CE contains the results to all the UE’s serving cells, while DU knows exactly which cell needs to activate TCI status. So, it make more sense for DU to handle the L3 report, in other words, CU-handling alternatives (Alt.1 and 3) are less feasible than Alt.2 where CU is transparent to the L3 report handling.

Observation 2: Alternatives with CU handling L3 report are less feasible than DU-handling alternative.

Based on above analysis, it is proposed RAN3 to confirm that CU is transparent to L3 report handling.

Proposal 3: RAN3 confirm that CU provides L3 measurements in transparent manner to DU to make SCell TCI state activation decision.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1: RAN3 to discuss how to support L3 measurement report based SCell activation in split gNB architecture.

Proposal 2: CU can initiate the L3-report based TCI activation for fast SCell activation, at reception of relative L3 report from UE.

Observation 1: Considering the L3 report handing in gNB-DU is already supported in Rel-18 and this is purely SCell related operation without cell switching, CU being involved in L3-report handling would not be necessary.

Observation 2: Alternatives with CU handling L3 report are less feasible than DU-handling alternative.

Proposal 3: RAN3 confirm that CU provides L3 measurements in transparent manner to DU to make SCell TCI state activation decision.

4 Reference

- R2-2309450, LS on FR2 SCell activation enhancements, RAN2#123bis, RAN4, to RAN2, cc RAN1

- R2-2313936, Introduction of FR2 SCell enhancements, RAN2#124, Apple, CATT, Ericsson, Xiaomi, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- R2-2313937, Introduction of FR2 SCell enhancements, RAN2#124, Xiaomi, Apple, CATT, Ericsson, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.4.0 (2024-12)