3GPP TSG-RAN WG3 Meeting #128                                                                  R3-253305

Malta, MT, 19th – 23rd, May 2025

Agenda Item:	12.3

Source:	CATT

Title:	TP for BLCR to 38.300 and 38.410 for NR Femto

Document for:	Approval

1 Introduction

The TPs for BLCR to TS 38.300 and TS 38.410 for NR Femto are provided.

2 TP to BL CR of TS 38.300

<<<<<<<<<<<<<<<<<<<< Start of TP >>>>>>>>>>>>>>>>>>>>

# 4	Overall Architecture and Functional Split

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto node may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto node and the 5GC. Based on implementation, the transport of NG-U between the NR Femto node and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the Core Network.

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto node shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto node connects via the NR Femto GW) and the procedures run between an NR Femto node and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femto nodes and between NR Femto nodes and gNBs, independent of whether any of the involved NR Femto nodes is connected to an NR Femto GW. One NR Femto node serves one or more cells.

### 4.X.2	Functional Split

#### 4.X.2.1	NR Femto node

An NR Femto node hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto node may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto node shall only connect to a single NR Femto GW at one time:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE:	NR Femto nodes may be deployed without network planning. An NR Femto node may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-	The TAC and PLMN ID(s) used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

#### 4.X.2.2	NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF and the NR Femto node serving the UE, applying the following additional functions:

-	Terminating the UE Context Release request procedure if an explicit GW Context Release Indication is included.  In this case, the NR Femto GW releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by another NR Femto node attached to it.

-	At UE context establishment (Initial Context Setup or NG Handover) the NR Femto GW sends to the NR Femto node the serving AMF’s GUAMI as well as the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Path Switch, the NR Femto GW sends to the NR Femto node the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	Terminating non-UE associated NGAP procedures towards the NR Femto node and towards the AMF, applying the following additional functions:

- 	In case of NGAP INITIAL UE MESSAGE message, verifying that reported CAG ID is valid for that NR Femto node.

-	In case of NGAP SETUP REQUEST message, verifying that the identity used by the NR Femto node is valid.

- 	In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, verifying that the indicated cell identity is valid.

-	In case of NG PWS Restart Indication and PWS Failure Indication, replacing the gNB ID of the NR Femto node with the NR Femto GW ID before sending the respective message to the AMF.

-	At Overload Start/Stop, the NR Femto GW should provide the NR Femto node with the identities of the affected AMF node(s). The NR Femto node uses the received information to identify the traffic and the AMF to which the overload indication applies. The NR Femto node shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto node receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Relaying the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the source AMF received from the NR Femto node.

#### 4.X.2.3	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto node directly connected:

-		verifying that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell.

-		verifying that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message.

-	verifying that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

A TAI used in a NR Femto GW shall not be reused in another NR Femto GW.

### 4.X.3	Interfaces

#### 4.X.3.1	Protocol Stack for NG User Plane

NG-U is defined as specified in clause 4.3.1.1 regardless of whether it is concentrated in the NR Femto GW. The figure below shows the NG-U protocol stack between NR Femto node and UPF.

Figure 4.X.3.1-1: User plane for NG Interface between NR Femto node and UPF

#### 4.X.3.2	Protocol Stacks for NG Control Plane

The figure below shows the NG-C protocol stack with the NR Femto GW.

When the NR Femto GW is not present, NG-C is defined as specified in clause 4.3.1.2, and all the NGAP procedures are terminated at the NR Femto node and the AMF.

Figure 4.X.3.2-1: Control plane for NG Interface for NR Femto node to AMF with the NR Femto GW

### 4.X.4	Access Control

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE:	The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto node may activate a physical cell shared by both PLMN and PNI-NPN, as specified in clause 4.6, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

<<<<<<<<<<<<<<<<<<<< End of TP >>>>>>>>>>>>>>>>>>>>

3 TP to BL CR of TS 38.410

<<<<<<<<<<<<<<<<<<<< Start of TP >>>>>>>>>>>>>>>>>>>>

## 5.7	NAS Node Selection function

The interconnection of NG-RAN nodes to multiple AMFs is supported in the 5GS architecture.

Therefore, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account other information (e.g. slicing information, onboarding indication) to determine the AMF. When the NG-RAN node is configured to ensure that the selected AMF serves the country where the UE is located, as described in TS 23.501 [8], the NG-RAN node takes into account UE location information, if available, when determining the AMF.

When the NR Femto node connects to a Femto GW, selection of an AMF at UE attachment is performed by the NR Femto GW.

This functionality is located in the NG-RAN node and enables proper routing via the NG interface. On NG, no specific procedure corresponds to the NAS Node Selection Function.

<<<<<<<<<<<<<<<<<<<< End of TP >>>>>>>>>>>>>>>>>>>>