3GPP TSG-RAN WG3 Meeting #128                                                                               R3-253308

Malta, MT, 19th – 23rd, May 2025

Source:	CATT

Title:	Potential impacts of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and Decision

1 Introduction

In the last meeting[1], RAN3 had following conclusions:

The maximum number of subgroups is decided to 31. - range of LP-WUS CN Subgroup ID is from 0 to 30: Samsung, E///, Nokia, HW, - range of LP-WUS CN Subgroup ID is from 1 to 31: ZTE, Turn the working assumption into agreement and remove the FFS on the presence of the LP-WUS Subgrouping Support Indication IE per cell in the F1AP Paging message. RAN3 to update the stage2 BLCR by removing the restrictions of LP-WUS in case of emergency PDU session. RAN3 will not send an LS to SA3 regarding this issue. Whether Extended UE Identity Index or Hashed UE_ID is used and how many bits are needed for UE_ID is pending on RAN2.

In this contribution, we will continue to discuss the remaining aspects of LP-WUS:

- Extended UE identity index

- Paging capability loss

- Emergency call back

2 Discussion

2.1 Extended UE identity index

It was agreed that the similar formular defined for PEI subgrouping is reused for LP-WUS subgroup in RAN2#129 meeting[2]:

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

In RAN2#129bis meeting[3], RAN2 sent out a LS to RAN3 to inform the further progress of subgrouping.

UE-ID calculation (RAN2#129bis): - Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X. - X is based on 32 subgrouping number. Details can be discussed in the running CR. LP-WUS with eDRX case (RAN2#129bis): - LP-WUS is supported with eDRX, FFS on exact impact if any

According to the formulation, X should be determined by max(N*Ns*Np*subgroupsNumForUEID). Based on agreement of RAN2, subgroupsNumForUEID used in the formulation is 32. The maximum value of Np is 8. The maximum value of Ns is 4. The maximum value of N is 256 if eDRX is not used, while it’s 1024 for eDRX. So, X can be calculated as:

- If eDRX is not used, X = 210 *23*25=262144

- If eDRX is used, X= 212 *23*25=1048576

As UE_ID=5G-S-TMSI mod X, UE_ID taking the LSBs of 5G-S-TMSI can be illustrated as:

A conclusion can be derived that that UE_ID needs 18 or 20 bits truncated 5G-S-TMSI for non-eDRX and eDRX respectively.

Observation 1: According to RAN2’s agreements for subgrouping, the UE_ID needs to take 18 or 20 LSBs of 5G-S-TMSI for non-eDRX and eDRX respectively.

For RAN paging, the UE_ID used for LP-WUS subgrouping should be sent over NG, Xn and F1 interfaces. However, existing Extended UE Identity Index Value IE sent over NG/Xn/F1 interface for deriving PF and PO for eDRX and PEI subgroup ID is only 16 bits. It is not long enough to contain the UE_ID used for calculating the LP-WUS subgroup ID, thus the existing IE cannot be reused.

#### TS 38.473: #### 9.3.1.285 Extended UE Identity Index Value This IE is used by the gNB-DU to calculate the Paging Frame and Paging Occasion for eDRX, and the UE_ID based subgroup ID as specified in TS 38.304 [24]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

Observation 2: The existing Extended UE Identity Index Value IE sent over F1/Xn/NG is not long enough for containing the UE_ID to calculate the subgrouping ID for LP-WUS.

A New IE of 20 bits length bit string needs to be introduced to NGAP, XnAP and F1AP spec, used by RAN to calculate the UE_ID based subgroup ID for LP-WUS.

Proposal 1: A new IE of 20bits length bit string should be introduced to NGAP, XnAP and F1AP for RAN to calculate the UE_ID based subgroup ID for LP-WUS.

2.2 UE paging capability loss

In the RAN2#129 meeting[2], RAN2 discussed the question from SA2 (R3-250021/S2-2412876)[4] on UE paging capability loss and provided the feedback in reply LS [5] as below:

- On the issue of UE radio capability for paging information, RAN2 had the following agreement: - Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for Release 15 gNB and Release 16 gNB and for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the coming RAN2 meetings based on company contributions.

In RAN2#129bis[3], RAN2 further discussed this issue and made more agreements, a LS(R3-253012)[6] was sent to RAN3 as below:

For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. - We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

According to the agreements above, RAN2 will not introduce any new solution for this issue, including enhancing the RRC spec for UE Radio Paging Capability to Rel-18 or earlier Releases, and the issue of paging capability loss will rely on network implementation. It means that the UE paging capability of LP-WUS (or paging features beyond Rel-19) may still be lost by the gNBs of Rel-18 or before Rel-18.

RAN2 also indicate they will introduce a container containing UE paging capabilities for Rel-19 or Rel-19 onwards.  It is assumed that this problem does not exist for gNBs of Rel-19 or on-wards. Therefore, all the gNBs of the area deploying the LP-WUS (or paging features beyond Rel-19) should be at least upgraded to Rel-19, which is exactly what RAN2 suggested in LS.

Proposal 2: All gNBs in the area deploying LP-WUS should be upgraded to Rel-19 for handling the Rel-19 container of UE Radio Paging Capabilities generated by UE.

2.2 Emergency callback and LP-WUS

In SA2#167 meeting, a CR to TS 23.501 (S2-2502428)[7] was approved for removing unnecessary restrictions to PEIPS and LP-WUS usage in emergency PDU session. The reason is that:

Reason for change:; PSAP callback is delivered as normal call in IMS PDU session. There is no need for the UE and network to disable functionalities like PEIPS or LP-WUS when emergency PDU session is active.

Accordingly, RAN3 agreed to remove the LP-WUS restriction for emergency PDU session for RRC inactive UE in the last meeting.

There is no restriction to LP-WUS usage with emergency PDU session, but it’s not clear whether LP-WUS usage is restricted for PSAP callback or normal IMS PDU session. Basically, voice-centric UEs have a permanent IMS PDU session setup, the normal IMS PDU session may be deactivated or keep activated during setup period. And the voice centric UEs are assume not in favor of long paging latency.

When LP-WUS is used, the paging latency would vary within {offset between LO and PO, offset between LO and PO + iDRX cycle}, wherein, the offset between LO and PO is no less than the wake-up delay which depends on UE’s capability. The maximum wake-up delay of UE capability plus the synchronization delay in case of 20ms SSB periodicity is around 1000ms. Though the exact maximum offset value is still pending on RAN1/4, it would be probably no more than 2 seconds in the worst case. This offset value is much smaller than a long eDRX cycle. Therefore, LP-WUS would not have critical impact to paging latency, it should not be restricted in PSAP callback or normal IMS PDU session.

Proposal 3: The restriction to LP-WUS for emergency callback or normal IMS PDU session is not necessary.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Proposal 1: A new IE of 20bits length bit string should be introduced to NGAP, XnAP and F1AP for RAN to calculate the UE_ID based subgroup ID for LP-WUS.

Proposal 2: All gNBs in the area deploying LP-WUS should be upgraded to Rel-19 for handling the Rel-19 container of UE Radio Paging Capabilities generated by UE.

Proposal 3: The restriction to LP-WUS for emergency callback or normal IMS PDU session is not necessary.

4 Reference

- RAN3 Chair’s Notes, RAN3#127bis

- draft_MeetingReport_RAN2#129, MCC

- draft_MeetingReport_RAN2#129bis, MCC

- R3-250021(S2-2412876), LS Reply on LP-WUS subgrouping, SA2, RAN3#127

- R3-251515(R2-2501388), Reply LS on LP-WUS subgrouping, RAN2, RAN3#127bis

- R3-253012 (R2-2503172), Reply LS on paging capability loss issue, RAN2, RAN3#128

- S2-2502428, Removing unnecessary restrictions for emergency PDU sessions, Qualcomm, SA2#167